=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6050

สาเหตุ: important parameter is modified.

การแก้ไข: turn power off once and on again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6051

สาเหตุ: when power was turned on, state point data 1 was smaller than zero or larger than machine coordinate value for one round of magazine.

การแก้ไข: switch to atc maintenance mode and adjust the magazine position.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6053

สาเหตุ: setting of machine parameters 2 (system 2) is faulty.

การแก้ไข: check the settings of machine parameters (system 2)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6055

สาเหตุ: atc arm is not at home.

การแก้ไข: turn off power and see 13.6 resetting atc.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6056

สาเหตุ: the atc arm is stopped midway, and cannot be reset by key operation.

การแก้ไข: switch to atc maintenance mode and adjust the magazine position

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6057

สาเหตุ: data structure of user parameter is destroyed.

การแก้ไข: press [rst] key, and delete the destroyed data by memory screen in edit node and create new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6058

สาเหตุ: setting of user parameters 2 (switch 2) faults.

การแก้ไข: check the setting of user parameters 2 (switch 2)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6059

สาเหตุ: setting of user parameters 1 (switch 1) faults.

การแก้ไข: check the setting of user parameters 1 (switch 1)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6150

สาเหตุ: slave cpu error signal was received, but the corresponding error cannot be identified.

การแก้ไข: press the [rst] key to reset the error. this error may occur due to machine failure and the like. contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6151

สาเหตุ: memory never cleared or data destroyed.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6152

สาเหตุ: the same date are to be edited at the same time.

การแก้ไข: check the communication mark has disappeared, then press [rst] key and clear the alarm.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6153

สาเหตุ: during editing, data is destroyed.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6154

สาเหตุ: during editing, data is destroyed.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6154

สาเหตุ: during editing, data is destroyed.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6155

สาเหตุ: during editing, data is destroyed.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6156

สาเหตุ: recovery distance when turning on the servo is too long. (door interlock is effective.)

การแก้ไข: press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6157

สาเหตุ: there is no user parameter data.

การแก้ไข: create the user parameter and turn power off and then on again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6158

สาเหตุ: data structure of user parameter is destroyed.

การแก้ไข: press [rst] key, and delete the destroyed data by memory screen in edit mode and create new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6159

สาเหตุ: data structure of user parameter is destroyed.

การแก้ไข: press [rst] key, and delete the destroyed data by memory screen in edit mode and create new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6160

สาเหตุ: program is structure is destroyed.

การแก้ไข: press [rst] key, and delete the destroyed data by memory screen in edit mode and create new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6161

สาเหตุ: when the power is on, the servo motor is on, or while the magazine is swiveling, the difference between start point data 2 and the absolute position data in the magazine amplifier has exceeded the value for one revolution of the magazine.

การแก้ไข: check the pot number displayed on the present position screen and the pot number actually indexed. if these differ, switch to atc maintenance mode and adjust the magazine position.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6162

สาเหตุ: the magazine position was shifted more than the value set for [lmt pulse for mz position alrm] in machine parameter 3 while the magazine was stopped (when the pot was fully lowered).

การแก้ไข: check the setting of machine parameter. this error may occur due to machine failure and the like. if the error still occurs, contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6163

สาเหตุ: zero dog signal was not confirmed while dog zero return was being performed.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6164

สาเหตุ: zero dog signal was not confirmed while dog zero return was being performed.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6165

สาเหตุ: zero dog signal was not confirmed while dog zero return was being performed.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6166

สาเหตุ: zero dog signal was not confirmed while dog zero return was being performed.

การแก้ไข: when the first orientation is performed after power is turned on, phase c signal cannot be received even after spindle motor has rotated one turn.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6167

สาเหตุ: zero dog signal was not confirmed while dog zero return was being performed. (the asterisk (*) indicates axis 4.)

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6168

สาเหตุ: zero dog signal was not confirmed while dog zero return was being performed. (the asterisk (*) indicates axis 5.)

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6169

สาเหตุ: zero dog signal was not confirmed while dog zero return was being performed. (the asterisk (*) indicates axis 6.)

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6170

สาเหตุ: zero dog signal was not confirmed while dog zero return was being performed. (the asterisk (*) indicates axis 7.)

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6171

สาเหตุ: zero dog signal was not confirmed while dog zero return was being performed. (the asterisk (*) indicates axis 8.)

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6172

สาเหตุ: main cpu is not synchronized with slave cpu.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6173

สาเหตุ: clamping axis was attempted but failed. (the asterisk (*) indicates axis 4.)

การแก้ไข: press the [rst] key. check if air pressure is appropriate. check the machine parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6174

สาเหตุ: unclamping axis was attempted but failed. (the asterisk (*) indicates axis 4.)

การแก้ไข: press the [rst] key. check if air pressure is appropriate. check the machine parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6175

สาเหตุ: while * axis (4 axis) is rotating, * axis is clamped.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6176

สาเหตุ: while * axis (4 axis) is holding, * axis is unclamped.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6177

สาเหตุ: the axis has shifted during clamping more than the value set to [lmt pulse for 4 position alrm]. if the value set to [in-position width 4] is larger than the value set to [lmt pulse for position alrm], however this error occurs when the axis has shifted more than the value set to [in-position width 4]. (these parameters are set on the <machine parameter (system 2)> screen).

การแก้ไข: press the [rst] key. check the axis position on the current position screen and move the axis to the correct position. check that the value set to [lmt pulse for 4 position alrm] is not too small. when this error occurs during memory operation reduce the cutting depth.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6178

สาเหตุ: clamping axis was attempted but failed. (the asterisk (*) indicates axis 5.)

การแก้ไข: press the [rst] key. check if air pressure is appropriate. check the machine parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6179

สาเหตุ: unclamping axis was attempted but failed. (the asterisk (*) indicates axis 5.)

การแก้ไข: press the [rst] key. check if air pressure is appropriate. check the machine parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6180

สาเหตุ: while axis (5 axis) is rotating, * axis is clamped.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6181

สาเหตุ: while axis (5 axis) is holding, * axis is unclamped.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6182

สาเหตุ: the axis has shifted during clamping more than the value set to [lmt pulse for 5 position alrm]. if the value set to [in-position width 5] is larger than the value set to [lmt pulse for 5 position alrm], however this error occurs when the axis has shifted more than the value set to [in-position width 5]. (these parameters are set on the <machine parameter (system 2)> screen).

การแก้ไข: press the [rst] key. check the axis position on the current position screen and move the axis to the correct position. check that the value set to [lmt pulse for 5 position alrm] is not too small. when this error occurs during memory operation reduce the cutting depth.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6183

สาเหตุ: clamping axis was attempted but failed. (the asterisk (*) indicates axis 6.)

การแก้ไข: press the [rst] key. check if air pressure is appropriate. check the machine parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6184

สาเหตุ: unclamping axis was attempted but failed. (the asterisk (*) indicates axis 6.)

การแก้ไข: press the [rst] key. check if air pressure is appropriate. check the machine parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6185

สาเหตุ: while * axis (6 axis) is rotating, * axis is clamped.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6186

สาเหตุ: while * axis (6 axis) is holding, * axis is unclamped.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6187

สาเหตุ: the axis has shifted during clamping more than the value set to [lmt pulse for 6 position alrm]. if the value set to [in-position width 6] is larger than the value set to [lmt pulse for 6 position alrm], however this error occurs when the axis has shifted more than the value set to [in-position width 6]. (these parameters are set on the <machine parameter (system 2)> screen).

การแก้ไข: press the [rst] key. check the axis position on the current position screen and move the axis to the correct position. check that the value set to [lmt pulse for 6 position alrm] is not too small. when this error occurs during memory operation reduce the cutting depth.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6188

สาเหตุ: for models with a double-arm atc: z-axis was commanded when the z-axis was above the atc zero point. for models with an armless atc: z-axis was commanded when the z-axis was above the z axis zero point. (atc sensor was turned on)

การแก้ไข: press the [rst] key. move the z-axis until it is below the z-axis zero point while holding down the [relse] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6189

สาเหตุ: for models with a double-arm atc: spindle was commanded while the atc arm was not at zero position. for models with an armless atc: spindle was commanded when the z-axis was above the z axis zero point. (atc sensor was turned on)

การแก้ไข: for models with a double-arm atc: press the [rst] key. press the [atc] key with the [relse] key held down in manual mode to move the atc arm to zero position. for models with an armless atc: press the [rst] key. move the z-axis until it is below the z-axis zero point while holding down the [relse] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6191

สาเหตุ: the commanded speed is zero or larger than the maximum speed.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6192

สาเหตุ: for arc cutting when the last point is not on the arc, the speed of the single axis is larger than the maximum speed.

การแก้ไข: check start point and end point of program as well as circular arc center point.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6193

สาเหตุ: the atc arm swivel is not completed within the time set for [atc arm swivel time out 1-4]

การแก้ไข: press the [rst] key to reset the error. for models with a double-arm atc: press the [rst] key with the [relse] key held down in manual mode.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6194

สาเหตุ: for models with an armless atc the [emergency stop] key was pressed while the magazine was swiveling and the magazine stopped midway. for models with an armless atc: after rotate the magazine, not index the commanded magazine. encoder signal is 0 or more than the number of magazine tools.

การแก้ไข: after the alarm indication has disappeared, press the [magz] key with the [relse] key held down.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6195

สาเหตุ: when magazine indexed incorrectly (atc cam sensor is turned off.), after the alarm indication has disappeared, rotate the magazine or command the z axis.

การแก้ไข: press the [rst] key with the [relse key held down in manual mode. after the alarm indication has disappeared, press the [magz] key with the [relse] key held down.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6196

สาเหตุ: when z axis position was not in the atc zero point, the magazine is rotate.

การแก้ไข: press the [rst] key to reset the alarm.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6197

สาเหตุ: a number exceeding the number of tools set in the magazine was required by atc command.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6198

สาเหตุ: pot position is incorrect.

การแก้ไข: switch to manual operation mode press the [rst] key with the [relse] key held down. then, press the [atc] key with the [relse] key held down to move the magazine pot to the correct position.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6199

สาเหตุ: magazine was not indexed in 5 seconds per a pitch or the thermal in the magazine motor functioned.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6200

สาเหตุ: raising and lowering of the pot was not completed within the specified time.

การแก้ไข: turn off power and eliminate the cause.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6201

สาเหตุ: pot sensor is faulty.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6202

สาเหตุ: atc arm is not at home.

การแก้ไข: press the [rst] key with the [relse] key held down in manual mode. see 13.6 resetting atc.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6203

สาเหตุ: the x-axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (system 1).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6204

สาเหตุ: the x-axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (system 1).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6205

สาเหตุ: the y-axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (system 1).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6206

สาเหตุ: the y-axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (system 1).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6207

สาเหตุ: the z-axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (system 1).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6208

สาเหตุ: the z-axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (system 1).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6209

สาเหตุ: the 4 axis was attempted to be moved past its stroke limit in positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6210

สาเหตุ: the 4 axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6211

สาเหตุ: the 5 axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6212

สาเหตุ: the 5 axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6213

สาเหตุ: the 6 axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6214

สาเหตุ: the 6 axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6215

สาเหตุ: the x-axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6216

สาเหตุ: the x-axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6217

สาเหตุ: the y-axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6218

สาเหตุ: the y-axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6219

สาเหตุ: the z-axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6220

สาเหตุ: the z-axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6221

สาเหตุ: the 4 axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6222

สาเหตุ: the 4 axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6223

สาเหตุ: 5 axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: (check the machine parameters (switch 2).) press the rst key and correct the program.

=== END ==

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6224

สาเหตุ: the 5 axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: (check the machine parameters (switch 2).) press the rst key and correct the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6225

สาเหตุ: the 6 axis was attempted to be moved past its stroke limit in the positive direction.

การแก้ไข: (check the machine parameters (switch 2).) press the [rst] key and correct the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6226

สาเหตุ: the 6 axis was attempted to be moved past its stroke limit in the negative direction.

การแก้ไข: (check the machine parameters (switch 2).) press the [rst] key and correct the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6227

สาเหตุ: the axis was attempted to be moved into the prohibited stroke zone.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (system1).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6228

สาเหตุ: the axis was attempted to be moved into the prohibited stroke limit zone.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6232

สาเหตุ: each workpiece counter has attained the preset value.

การแก้ไข: press the [rst] key and correct the program. (check the machine parameters (switch 2).)

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6233

สาเหตุ: each workpiece counter has attained the preset value.

การแก้ไข: press the [rst] key and check the measurement position.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6234

สาเหตุ: each workpiece counter has attained the preset value.

การแก้ไข: press the [rst] key and check the measurement position.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6235

สาเหตุ: each workpiece counter has attained the preset value.

การแก้ไข: press the [rst] key and check the end point, center point and feedrate in the thread cutting.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6236

สาเหตุ: detection signal was not turned on during measurement.

การแก้ไข: purchase a thread cutting option.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6237

สาเหตุ: detection signal was not turned off during measurement.

การแก้ไข: press the [rst] key and check the measurement position.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6238

สาเหตุ: the z axis feedrate is not obtained from the x/y axis feedrate during the thread cutting.

การแก้ไข: press the [rst] key and check the end point, center point and feedrate in the thread cutting.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6239

สาเหตุ: there is no optional thread cutting function.

การแก้ไข: purchase a thread cutting option.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6240

สาเหตุ: after external signal is given, completion signal can not be detected even when external signal reading limit time is over. m46 signals are not detected even after the time preset to maximum time of exit signal has passed after motions of signal output functions 46 have been completed.

การแก้ไข: press [rst] key and set external reading completion signal within the limit time, or charge parameter value.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6241

สาเหตุ: the graphic calculation is not available any longer.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6242

สาเหตุ: a signal was output from the tool detection sensor.

การแก้ไข: check the service life of the broken tool on the screen, and then press the [rst] key. if a suitable spare tool is registered, operation can be resumed. if no suitable spare tool is registered, replace the tool, set the tool length and tool life, and then resume operation. note: when the [rst] key is pressed, the tool life indication value is automatically cleared. note: if the tool is not broken, the tool breakage detection device needs adjustment. refer to option tool breakage detection device -> 4.adjustment.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6243

สาเหตุ: the tool life of the tool in use or the one to be used has reached zero. (*) the figure in brackets () indicates the tool number.

การแก้ไข: replace tools and set tool life according after pressing [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6244

สาเหตุ: the result of zero point measurement has exceeded the tolerance set by the user parameter (zero point measurement).

การแก้ไข: press the [rst] key and check the measurement position in the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6245

สาเหตุ: positioning to the measuring point was attempted (g120) although measuring data does not exist.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6246

สาเหตุ: displacement measured in z-axis thermal displacement offset mode exceeded parameter z-axis machining accuracy.

การแก้ไข: see "parameter setting and change z-axis thermal displacement offset mode to appropriate conditions."

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6247

สาเหตุ: thermal displacement compensation was attempted for areas other than z-axis machine zero position.

การแก้ไข: do not move the axis in manual mode switching motion

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6248

สาเหตุ: offset at stroke end exceeded thrm offset amt limit value in z-axis thermal displacement offset mode.

การแก้ไข: check if parameter measuring device installation position z and tool length menu the tool length measuring device for chip are correct. check the measuring section of for chip adhesion.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6249

สาเหตุ: when cover open chcking is executed, cover opening cannot be comfirmed within 10 seconds.

การแก้ไข: check tool length measuring device cover.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6250

สาเหตุ: tool breakage detection command or thermal measurement command was attempted to be executed for machines without optional z-axis measurement system.

การแก้ไข: change program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6252

สาเหตุ: an attempt is made to modify the data while the protect sw is on.

การแก้ไข: set the protect switch to off, and modify the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6253

สาเหตุ: a signal output 8xx is executed without sequence controller (option).

การแก้ไข: change the signal output into a signal output xx.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6254

สาเหตุ: an error signal was received from the external plc. at this time, no bit of internal auxiliary contact 20 signal (8-bit data) is on.

การแก้ไข: check the conditions when the external plc alarm occurs, eliminate the cause, and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6255

สาเหตุ: error signal was input from the external plc.

การแก้ไข: check the conditions that external plc alarm occur eliminate the cause and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6256

สาเหตุ: error signal was input from the external plc.

การแก้ไข: check the conditions that external plc alarm occurs eliminate the cause and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6257

สาเหตุ: error signal was input from the external plc.

การแก้ไข: check the conditions that external plc alarm occurs eliminate the cause and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6258

สาเหตุ: error signal was input from the external plc.

การแก้ไข: check the conditions that external plc alarm occurs eliminate the cause and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6259

สาเหตุ: error signal was input from the external plc.

การแก้ไข: check the conditions that external plc alarm occurs eliminate the cause and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6260

สาเหตุ: error signal was input from the external plc.

การแก้ไข: check the conditions that external plc alarm occurs eliminate the cause and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6261

สาเหตุ: error signal was input from the external plc.

การแก้ไข: check the conditions that external plc alarm occurs eliminate the cause and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6262

สาเหตุ: exer 10 signal comes on.

การแก้ไข: turn off the exer 10 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6263

สาเหตุ: exer11 signal comes on.

การแก้ไข: turn off the exer11 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6264

สาเหตุ: exer12 signal comes on.

การแก้ไข: turn off the exer12 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6265

สาเหตุ: exer13 signal comes on.

การแก้ไข: turn off the exer13 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6266

สาเหตุ: exer14 signal comes on.

การแก้ไข: turn off the exer14 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6267

สาเหตุ: exer15 signal comes on.

การแก้ไข: turn off the exer125 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6268

สาเหตุ: exer16 signal comes on.

การแก้ไข: turn off the exer16 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6269

สาเหตุ: exer17 signal comes on.

การแก้ไข: turn off the exer17 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6270

สาเหตุ: exer18 signal comes on.

การแก้ไข: turn off the exer18 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6271

สาเหตุ: exer19 signal comes on.

การแก้ไข: turn off the exer19 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6272

สาเหตุ: exer20 signal comes on.

การแก้ไข: turn off the exer20 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6273

สาเหตุ: exer21 signal comes on.

การแก้ไข: turn off the exer21 and push [rst key

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6274

สาเหตุ: exer22 signal comes on.

การแก้ไข: turn off the exer22 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6275

สาเหตุ: exer23 signal comes on.

การแก้ไข: turn off the exer23 and push [rst]key

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6276

สาเหตุ: exer24 signal comes on.

การแก้ไข: turn off the exer24 and push [rst]key

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6277

สาเหตุ: exer25 signal comes on.

การแก้ไข: turn off the exer25 and push [rst]key

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6278

สาเหตุ: 1) the automatic door limit switch has turned on both at door closing and opening ends. 2) it took x seconds (parameter value) for automatic door closing. 3) it took x seconds (parameter value) for automatic door opening.

การแก้ไข: eliminate the error cause and press the rsti key to reset the error.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6279

สาเหตุ: tool required for operation is not set in the magazine.

การแก้ไข: press the [rst] key, and set the tool on the screen correctly.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6280

สาเหตุ: an error was received but the corresponding error cannot be identified.

การแก้ไข: press the [rst] key and clear the alarm. this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6281

สาเหตุ: an error was received but the corresponding error cannot be identified.

การแก้ไข: press the [rst] key and clear the alarm. this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6282

สาเหตุ: the tool was attempted to be replaced with a prohibited pot.

การแก้ไข: -

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6283

สาเหตุ: the tool was attempted to be replaced with a pot with the cap specified.

การแก้ไข: -

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6284

สาเหตุ: an "a" axis program was attempted to be executed although an optional "a" axis is not provided.

การแก้ไข: purchase an optional "a" axis. when already provided, set [a option] to [1. yes].

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6285

สาเหตุ: a "b" axis program was attempted to be executed although an optional "b" axis is not provided.

การแก้ไข: purchase an optional "b" axis. when already provided, set [b option] to [1. yes].

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6286

สาเหตุ: a "c" axis program was attempted to be executed although an optional "c" axis is not provided.

การแก้ไข: purchase an optional "c" axis. when already provided, set [c option] to [1. yes].

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6287

สาเหตุ: raising or lowering of the pot has been completed earlier than the specified time.

การแก้ไข: turn off power and eliminate the cause.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6288

สาเหตุ: no pot was available when changing from standard to large tool or vice versa.

การแก้ไข: prepare an empty pot.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6289

สาเหตุ: the [protect] switch was turned on as soon as [tool length set] was executed in automatic tool length measurement mode or [tool length set] could not be executed for some reason.

การแก้ไข: multiple errors have occurred concurrently. check the causes on the screen and eliminate them.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6291

สาเหตุ: the z-axis measurement system was attempted to be operated using the pallet without a measurement device.

การแก้ไข: operate the system using the pallet with a measurement device.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6292

สาเหตุ: the measurement was attempted to be operated although the pallet was not indexed.

การแก้ไข: index the pallet and operate the system.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6293

สาเหตุ: tools are not set in the magazine correctly.

การแก้ไข: set the tools in the magazine correctly.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6294

สาเหตุ: the address of the additional axis is not set correctly.

การแก้ไข: set the address correctly.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6295

สาเหตุ: the additional axis of the pallet opposite the pallet in operation was attempted to be moved.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6296

สาเหตุ: zero position measurement was attempted although zero position measurement option is not provided.

การแก้ไข: purchase this option or change the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6297

สาเหตุ: tool breakage detecting sensor input is illegal.

การแก้ไข: when a tool breakage detection option is not provided, set tool breakage detect option to [0: no]. when provided, check to see if chips are stuck to the detection arm, if so, remove them. this error may occur due to machine failure and the like. if error still occurs, contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6298

สาเหตุ: tool breakage detecting motion (extend or fold the detevting arm) time exceeds 5 sec.

การแก้ไข: push [rst] key and check the air pressure (3-5kg/cm2). if the air is ok, contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6299

สาเหตุ: software switch data was damaged.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6300

สาเหตุ: the spindle override connector is not connected.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6304

สาเหตุ: automatic z-axis thermal displacement compensation was attempted although that optional function is not provided.

การแก้ไข: purchase this option or change the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6305

สาเหตุ: when door interlock is valid for machines conforming to specifications, the door was opened while the axis " " (axis 4) was in motion. conformance to ce specifications is standard for the tc-32a, r2a, 20a, and s2b.

การแก้ไข: press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6306

สาเหตุ: when door interlock is valid for machines conforming to specifications, the door was opened while the axis"*" (axis 5) was in motion. conformance to ce specifications is standard for the tc-32a, r2a, 20a, and s2b.

การแก้ไข: press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6307

สาเหตุ: when door interlock is valid for machines conforming to specifications, the door was opened while the axis "*" (axis 6) was in motion. conformance to ce specifications is standard for the tc-32a, r2a, 20a, and s2b.

การแก้ไข: press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6308

สาเหตุ: the spindle was attempted to be rotated while the coolant blowing function was on.

การแก้ไข: turn the coolant blowing function off and then rotate the spindle.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6309

สาเหตุ: the coolant blowing function was attempted to be turned on while the spindle was rotating.

การแก้ไข: stop the spindle and then turn the coolant blowing function on.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6310

สาเหตุ: the door was open when swiveling the atc arm was attempted.

การแก้ไข: close the door and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6311

สาเหตุ: either the high pressure or filter pump is overloaded.

การแก้ไข: eliminate the cause of the overload.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6312

สาเหตุ: the line filter is clogged.

การแก้ไข: check that the element in the line filter and pipe are not damaged, and clean the element.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6313

สาเหตุ: the end mill tap commanded, zaxis speed of the end mill is faster than tapping section.

การแก้ไข: check the program after pressing [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6314

สาเหตุ: don't have the zaxis fixed speed area of the end mill when the end mill tap was commanded.

การแก้ไข: check the program after pressing [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6315

สาเหตุ: don't have the tapping section of zaxis fixed speed area when the end mill tap was commanded.

การแก้ไข: check the program after pressing [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6316

สาเหตุ: the spindle command was ordered by intervention operate during the spindle rotation of the end mill tap is command.

การแก้ไข: reset the alarm to press [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6317

สาเหตุ: the cover did not complete opening and closing within five seconds after opening and closing operation was commenced.

การแก้ไข: press the [rst key and clear the alarm. this error may occur due to machine failure and the like. if the error still occurs, contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6318

สาเหตุ: the magazine cover was not fully open when the arm started swiveling.

การแก้ไข: reset the alarm to press [rst] key. this error may occur due to machine failure and the like. if the error still occurs, contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6319

สาเหตุ: the sensors to detect the magazine cover opening and closing ends were on when the power was turned on and the [rst] key pressed.

การแก้ไข: this error may occur due to machine failure and the like. if the error still occurs, contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6320

สาเหตุ: the x and y axes were beyond their stroke limits when the [atc] key was pressed.

การแก้ไข: press the [rst] key to reset the alarm and return the x and y axes to within their stroke limits.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6321

สาเหตุ: the machine was attempted to be operated when the x axis was within the atc operation range or the atc operation limit switch was on, or the atc operation limit switch was turned off before the atc arm was attempted to be swiveled.

การแก้ไข: press the [rst] key to reset the alarm and return the x axis to within their stroke limits. this error may occur due to machine failure and the like. if this error occurs, contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6323

สาเหตุ: zero position return was attempted when the c-axis (pallet) had not been indexed.

การแก้ไข: press the [rst] key to reset the alarm. index the c axis to 0 degrees or -180 degrees and then conduct zero position return.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6324

สาเหตุ: the zero point limit switch for axis no. 4 was not on when zero position return was attempted by pressing the [relse] and [z.rtn] keys.

การแก้ไข: press the [rst] key to reset the alarm. move the axis no. 4 to the position where the zero position limit switch for the axis no. 4 turns on, and press the [relse] and [z.rtn] keys. ->refer to the attached sheet.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6325

สาเหตุ: the zero point limit switch for axis no. 5 was not on when zero position return was attempted by pressing the [relse] and [z.rtn] keys.

การแก้ไข: press the [rst] key to reset the alarm. move the axis no. 5 to the position where the zero position limit switch for the axis no. 5 turns on, and press the [relse] and [z.rtn] keys. ->refer to the attached sheet.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6326

สาเหตุ: the zero point limit switch for axis no. 6 was not on when zero position return was attempted by pressing the [relse] and [z.rtn] keys.

การแก้ไข: press the [rst] key to reset the alarm. move the axis no. 6 to the position where zero position limit switch for the axis no.6 turns on, and press the [relse] and [z.rtn] keys. ->refer to the attached sheet.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6327

สาเหตุ: there is no memory area for automatic x and y axes thermal displacement compensation data.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6328

สาเหตุ: automatic x-axis thermal displacement compensation was attempted although that optional function is not provided.

การแก้ไข: please amend a program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6329

สาเหตุ: automatic y-axis thermal displacement compensation was attempted although that optional function is not provided.

การแก้ไข: please amend a program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6330

สาเหตุ: the diameter is being corrected during xz and yz circular interpolation.

การแก้ไข: please amend a program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6331

สาเหตุ: the coordinates are being rotated during xz and yz circular interpolation.

การแก้ไข: please amend a program

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6332

สาเหตุ: the difference between the current zero measurement result and the previous zero measurement result has exceeded measurement tolerance 2 of user parm (zero measurement).

การแก้ไข: press the [rst] key and check the measurement position in the program/

=== END ==

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6334

สาเหตุ: tool length setting for [tool breakage detect option (type 3)] is incorrect. this error occurs when the length of the tool subject to breakage detection corresponds to any of the following:

1.(tool length - tool breakage detector return position) < 60 mm

2.(tool length - tool breakage detector return position) > (tool breakage detection motor stroke + 60 mm)

การแก้ไข: check the settings for [tool breakage detect option] and [tool brk detect return pos) (user parameters), and tool length on the tool list screen.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6335

สาเหตุ: io board, tb board, or tool breakage detector is faulty.

การแก้ไข: press the [rst] key to reset the error. if the error still occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6336

สาเหตุ: tool breakage detection motor did not move to the specified position.

การแก้ไข: press the [rst] key to reset the error. if the error still occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6337

สาเหตุ: the tool breakage detector is faulty.

การแก้ไข: press the [rst] key to reset the error. if the error still occur due to machine failure and the like . contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6338

สาเหตุ: m496 command is used while cts on.

การแก้ไข: press the [rst]key and clear the alarm. this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6339

สาเหตุ: the cts pump thermal signal was input.

การแก้ไข: press the [rst]key and clear the alarm. this error may occur due to machine failure and the like. contact the nearest service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6340

สาเหตุ: the line filter's coolant through spindle differential pressure switch turned on.

การแก้ไข: press the [rst]key and clear the alarm. this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6850

สาเหตุ: delimiter of data in program is lost.

การแก้ไข: delete the program and reedit it after pressing [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6851

สาเหตุ: important parameter is modified.

การแก้ไข: press [rst] key

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6852

สาเหตุ: exer 10 signal comes on.

การแก้ไข: turn off the exer 10 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6853

สาเหตุ: exer 11 signal comes on.

การแก้ไข: turn off the exer 11 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6854

สาเหตุ: exer 12 signal comes on.

การแก้ไข: turn off the exer 12 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6855

สาเหตุ: exer 13 signal comes on.

การแก้ไข: turn off the exer 13 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6856

สาเหตุ: exer 14 signal comes on.

การแก้ไข: turn off the exer 14 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6857

สาเหตุ: exer 15 signal comes on.

การแก้ไข: turn off the exer 15 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6858

สาเหตุ: exer 16 signal comes on.

การแก้ไข: turn off the exer 16 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6859

สาเหตุ: exer 17 signal comes on.

การแก้ไข: turn off the exer 17 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6860

สาเหตุ: exer 18 signal comes on.

การแก้ไข: turn off the exer 18 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6861

สาเหตุ: exer 19 signal comes on.

การแก้ไข: turn off the exer 19 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6862

สาเหตุ: exer 20 signal comes on.

การแก้ไข: turn off the exer 20 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6863

สาเหตุ: exer 21 signal comes on.

การแก้ไข: turn off the exer 21 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6864

สาเหตุ: exer 22 signal comes on.

การแก้ไข: turn off the exer 22 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6865

สาเหตุ: exer 23 signal comes on.

การแก้ไข: turn off the exer 23 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6866

สาเหตุ: exer 24 signal comes on.

การแก้ไข: turn off the exer 24 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6867

สาเหตุ: exer 25 signal comes on.

การแก้ไข: turn off the exer 25 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6868

สาเหตุ: data on used tools are modified during operation.

การแก้ไข: press [rst] key and restart operation.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6869

สาเหตุ: resetting atc arm was attempted by pressing the [relse] and [atc] keys.

การแก้ไข: check the magazine tool and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6870

สาเหตุ: the s command is not given during the spindle rotation, or the commanded value exceeds the allowable range.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6871

สาเหตุ: setting of user parameters 6 (z measurement) is faulty.

การแก้ไข: check the setting of user parameters 6 (z measurement).

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6872

สาเหตุ: cause 1) the filter is clogged. 2) the coolant in the clean tank has fallen below the minimum required level.

การแก้ไข: 1) replace the filter. 2) check the coolant level of the 200 l tank and replenish coolant if necessary.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6873

สาเหตุ: for table movement type, the mass loaded on the table exceeds [table loaded mass] + mass error tolerance. for column movement type, the current value of the c-axis (quick table) command exceeds the tolerance.

การแก้ไข: check that the mass loaded on the table is the same as the setting for [table loaded mass] (user parameter).

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 6874

สาเหตุ: the back washing filter's differential pressure switch turned on.

การแก้ไข: press the [rst]key and clear the alarm. this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7104

สาเหตุ: exer 10 signal comes on.

การแก้ไข: turn off the exer 10 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7105

สาเหตุ: exer 11 signal comes on.

การแก้ไข: turn off the exer 11 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7106

สาเหตุ: exer 12 signal comes on.

การแก้ไข: turn off the exer 12 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7107

สาเหตุ: exer 13 signal comes on.

การแก้ไข: turn off the exer 13 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7108

สาเหตุ: exer 14 signal comes on.

การแก้ไข: turn off the exer 14 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7109

สาเหตุ: exer 15 signal comes on.

การแก้ไข: turn off the exer 15 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7110

สาเหตุ: exer 16 signal comes on.

การแก้ไข: turn off the exer 16 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7111

สาเหตุ: exer 17 signal comes on.

การแก้ไข: turn off the exer 17 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7112

สาเหตุ: exer 18 signal comes on.

การแก้ไข: turn off the exer 18 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7113

สาเหตุ: exer 19 signal comes on.

การแก้ไข: turn off the exer 19 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7114

สาเหตุ: exer 20 signal comes on.

การแก้ไข: turn off the exer 20 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7115

สาเหตุ: exer 21 signal comes on.

การแก้ไข: turn off the exer 21 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7116

สาเหตุ: external error 22

การแก้ไข: exer 22 signal comes on. turn off the exer 22 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7117

สาเหตุ: exer 23 signal comes on .

การแก้ไข: turn off the exer 23 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7118

สาเหตุ: exer 24 signal comes on.

การแก้ไข: turn off the exer 24 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7119

สาเหตุ: exer 25 signal comes on.

การแก้ไข: turn off the exer 25 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7120

สาเหตุ: an error has occurred in the seq unit connected to nc board connector seq.

การแก้ไข: refer to the instruction manual of the connected seq unit eliminate the cause and turn on power again.

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7121

สาเหตุ: the pressure sensor was not off when oil supply was attempted.

การแก้ไข: press the [rst] key. this error may occur due to machine failure and the like. if the error still occurs, contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7122

สาเหตุ: the pressure sensor did not turn on within the time set for [automatic oiling monitoring time] in the machine parameter (system 1) after the oiling pump was turned on.

การแก้ไข: press the [rst] key. this error may occur due to machine failure and the like. if the error still occurs, contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7123

สาเหตุ: insufficient oil input signal was turned on.

การแก้ไข: supply oil.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7400

สาเหตุ: areas for directories and data are insufficient.

การแก้ไข: if you are editing, reduce the current program capacity before storing it or delete other programs to create greater free memory area and edit the program again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7401

สาเหตุ: when formatting or deleting all data banks, [parameter change] was set to [no].

การแก้ไข: when deleting machine parameters, set [parameter change] to [yes] on the [i/o] screen.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7402

สาเหตุ: the outer door was opened while the door interlock was valid for the ce specification machine. (*-axis (4 axis) servomotor power was turned off.)

การแก้ไข: turn off power and see 13.6 resetting atc.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7403

สาเหตุ: the outer door was opened while the door interlock was valid for the ce specification machine. (*-axis (5 axis) servomotor power was turned off.)

การแก้ไข: turn off power and see 13.6 resetting atc.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7404

สาเหตุ: the outer door was opened while the door interlock was valid for the ce specification machine. (*-axis (6 axis) servomotor power was turned off.)

การแก้ไข: turn off power and see 13.6 resetting atc.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7500

สาเหตุ: an attempet is made to start the mdi operation during the halt of memory operation. the magazine tool data was attempted to be set during memory operation. copying, detection, formatting, or changing data number was attemped on the screen during memory operation. editing data bank was attemped during memory operation.

การแก้ไข: execute the mdi operation after the memory operation is finished. complete memory operation and then set the data. complete memory operation and then perform the desired operation.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7501

สาเหตุ: an attempt is made to start the memory operation during the halt of mdi operation. the magazine tool data was attempted to be set during mdi operation. editing data bank was attempted during mdi operation.

การแก้ไข: execute the memory operation after the mdi operation is finished. complete mdi operation and then set the data. complete mdi operation and then edit the data bank.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7502

สาเหตุ: manual mode operation is performed during manual pulse generator mode.

การแก้ไข: set to off axis selector switch of manual pulse generator.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7503

สาเหตุ: the workpiece counter has attained the end notice number.

การแก้ไข: -

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7504

สาเหตุ: the workpiece counter has attained the end notice number.

การแก้ไข: -

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7505

สาเหตุ: the workpiece counter has attained the end notice number.

การแก้ไข: -

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7506

สาเหตุ: the workpiece counter has attained the end notice number.

การแก้ไข: -

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7507

สาเหตุ: attempt of runnung the machine while the door is open and the interlock unit is on.

การแก้ไข: close the door or set the interlock unit to off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7508

สาเหตุ: the mode is changed during center aliginment.

การแก้ไข: finish the center alignment operation.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7509

สาเหตุ: an attempt is made to execute the center alignment in the machine lock.

การแก้ไข: turn off the respective lock status before executing the center alignment.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7510

สาเหตุ: an attempt is made to return to m.z. point during the machine lock.

การแก้ไข: release the machine lock before executing the zero point return.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7511

สาเหตุ: the door was opened during spindle rotation (the door interlock is effective).

การแก้ไข: close the door and restart the operation. the spindle starts rotating from the position when the operation was stopped.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7512

สาเหตุ: tap time constant set by m code was smaller than the minimum tap time constant operable for tap rotation.

การแก้ไข: set the time constant by m code appropriately for tap rotation.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7513

สาเหตุ: a memory operation is commanded while the pallet is not indexed.

การแก้ไข: change to the manual mode and index pallet with [c index] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7514

สาเหตุ: the inner door is opened in the operation.

การแก้ไข: close the inner door and push [can] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7515

สาเหตุ: the c axis is going to rotate while the safety switch is on and the outer door is open. or machine is going to be operated while the pallet is not indexed.

การแก้ไข: close the outer door, or turn off the safety switch.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7516

สาเหตุ: the inner pallet is going to carry out before take off the outer pallet work.

การแก้ไข: close the outer pallet work and press [pallet start] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7517

สาเหตุ: the program different from the one set to user parameter (quick table) was selected in memory operation mode.

การแก้ไข: check the pallet program number set to user parameter (quick table).check the parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7518

สาเหตุ: when the pallet beyond its stroke, the [z. rtn] or [p. idx] key was pressed.

การแก้ไข: press the [+c] or [-c] key to move the c-axis between -180.000 degrees and 0.000 degrees, and retry operation.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7519

สาเหตุ: life of tool used in machining is over.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7520

สาเหตุ: swichching the mode was attempted during zero position measurement.

การแก้ไข: complete zero position measurement.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7521

สาเหตุ: m406 signal was active during a memory operation was executed.

การแก้ไข: output m407 signal in mdi operation mode (block operation) to turn the m406 signal off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7522

สาเหตุ: tool life has reached the preset value, the figure in brackets() indicating the tool number.

การแก้ไข: -

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7523

สาเหตุ: data beyond the specified range is set.

การแก้ไข: set the data within the specified range.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7524

สาเหตุ: an attempt is made to start operations or automatic setting without 	executing m.z. return.

การแก้ไข: -

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7525

สาเหตุ: an attempt is made to set the program no. which is not allowed.

การแก้ไข: set the program no. correctly.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7526

สาเหตุ: the program or data bank to be copied does not exist.

การแก้ไข: check the program no. or data bank no.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7527

สาเหตุ: a program or data bank with an identical no. to those to be copied already exists.

การแก้ไข: copy to a different no. or delete the existing program or the data bank.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7528

สาเหตุ: an attempt is made to modify the data while the protect sw is on.

การแก้ไข: set the protect switch to off, and modify the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7529

สาเหตุ: number of characters exceeds 128.

การแก้ไข: reduce a number of characters.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7530

สาเหตุ: an attempt is made to edit or delete the file concerning the program in operation.

การแก้ไข: execute it after finishing the operation.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7531

สาเหตุ: the additional axis select switch was not set after power was turned on.

การแก้ไข: set the switch correctly.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7532

สาเหตุ: when the program or data is to be modified, the same data is in communication.

การแก้ไข: execute it after finishing the communication.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7533

สาเหตุ: there is no specified program or data.

การแก้ไข: check the program no. or data no.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7534

สาเหตุ: the data desired for communication is currently being used.

การแก้ไข: stop operation and start communication.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7535

สาเหตุ: the data desired for communication is currently being edited.

การแก้ไข: complete editing and start communication.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7536

สาเหตุ: [7 bit] is set to [character] and era is set to [trans data code] for communication parameter.

การแก้ไข: check the parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7537

สาเหตุ: dr signal was turned off during communication. (only when dr signal check in parameter 4 is effecive.)

การแก้ไข: check the parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7538

สาเหตุ: cs signal was off during output of dc1 and dc3 codes.

การแก้ไข: check the parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7539

สาเหตุ: receiving buffer has over flown (256 bytes).

การแก้ไข: check the parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7540

สาเหตุ: parity error has occurred during data entry.

การแก้ไข: check the parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7541

สาเหตุ: overrun error (data skip) has occurred during data entry.

การแก้ไข: check the parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7542

สาเหตุ: framing error (stop bit was not detected) occurred during data entry.

การแก้ไข: check the parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7544

สาเหตุ: data could not be received or transmitted during the time preset to response monitoring time of the parameter.

การแก้ไข: check the communication parameter or data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7545

สาเหตุ: the header, data portion, or footer cannot be recognized.

การแก้ไข: check the format.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7546

สาเหตุ: the format of the received header is not correct.

การแก้ไข: check the data name.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7547

สาเหตุ: the data name of the received header is not correct.

การแก้ไข: check the response header.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7548

สาเหตุ: the received response header is not correct for the command requested through communication.

การแก้ไข: check any cause originating in the connected communication equipment.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7549

สาเหตุ: the response of error end was received for the command that has been sent.

การแก้ไข: check the communication parameter or data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7548_2 (หมายเหตุ: ในคู่มือมีรหัส 7548 ซ้ำสองจุด)

สาเหตุ: the contents of the machine data differ from those of input data.

การแก้ไข: check the format.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7550

สาเหตุ: invalid symbol was detected in the received data, thus the corresponding record was ignored.

การแก้ไข: check the symbol in the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7551

สาเหตุ: the symbol in the received data is not in chronological order, thus the corresponding record was ignored.

การแก้ไข: check the order of symbols in the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7552

สาเหตุ: the number of items in one record of the received data is not correct. excessive items are ignored. nothing is set for missing items.

การแก้ไข: check the record format.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7553

สาเหตุ: the range of the item of the received data is not correct, thus changed. nothing is set for the changed portion.

การแก้ไข: check the value of each item.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7554

สาเหตุ: edit error has occurred during processing of communication data.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7555

สาเหตุ: the saved data format is not correct.

การแก้ไข: edit the corresponding data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7556

สาเหตุ: the number of loaded directories has exceeded 256.

การแก้ไข: the number of directories to be loaded should be up to a maximum of 256.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7557

สาเหตุ: the number of data in one block is incorrect.

การแก้ไข: check the communication protocol.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7558

สาเหตุ: the data that cannot be converted was received.

การแก้ไข: check the communication protocol.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7560

สาเหตุ: overwriting is prohibited for communication with slave station.

การแก้ไข: check the communication parameter.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7561

สาเหตุ: the program indicated in brackets () was sent to an external device.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7562

สาเหตุ: the data indicated in brackets () was sent to an external device.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7564

สาเหตุ: the program indicated in brackets () was received from an external device .

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7565

สาเหตุ: the data indicated in brackets () was received from an external device.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7566

สาเหตุ: the checksum of receiving response is not correct.

การแก้ไข: set the checksum value correctly or set the checksum of user parameter (communication) to [0: invalid).

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7567

สาเหตุ: error signal was input from the external plc.

การแก้ไข: check the conditions that external plc alarm occur eliminate the cause.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7568

สาเหตุ: error signal was input from the external plc.

การแก้ไข: check the conditions that external plc alarm occur eliminate the cause.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7569

สาเหตุ: exer10 signal comes on.

การแก้ไข: turn off the exer10 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7570

สาเหตุ: exer11 signal comes on.

การแก้ไข: turn off the exer11 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7571

สาเหตุ: exer12 signal comes on.

การแก้ไข: turn off the exer12 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7572

สาเหตุ: exer13 signal comes on.

การแก้ไข: turn off the exer13 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7573

สาเหตุ: exer14 signal comes on.

การแก้ไข: turn off the exer14 and push [ rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7574

สาเหตุ: exer15 signal comes on.

การแก้ไข: turn off the exer15 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7575

สาเหตุ: exer16 signal comes on.

การแก้ไข: turn off the exer16 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ:  alarm 7576

สาเหตุ: exer17 signal comes on.

การแก้ไข: turn off the exer17 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7577

สาเหตุ: exer18 signal comes on.

การแก้ไข: turn off the exer18 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7578

สาเหตุ: exer19 signal comes on.

การแก้ไข: turn off the exer19 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7579

สาเหตุ: exer20 signal comes on.

การแก้ไข: turn off the exer20 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7580

สาเหตุ: exer21 signal comes on.

การแก้ไข: turn off the exer21 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7581

สาเหตุ: exer22 signal comes on.

การแก้ไข: turn off the exer22 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7582

สาเหตุ: exer23 signal comes on.

การแก้ไข: turn off the exer23 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7583

สาเหตุ: exer24 signal comes on.

การแก้ไข: turn off the exer24 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7584

สาเหตุ: exer25 signal comes on.

การแก้ไข: turn off the exer25 and push [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7586

สาเหตุ: editing contents have exceeded the editing capacity (64 kbytes). when this error occurs when starting editing. this means the program is too large to edit.

การแก้ไข: the memory area cannot be increased during editing, thus divide the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7587

สาเหตุ: the spindle lock signal is on.

การแก้ไข: turn it off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7588

สาเหตุ: the atc lock signal is on.

การแก้ไข: turn it off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7589

สาเหตุ: the xy-axis lock signal is on.

การแก้ไข: turn it off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7590

สาเหตุ: the z-axis lock signal is on.

การแก้ไข: turn it off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7591

สาเหตุ: the axis 4 lock signal is on.

การแก้ไข: turn it off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7592

สาเหตุ: the axis 5 lock signal is on.

การแก้ไข: turn it off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7593

สาเหตุ: the axis 6 lock signal is on.

การแก้ไข: turn it off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7594

สาเหตุ: the spindle override is set to the value other than 100%.

การแก้ไข: set the spindle override to 100%.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7595

สาเหตุ: cutting feed override is set to a value other than 100%.

การแก้ไข: set the cutting feed override to 100%.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7596

สาเหตุ: rapid feed override is set to a value other than 100%.

การแก้ไข: set the rapid feed override to 100%.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7597

สาเหตุ: mode changeover prohibition signal is on.

การแก้ไข: turn the signal off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7598

สาเหตุ: the door open/close switch was pressed while the door was locked.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7599

สาเหตุ: the beam of the area sensor was obstructed while the door was opening or closing.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7600

สาเหตุ: the tool attempted to be set to [magazine tool] is already set for another pot.

การแก้ไข: delete the current tool and set a new one.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7601

สาเหตุ: the tool necessary for operation is not set in the magazine.

การแก้ไข: set the correct tool on the screen.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7602

สาเหตุ: deleting and copying the data bank or changing the data bank number was attempted on the screen during data bank editing.

การแก้ไข: complete the data bank currently being edited.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7603

สาเหตุ: replacement with a tool of incorrect type was attempted in manual operation mode.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7604

สาเหตุ: 1. setting other than "attachment of cap" was attempted for a prohibited pot on the screen. 2. the tool was attempted to be changed with a prohibited pot.

การแก้ไข: 1. only "attachment of cap" can be set for a prohibited pot.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7605

สาเหตุ: the tool was attempted to be replaced with a pot with the cap specified.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7606

สาเหตุ: setting more than 30 tools per tool group was attempted.

การแก้ไข: up to 30 tools can be set for one group.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7607

สาเหตุ: although a tool is set in the adjacent pot, a large tool was attempted to be set.

การแก้ไข: delete the tool set to the adjacent pot or set the large tool in another location.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7608

สาเหตุ: deletion was attempted at an incorrect position.

การแก้ไข: move the cursor to the correct position.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7609

สาเหตุ: automatic setting (parameter) was attempted although the measurement data does not exist.

การแก้ไข: perform measurement using automatic centering function.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7611

สาเหตุ: 1. changing mode was attempted during automatic tool length measurement. 2. the [m. lck] key was pressed.

การแก้ไข: complete automatic tool length measurement.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7612

สาเหตุ: start data was attempted to be set during jog movement in atc maintenance mode.

การแก้ไข: stop jog movement and set the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7615

สาเหตุ: although an optional axis 4 is not provided, axis 4 operation was attempted to be performed.

การแก้ไข: purchase an optional axis 4. when already provided, set [4 option] to [1: yes].

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7616

สาเหตุ: although an optional axis 5 is not provided, axis 5 operation was attempted to be performed.

การแก้ไข: purchase an optional axis 5. when already provided, set [5 option] to [1: yes].

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7617

สาเหตุ: although an optional axis 6 is not provided, axis 6 operation was attempted to be performed.

การแก้ไข: purchase an optional axis 6. when already provided, set [6 option] to [1: yes].

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7618

สาเหตุ: key entry cannot be accepted because key operation prohibition signal is on.

การแก้ไข: turn the signal off and then perform key entry.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7619

สาเหตุ: changing data was attempted while the program edit signal was on.

การแก้ไข: turn the signal off and then change the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7620

สาเหตุ: changing data was attempted while the edit prohibition signal was on.

การแก้ไข: turn the signal off and then change the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7621

สาเหตุ: setting of user parameter 3 (communication) is incorrect.

การแก้ไข: check the setting.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7622

สาเหตุ: the machine was attempted to be operated while the side door was open.

การแก้ไข: close the door and then operate the machine.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7623

สาเหตุ: data structure of the non-selected data banks is damaged.

การแก้ไข: delete the damaged data on the screen in edit mode, and create new data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7624

สาเหตุ: setting data was attempted for a location where data cannot be set.

การแก้ไข: set the data in other locations.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7625

สาเหตุ: the command received during communication with the tc slave resulted in an error.

การแก้ไข: check the cause and send the command again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7626

สาเหตุ: an error occurred in the interface with communication device.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7627

สาเหตุ: the version of the data attempted to be transmitted differs from that of system data version.

การแก้ไข: edit the data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7628

สาเหตุ: communication command indicated in brackets () was ended.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7629

สาเหตุ: the data indicated in brackets () was deleted during communication.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7630

สาเหตุ: the data indicated in brackets () was deleted during communication.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7634

สาเหตุ: although input of machine parameter was attempted through communication, [parameter change] was set to [no].

การแก้ไข: when inputting the machining parameter, select 1. input/output on the < /o> screen, and set [parameter change] to [yes].

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7635

สาเหตุ: the pot is not at the upper end.

การแก้ไข: press the [atc] key to move the pot to the upper end.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7636

สาเหตุ: changing to another mode was attempted during tool change.

การแก้ไข: the mode cannot be changed during tool change.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7637

สาเหตุ: the screen was attempted to be opened although the automatic thermal displacement compensation function is not provided.

การแก้ไข:

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7638

สาเหตุ: the c-axis is operating when the c-axis start point data is cleared.

การแก้ไข: clear the data after the c-axis has stopped.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7639

สาเหตุ: the door of the control box is open.

การแก้ไข: close the door of the control box.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7996

สาเหตุ: an error has occurred in the main cpu operating system.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7997

สาเหตุ: an error has occurred in the slave cpu operating system.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7998

สาเหตุ: an error has occurred in the local cpu operating system.

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 7999

สาเหตุ:

การแก้ไข: this error may occur due to machine failure and the like. contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8200

สาเหตุ: there is no data of working zero position.

การแก้ไข: create the data on working zero position in the data setting mode or input it externally.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8202

สาเหตุ: there is no tool data.

การแก้ไข: create the tool data in the data setting mode. or input it externally.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8203

สาเหตุ: there is no macro parameter data. working coordinate zero position data was damaged.

การแก้ไข: create a macro parameter data or input externally. press rst key, and data by memory screen new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8204

สาเหตุ: tool data was damaged.

การแก้ไข: press [rst] key, and data by memory screen new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8205

สาเหตุ: macro parameter data was damaged.

การแก้ไข: press [rst] key, and data by memory screen new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8206

สาเหตุ: the structure of the working coordinate zero position data was damaged.

การแก้ไข: press [rst] key, and data by memory screen new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8207

สาเหตุ: the structure of the tool data was damaged.

การแก้ไข: press [rst] key, and data by memory screen new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8208

สาเหตุ: the structure of the macro parameter data was damaged.

การแก้ไข: press [rst] key, and data by memory screen new data again.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8209

สาเหตุ: 1) the tool dia offset start-up or cancel motions are by the circular arc command. 2) a command that cannot be used in tool diameter offset mode was used.

การแก้ไข: press the [rst] key and change this motions according to the linear command. check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8210

สาเหตุ: the programmed radius is smaller than the dia offset amount during the inner circular arc cutting.

การแก้ไข: press the [rst] key and change the programmed radious to be more

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8211

สาเหตุ: the r or z point is not commanded in the canned cycle.

การแก้ไข: press the [rst] key and command the r and z points.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8212

สาเหตุ: the tapping pitch is set to 0 or not set.

การแก้ไข: press the [rst] key and set the pitch data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8213

สาเหตุ: the specified sequence no. is not found.

การแก้ไข: press the [rst] key and check the specified sequence no.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8214

สาเหตุ: the number of tools used for the program is larger than the number of tool pots.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8215

สาเหตุ: necessary address is not found in canned cycle, or unnecessary address was specified.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8216

สาเหตุ: necessary address is not found in zero position measurement command or unnecessary address was specified.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8217

สาเหตุ: thermal measurement command or z mesrment comd er tool breakage detection command format is incorrect.

การแก้ไข: press the (rst key and check the program,

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8218

สาเหตุ: the format specified by the macro command is incorrect.

การแก้ไข: press the rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8219

สาเหตุ: thermal compensation was attempted although thermal measurement was not performed.

การแก้ไข: refer to the reference program for thermal measurement.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8220

สาเหตุ: the "t" command is out of the specified range.

การแก้ไข: check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8221

สาเหตุ: program size is larger than buffer capacity

การแก้ไข: press the [rst] key and correct the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8223

สาเหตุ: tape run was attempted although the machine is not equipped with tape run option.

การแก้ไข: purchase the tape run option.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8700

สาเหตุ: 3-digit g code which is not available is specified.

การแก้ไข: press the [reset] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8701

สาเหตุ: g and m codes are commanded in the same block, though it is not allowable.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8702

สาเหตุ: 3-digit m code which is not available is specified.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8703

สาเหตุ: characters not accepted are commanded.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8704

สาเหตุ: more than one m code is commanded in a block.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8705

สาเหตุ: the command format is not correct.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8706

สาเหตุ: commanded value exceeds address range.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8707

สาเหตุ: no. of characters in one block exceeds 128.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8708

สาเหตุ: g and m codes which cannot be intervened mutually in the mdi mode are commanded in the memory operation.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8709

สาเหตุ: there is no m02 or m30 command.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8710

สาเหตุ: wrong command is given in the circular arc command.

การแก้ไข: press the [reset] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8711

สาเหตุ: the subprogram nesting exceeds 8 loops.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8712

สาเหตุ: the subprogram which can not be called is called.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8713

สาเหตุ: g66 was commanded in mode other than macro modal call mode or tool dia offset was not canceled when returning from the sub program.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8714

สาเหตุ: the f command is not given during the cutting feed, or the commanded value exceeds the limit.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8715

สาเหตุ: the dr signal turned off during communication.

การแก้ไข: check the communication parameter or the connection object.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8716

สาเหตุ: the cross point is not obtained during the inner cutting of tool dia offset.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8717

สาเหตุ: necessary address is not found when the coordinate calculation is commanded, or the value exceeding the allowable range is commanded.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8718

สาเหตุ: the coordinate calculation is commanded in the circular arc mode.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8719

สาเหตุ: the control out/in codes are not in the same block.

การแก้ไข: press the [reset] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8720

สาเหตุ: the t command is not given while g100 or m06 is in execution.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8721

สาเหตุ: the return number was specified by the program return command while the number of repeats was specified for sub program call.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8722

สาเหตุ: the macro program call nesting exceeds 4 loops.

การแก้ไข: press the [rst] key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8723

สาเหตุ: the sequence no. specified by the sub program return command or that specified by the macro command (goto) was not found.

การแก้ไข: press the rst key and check the program.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8724

สาเหตุ: tape run was attempted when [connection object] of [user parameter (communication)] was set to [computer].

การแก้ไข: set [connection object] to [ptp/ptr].

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8725

สาเหตุ: tape run was attempted when the external program signal was on.

การแก้ไข: turn the external program signal off.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8726

สาเหตุ: communication was attempted while the communication port was recovering from an error.

การแก้ไข: wait until the communication recovery time elapses, and press the [rst] key.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8727

สาเหตุ: non-response time exceeded response monitor time during communication.

การแก้ไข: check the communication parameter or the connection object.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8729

สาเหตุ: the tc received invalid data when the invalid data conversion function was off.

การแก้ไข: check the communication parameter or the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8730

สาเหตุ: communication header, data section, or footer not recognized.

การแก้ไข: check the communication parameter or the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8731

สาเหตุ: the number of characters in one block is not correct when receiving the program.

การแก้ไข: check the communication parameter or the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8732

สาเหตุ: the cs signal was off when sending flow control dc1 and dc3 codes.

การแก้ไข: check the communication parameter or the communication object.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8733

สาเหตุ: capacity of the receiving buffer is exceeded

การแก้ไข: check the communication parameter or the communication object.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8734

สาเหตุ: the stop bit of the receiving character could not be detected.

การแก้ไข: check the communication parameter or the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8735

สาเหตุ: parity error occurred in the receiving characters.

การแก้ไข: check the communication parameter or the data.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8736

สาเหตุ: the next characters came in although the tc was not ready to receive them. tape run was attempted in master station communication mode.

การแก้ไข: check the communication parameter or the data. exit the external i/o menu, and then perform tape run.

=== END ===

=== MACHINE: BROTHER TCS2A  ===

อาการ: alarm 8737

สาเหตุ: a program different from the one set for [user parameter (quick table)] was set for tape run.

การแก้ไข: check the program number of each pallet set for [user parameter (quick table) or the program number to be operated.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9450

สาเหตุ: specified program is not found. figures in () indicate the program no.

การแก้ไข: check the program no.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9451

สาเหตุ: specified subprogram is not found. figures in () indicate the subprogram no.

การแก้ไข: set the subprogram, or delete the subprogram call cycle.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9452

สาเหตุ: specified sequence no. is not found.

การแก้ไข: check the specified sequence no.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9453

สาเหตุ: the sequence no. in the program is not correct.

การแก้ไข: check the sequence no. in the program.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9454

สาเหตุ: the subprogram no. in the program is not correct.

การแก้ไข: check the subprogram no. in the program.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9455

สาเหตุ: measuring direction is not set.

การแก้ไข: set measuring direction.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9456

สาเหตุ: the data other than numerical values are set to [center alignment] or [automatic tool length measurement).

การแก้ไข: set the data correctly.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9457

สาเหตุ: the group number was attempted to be set although the tool number was not set to [magazine tool].

การแก้ไข: set the tool number and then group number.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9458

สาเหตุ: mdi operation was attempted to be executed during memory operation.

การแก้ไข: cancel the coordinate rotation and execute mdi operation.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9459

สาเหตุ: restart or sequence search was attempted while the pallet select switch is on.

การแก้ไข: turn off the pallet select switch and retry.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9460

สาเหตุ: the mdi was operated while memory operation of the corner c/r was operating.

การแก้ไข: start the mdi operation when the corner c/r operation was finished.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9461

สาเหตุ: the number of blocks to be loaded or deleted is too large.

การแก้ไข: delete the number of such blocks.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9462

สาเหตุ: the character string to be searched for or replaced was not found.

การแก้ไข: change the character string to be searched for or replaced.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9463

สาเหตุ: deleting or copying the data bank was attempted during memory operation.

การแก้ไข: complete the memory operation first and then delete or copy the data bank.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9464

สาเหตุ: changing the data or executing mdi operation was attempted while restarting.

การแก้ไข: execute it after finishing the restart motions.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9465

สาเหตุ: master station communication was attempted during tape run.

การแก้ไข: try communication after tape run is completed.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9466

สาเหตุ: if you are editing, reduce the current program capacity before storing it or delete other programs to create greater free memory area, and edit the program again.

การแก้ไข: reduce the program capacity and store it during editing. or delete other programs to make a free area and edit it again.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9467

สาเหตุ: a tool is not set to the group that was attempted to be set to [magazine tool].

การแก้ไข: check the tool group setting.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9468

สาเหตุ: tape run is completed.

การแก้ไข:-

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ: alarm 9469

สาเหตุ: tape run was attempted when [restart] of [user parameter (switch 1)] was set to [restart].

การแก้ไข: set [restart] of [user parameter (switch 1)] to [stop].

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo oc 1

สาเหตุ:  overcurrent

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo aoh 2

สาเหตุ:  register db in the amplifier is overheating.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo eoh 3

สาเหตุ:  the external regenerative register is overheating.

การแก้ไข:  turn off the power supply. after 15 to 20 minutes, condition.

if the error still occurs, it may be due to machine failure and the like contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo ov 5

สาเหตุ:  over voltage

การแก้ไข:  check power supply voltage, capacity, momentary service interruption, and correct them as necessary.

if the error still occurs, it may be due to machine failure and the like.

contact the nearest brother approved device dealer.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo cpe 6

สาเหตุ:  controller power supply has decreased.

การแก้ไข:  check power supply voltage, capacity, momentary service interruption, and correct them as necessary.

if the error still occurs, it may be due to machine failure and the like.

contact the nearest brother approved device dealer.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo de1 7

สาเหตุ:  encoder signal line is broken.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo de2 8

สาเหตุ:  serial broken wire

การแก้ไข:  main power supply voltage drop.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo ol 9

สาเหตุ:  overloaded

การแก้ไข:  turn off the power supply. after 15 to 20 minutes, condition.

if the error still occurs, it may be due to machine failure and the like contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo os 10

สาเหตุ:  overseed

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo mpe 11

สาเหตุ:  main power error

การแก้ไข:  check power supply voltage, capacity, momentary service interruption, and correct them as necessary.

if the error still occurs, it may be due to machine failure and the like.

contact the nearest brother approved device dealer.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo fp 12

สาเหตุ:  main circuit open phase

การแก้ไข:  check power supply voltage, capacity, momentary service interruption, and correct them as necessary.

if the error still occurs, it may be due to machine failure and the like.

contact the nearest brother approved device dealer.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo rge 13

สาเหตุ:  regenerative register is overloaded.

การแก้ไข:  turn off the power supply. after 15 to 20 minutes, condition.

if the error still occurs, it may be due to machine failure and the like contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo dspe 14

สาเหตุ:  dsp error

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo romo 15

สาเหตุ:  memory error

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo pare 16

สาเหตุ:  servo parameter error

การแก้ไข:  check the following servo parameter setting

motor type (*13)

limit of current while preventing drop due to dead load (*18)

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo batt drop 17

สาเหตุ:  battery voltage drop.

การแก้ไข:  replace alkaline battery for absolute encoder backup.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo cse 18

สาเหตุ:  encoder error in initial stat.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo moc 19

สาเหตุ:  motor over current

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo dto 20

สาเหตุ:  command monitor timer has expired (no command has been received from the amplifier within 4 msec)

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo dto 21

สาเหตุ:  response monitor timer has expired (no command has been received from the amplifier within 4 msec)

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo sys 22

สาเหตุ:  servo error signal was input but alarm number was not notified.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo sys1 23

สาเหตุ:  servo error signal was input but undefined alarm number was not notified.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo ini 24

สาเหตุ:  communication between the nc and amplifier was impossible when the power was turned on.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo ss 25

สาเหตุ:  already transmitting when the data was attempted to be transferred to be amplifier.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo sr 26

สาเหตุ:  already received when the date was attempted to be transferred to be amplifier.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo crce 27

สาเหตุ:  crc error

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo frme 28

สาเหตุ:  framing error

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo vere1 29

สาเหตุ:  output data amplifier was incorrect (data bank of servo amplifier 3,4,5)

การแก้ไข:  this error may occur when power is tuned on the first time after replacing the servo amplifier. turn the power off and then on again.

if the error still occurs, check the following servo parameter settings.

if the error still occurs, it may be due to machine failure and the like. contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo ato 30

สาเหตุ:  amplifier timeout error.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo rstm 31

สาเหตุ:  amplifier rst input has decreased during servo on.

การแก้ไข:  check power supply voltage, capacity, monetary service interruption, and correct them as necessary.

if the error still occurs, it may be due to machine failure and the like. contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo sce 32

สาเหตุ:  crc error for synchronized frame.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo recd 33

สาเหตุ:  encoder signal reading impossible error.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo veri 34

สาเหตุ:  the motor type, encoder pulse, and encoder type previously set do not coincide with those set this time.

this error will occur only to the pv amplifier and py2c amplifier.

การแก้ไข:  set the following servo parameters correctly, and turn the power off and then on again. upon this operation, the [servo (veri)] message will appear again. just turn the power off and then on again

motor type (*13)

encoder pulse (*14)

encoder type (*15)

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo sre 35

สาเหตุ:  serial receiving error

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer.

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo pse 36

สาเหตุ:  encoder initial receiving error.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo vere2 37

สาเหตุ:  check the servo parameter setting

การแก้ไข:  if the error still occurs, it may be due to machine failure and the like. contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER TCS2A ===

อาการ:  alarm servo de11 38

สาเหตุ:  the encoder signal was recorded while the de1 alarm is current.

การแก้ไข:  this error occurs due to machine failure and the like contact the nearest brother approved service dealer

=== END ===

=== MACHINE: BROTHER ===

อาการ: ปุ่ม Start กดไม่ติด / เครื่องไม่ทำงาน

สาเหตุ:  น้ำเข้าที่ contact ของ switch start

การแก้ไข:  ทำความสะอาด contact ของ switch start

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Overrun X+

สาเหตุ:  Limit Switch NG

การแก้ไข:  เปลี่ยน Limit Switch ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: เจาะงานด้าน L เยื้อง Center

สาเหตุ:  Warm gear สึกหรอ

การแก้ไข:  ตรวจ Accuracy + เปลี่ยน Ball screw + Adjust backlash

=== END ===

=== MACHINE: BROTHER ===

อาการ: Magazine จับ Tool ไม่ตรง

สาเหตุ:  Board I/O PCB NG

การแก้ไข:  ตรวจ ATC + เปลี่ยน Cam Switch + เปลี่ยน Board I/O

=== END ===

=== MACHINE: BROTHER ===

อาการ: Y-axis Servo Error

สาเหตุ:  Cover เสื่อม / Chip เข้า

การแก้ไข:  ทำความสะอาดแกน Y

=== END ===

=== MACHINE: BROTHER ===

อาการ: Y Servo DE1

สาเหตุ:  Ball screw เสื่อม

การแก้ไข:  สลับ Servo Amp + อัดจารบี

=== END ===

=== MACHINE: BROTHER ===

อาการ: Door Error / Alarm จังหวะเปิด

สาเหตุ:  Reed Switch เพี้ยน

การแก้ไข:  ปรับตำแหน่ง Reed Switch

=== END ===

=== MACHINE: BROTHER ===

อาการ: Y Overrun

สาเหตุ:  Limit Switch กลางเสีย

การแก้ไข:  สลับ Limit Switch

=== END ===

=== MACHINE: BROTHER ===

อาการ: 24V overcurrent

สาเหตุ:  น้ำเข้าหลอดไฟ

การแก้ไข:  เปลี่ยนหลอดไฟ

=== END ===

=== MACHINE: BROTHER ===

อาการ: X axis มีเสียงดัง

สาเหตุ:  Amp เสีย

การแก้ไข:  เปลี่ยน Amp

=== END ===

=== MACHINE: BROTHER ===

อาการ: Cannot Clamp A

สาเหตุ:  Air Cylinder ค้าง

การแก้ไข:  ทำความสะอาด

=== END ===

=== MACHINE: BROTHER ===

อาการ: 5596 A-Axis O-Ring Index เสื่อม

สาเหตุ:  O-Ring Index เสื่อม

การแก้ไข:  เปลี่ยน O-Ring/Seal

=== END ===

=== MACHINE: BROTHER ===

อาการ: Un Clamp A

สาเหตุ:  Proximity สกปรก

การแก้ไข:  ทำความสะอาด + เปลี่ยน Solenoid

=== END ===

=== MACHINE: BROTHER ===

อาการ: alarm 5516 Deviation

สาเหตุ:  Index NG

การแก้ไข:  เปลี่ยน O-Ring, Flushing oil

=== END ===

=== MACHINE: BROTHER ===

อาการ: Conveyor Abnormal

สาเหตุ:  Conveyor ขาด

การแก้ไข:  ตัด/ซ่อม

=== END ===

=== MACHINE: BROTHER ===

อาการ: CPU Fan rotation

สาเหตุ:  Fan เสีย

การแก้ไข:  เปลี่ยน Fan + Servo Motor

=== END ===

=== MACHINE: BROTHER ===

อาการ: Main breaker trip

สาเหตุ:  Motor Index เสีย

การแก้ไข:  เปลี่ยน Motor

=== END ===

=== MACHINE: BROTHER ===

อาการ: ประตูปิดึเปิด Alarm

สาเหตุ: น๊อตยึดรางประตูขาด

การแก้ไข: ถอดประตูออก เปลี่ยนน๊อตรางประตูใหม่ ทดลองเปิด-ปิด OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: ค่างานไม่นิ่ง เล็กึโต

สาเหตุ: ค่า Runout ของ Spindle เกิน

การแก้ไข: วัดค่า Runout ใหม่, เปลี่ยน Spindle ใหม่, วัดค่า Parallel ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: ท่อน้ำฉีด Tool ไม่ไหล

สาเหตุ: Pump Coolant NG

การแก้ไข: ทำความสะอาด Y-Strainer, เปลี่ยน Pump Coolant

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm *A Clamp

สาเหตุ: น้ำมันรั่วซึม

การแก้ไข: เติมน้ำมัน Index ทดลองเดินงาน OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm *A Clamp (เติมน้ำมันแล้วไม่หาย)

สาเหตุ: ชุด Brake ของ Index อาจมีปัญหา

การแก้ไข: ถอดชุด Brake ทำความสะอาด ใช้ O-ring เดิม ประกอบคืน ทดลองรอดูอาการ

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm ประตู

สาเหตุ: น๊อตยึดกุญแจ Safety door หลุด

การแก้ไข: ขันน๊อตยึดใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: OP3 ค้าง Start ไม่ได้ / Clamp ไม่ทำงาน

สาเหตุ: Air gap sensor check workpiece ไม่ทำงาน ทำให้ Tail stock ไม่ทำงาน

การแก้ไข:เช็คสัญญาณประตู OK

Manual tail stock OK

ปิดเปิดเครื่องใหม่ยัง NG

ติดต่อ Maker มาแก้ไข  แก้สัญญาณ Air gap

เดินงานต่อเนื่อง OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm CTS Filter สกปรก

สาเหตุ:  Filter สกปรก

การแก้ไข:  ถอด Filter, ทำความสะอาด, ปรับ Pressure CTS, ทดลองผลิตงาน OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm door open error

สาเหตุ:  เหล็ก Support บานประตูหัก

การแก้ไข:  ถอด Support, เชื่อมใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm 5059.07 * X Servo (DE1)

สาเหตุ:  Servo motor X เสีย

การแก้ไข:  เปลี่ยน Servo motor X Axis, Set home position X Axis, ให้ฝ่ายผลิต Set up workpiece ใหม่, ทดลองเดินงาน OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: Motor coolant มีเสียงดังและมีกลิ่นไหม้

สาเหตุ:  Bearing ของ Motor pump coolant ชำรุด

การแก้ไข:  เปลี่ยน Motor Coolant ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: 7015* External error 11

สาเหตุ:  Pressure switch unclamp เสีย

การแก้ไข:  เปลี่ยน Pressure Switch, Set Pressure Test OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm 2225CTS Pressure has not increase

สาเหตุ:  Filter Coolant สกปรก

การแก้ไข:  เปลี่ยน Filter coolant CTS ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: น้ำมันรั่ว Pressure switch

สาเหตุ:  Seal pressure switch ชำรุด

การแก้ไข:  เปลี่ยน Pressure Switch, Set Pressure Test OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: Switch check tool ไม่ทำงาน

สาเหตุ:  Limit Switch Check Tool NG

การแก้ไข:  ทำความสะอาด Limit Switch, Test NG, เปลี่ยน Limit Switch ตัวใหม่, Test OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm robot3 ค้างในเครื่อง OP7

สาเหตุ:  ระยะ Reed switch clamp-unclamp ใกล้กัน

การแก้ไข:  ปรับตั้งตำแหน่ง Reed switch, ทดลองเดินงาน OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: CTS Pressure has not increase

สาเหตุ:  Pressure switch ทำงานค้าง

การแก้ไข:  ปรับตั้ง Pressure switch CTS ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: บานประตูชำรุด ฉีกขาด

สาเหตุ:  เสื่อมสภาพการใช้งาน

การแก้ไข:  เปลี่ยนบานประตูใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Pressure has not increased

สาเหตุ:  Pressure sw CTS ทำงานช้า

การแก้ไข:  เปลี่ยน Pressure CTS ตัวใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Overrun Y+

สาเหตุ:  Limit Switch Origin Y NG

การแก้ไข:  เปลี่ยน Limit Switch ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: ตำแหน่ง Index ไม่ตรงตำแหน่ง 0

สาเหตุ:  สาย Limit SW. Check home position ขาด

การแก้ไข:  ตัดสายและย้ำใหม่, ตั้งตำแหน่ง Zero, ทดลองหมุน Index, ทดลองเดินงาน OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: เครื่องดับเปิดไม่ติด

สาเหตุ:  Power Supply I/O VAR2 NG, Servo motor Y Axis NG

การแก้ไข:  เปลี่ยน Power Supply I/O VAR2, เปลี่ยน servo motor Y axis, ตั้งค่า origin, ทดลองเข้า Zero OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: ท่อน้ำ CTS แตก

สาเหตุ:  สายน้ำ Coolant CTS แตก

การแก้ไข:  เปลี่ยนสายน้ำ Coolant CTS ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Limit check Tool เสีย

สาเหตุ:  Limit sw. NG / สัญญาณไม่ทำงาน

การแก้ไข:  เปลี่ยนหัว Limit check tool ใหม่, ทดลองเดินงาน OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm 6195MGZN INDEX ERROR

สาเหตุ:  ตำแหน่ง MGZ ไม่ตรงตำแหน่ง

การแก้ไข:  ปรับตั้งตำแหน่ง MGZ ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Door error

สาเหตุ:  Reed SW Door เสีย/เสื่อมสภาพ

การแก้ไข:  เปลี่ยน Reed SW ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Line Filter Error

สาเหตุ:  Filter อุดตัน

การแก้ไข:  เปลี่ยน Filter ใหม่, Reset Alarm, ทดลองเดินงาน OK

=== END ===

=== MACHINE: BROTHER ===

อาการ: หน้าจอมองไม่เห็น

สาเหตุ: หน้าจอไม่แสดงตัวอักษร

การแก้ไข: เปลี่ยนสายแพรใหม่ โดยถอดจากเครื่อง Spare ที่โรง 4

=== END ===

=== MACHINE: BROTHER ===

อาการ: หน้าจอมองไม่เห็น

สาเหตุ: หน้าจอไม่แสดงตัวอักษร

การแก้ไข: เปลี่ยนหน้าจอใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm 5060.07*# Y Servo

สาเหตุ: Servo แกน Y ขัดข้อง

การแก้ไข: เปลี่ยน Motor แกน Y ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: ปั๊มน้ำยา Coolant ไม่ตัด

สาเหตุ: น้ำ Coolant ไหลไม่หยุด

การแก้ไข: เปลี่ยน Solenoid Valve ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Tool Break Detection Sensor

สาเหตุ: Limit Check Tool ผิดปกติ

การแก้ไข: ตรวจสอบการทำงาน Limit Switch และเปลี่ยน Limit Switch ใหม่ ทดสอบแล้วใช้งานได้ปกติ

=== END ===

=== MACHINE: BROTHER ===

อาการ: น้ำมัน Hydraulic รั่ว

สาเหตุ: น็อตอุด Index คลายตัว

การแก้ไข: ขันยึดน็อตให้แน่น

=== END ===

=== MACHINE: BROTHER ===

อาการ: เครื่องดับ เปิดไม่ติด

สาเหตุ: Power Supply และ I/O Board ขัดข้อง

การแก้ไข: เปลี่ยน Power Supply (AWR2) และตรวจสอบระบบไฟ ทดสอบแล้วใช้งานได้

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Twice Processing Error Prevention

สาเหตุ: ครบชั่วโมงการทำงานของอุปกรณ์

การแก้ไข: Reset Counter ใหม่

=== END ===

=== MACHINE: BROTHER ===(TC-S2D)

อาการ: Alarm CTS Pressure

สาเหตุ: Pressure Coolant ไม่ทำงาน

การแก้ไข: ปรับตั้ง Pressure Coolant

=== END ===

=== MACHINE: BROTHER ===(Robodrill-T14iFb)

อาการ: สายน้ำมัน Hydraulic แตก

สาเหตุ: สายน้ำมันเกิดการเสียดสี

การแก้ไข: เปลี่ยนสาย Hydraulic ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: External Error 19

สาเหตุ: Battery PLC ต่ำ

การแก้ไข: เปลี่ยน Battery PLC

=== END ===

=== MACHINE: BROTHER ===

อาการ: ไฟเครื่องดับ

สาเหตุ: Motor Coolant ไหม้

การแก้ไข: เปลี่ยน Motor Pump (ยืมจากเครื่องเก่า) ทดสอบใช้งานปกติ

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Pulse Error Y-Axis

สาเหตุ: Encoder Servo Motor เสีย

การแก้ไข: เปลี่ยน Servo Motor แกน Y

== = END ===

=== MACHINE: CAMCUTTER ===

อาการ: Alarm Over Outer

สาเหตุ: Roller Speed Control สึก

การแก้ไข: ปรับตั้ง Speed Control ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Breaker Trip

สาเหตุ: Motor Index ช๊อต

การแก้ไข: เปลี่ยน Motor Index ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm External Error 17

สาเหตุ: Limit Check Tool ผิดปกติ

การแก้ไข: เปลี่ยน Limit Switch Check Tool และ Set Position ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Thermal (Chip Shower)

สาเหตุ: Pump Coolant ไม่หมุน

การแก้ไข: เปลี่ยน Pump Coolant ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Over Y+

สาเหตุ: Limit Switch Overrun เสีย

การแก้ไข: เปลี่ยน Limit Switch ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Pulse Error X-Axis

สาเหตุ: Motor Axis X เสีย

การแก้ไข: เปลี่ยน Motor Axis X ใหม่ และตั้งค่า Axis X

=== END ===

=== MACHINE: BROTHER ===

อาการ: Limit Switch Check Tool พัง

สาเหตุ: แกน Z ลงเกินตำแหน่ง Check Tool

การแก้ไข: เปลี่ยน Limit Switch Check Tool ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm 5060.10 Y Servo (OS)

สาเหตุ: Housing Ball Screw Y Axis แตก

การแก้ไข: เปลี่ยน Housing Ball Screw Y Axis

=== END ===

=== MACHINE: BROTHER ===

อาการ: EXTERNAL Error 20

สาเหตุ: Limit Switch Check Tool NG

การแก้ไข: เปลี่ยน Limit Switch Check Tool

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm A Servo (OC) / 5063.xx

สาเหตุ: Motor Index หรือ Servo Amp เสีย / สายไฟชำรุด

การแก้ไข: เปลี่ยน Motor Index หรือ Servo Amp และตั้ง Zero ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Check Tool / External Error 20

สาเหตุ: Limit Check Tool ทำงานค้างหรือเสีย

การแก้ไข: เปลี่ยน Limit Check Tool ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: ประตูเครื่องจักรไม่เปิด / ค้าง

สาเหตุ: รางประตูเสีย น็อตคลาย หรือ Cylinder ผิดปกติ

การแก้ไข: เชื่อม/เปลี่ยนรางประตู ขันน็อต และปรับแรงลม

=== END ===

=== MACHINE: BROTHER ===

อาการ: เครื่องดับ เบรกเกอร์ทริป

สาเหตุ: Motor Index มีปัญหา

การแก้ไข: เปลี่ยน Motor Index ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Pulse Error / Overrun (+Y) / External Error 11 / Clamp Error

สาเหตุ:

1.	Servo Motor NG

2.	Limit Switch / Pressure Switch ค้างหรือเสีย

3.	Solenoid Valve ขาดการหล่อลื่น

การแก้ไข:

1.	เปลี่ยน Servo Motor และ Set Origin

2.	เปลี่ยน Limit / Pressure Switch

ทำความสะอาดหรือเปลี่ยน Solenoid Valve

=== END ===

=== MACHINE: BROTHER ===

อาการ: ท่อน้ำแตก ไฟฟ้าช็อต

สาเหตุ: สายน้ำ Coolant แตกไปโดน Fan Motor

การแก้ไข: อุดรอยรั่ว เปลี่ยน Fan Motor Spindle

=== END ===

=== MACHINE: BROTHER ===(TC-S2D)

อาการ: Alarm 5061 Z Servo

สาเหตุ: Servo Amp แกน Z ผิดปกติ

การแก้ไข: เปลี่ยน Servo Amp และตั้ง Zero ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm 5058 SP Servo (OC)

สาเหตุ: Spindle Servo Amp เสีย

การแก้ไข: เปลี่ยน Spindle Servo Amp

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Overrun +Y

สาเหตุ: Limit Switch แกน Y ค้าง

การแก้ไข: เปลี่ยน Limit Switch แกน Y

=== END ===

=== MACHINE: BROTHER ===(TC-S2D)

อาการ: ก้ามปู Tool หัก

สาเหตุ: ก้ามปูชำรุด

การแก้ไข: เปลี่ยนก้ามปู Tool ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: MGZN POS ERROR / ATC Magazine ผิดตำแหน่ง

สาเหตุ:  Limit Switch ATC NG

การแก้ไข:  เปลี่ยน Limit Switch ATC และตรวจสอบการเคลื่อนที่แกน Z

=== END ===

=== MACHINE: BROTHER ===

อาการ: เปิดเครื่องไม่ติด / Breaker Trip

สาเหตุ:  กระแสเกินขณะเปิดเครื่อง

การแก้ไข:  ยก Breaker ขึ้นใหม่และเปิดเครื่องใช้งาน

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm 5058.03 Servo Overrun Z

สาเหตุ:  Limit Switch Origin Z และ Overrun -Z NG

การแก้ไข:  เปลี่ยน Limit Switch ใหม่=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Y Servo (DE1)

สาเหตุ:  Servo Motor Y Axis NG

การแก้ไข:  เปลี่ยน Servo Motor Y Axis และตั้งค่า Origin ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm 6178 Can Clamp แก้ไขไม่หาย

สาเหตุ:  Solenoid Index เสีย

การแก้ไข:  เปลี่ยน Solenoid Index ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: CTS Presser ไม่ลง

สาเหตุ:  Line Filter Coolant อุดตัน / Coolant สกปรก

การแก้ไข:  ทำความสะอาด Line Filter และทดสอบการทำงาน

=== END ===

=== MACHINE: BROTHER ===

อาการ: ประตูค้าง เปิดไม่ได้

สาเหตุ:  Safety Door เสื่อมสภาพ

การแก้ไข:  เปลี่ยน Safety Door ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm External Error 20

สาเหตุ:  Limit Switch Check Tool ทำงานค้าง

การแก้ไข:  เปลี่ยน Limit Switch Check Tool ใหม่ และปรับตั้งระยะการกด

=== END ===

=== MACHINE: BROTHER ===

อาการ: Cover Y Axis เสีย

สาเหตุ:  การเลื่อนแกนทำให้ Cover ติดขัด

การแก้ไข:  ยืม Cover จาก Line อื่นมาใช้งานชั่วคราว

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm A-axis Position Shifted

สาเหตุ:  O-ring ของชุด Index เสื่อมสภาพ

การแก้ไข:  ถอดชุด Index ทำความสะอาด และเปลี่ยน O-ring ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Magazine Tool 2 จับไม่อยู่

สาเหตุ:  Grip Assy No.2 NG

การแก้ไข:  เปลี่ยน Grip Assy ตัวใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: ประตูชำรุด ไม่สามารถเดินงานได้

สาเหตุ:  ประตูตกรางเลื่อน

การแก้ไข:  ถอด Roller และประกอบประตูเข้ารางใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm EXTERNAL ERROR 11

สาเหตุ:  Limit Switch Clamp Contact ค้าง

การแก้ไข:  เปลี่ยน Limit Switch Clamp ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Thermal Spindle Motor

สาเหตุ:  Motor Spindle อุณหภูมิสูง

การแก้ไข:  เปลี่ยน Motor Spindle ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: เครื่องเปิดไม่ติด Main Breaker Trip

สาเหตุ:  Servo Motor Y Axis ค่า MΩ เป็น 0

การแก้ไข:  เปลี่ยน Servo Motor Y Axis และตั้ง Origin ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm Overrun (Y+)

สาเหตุ:  Limit Switch Home Y Axis เสีย

การแก้ไข:  เปลี่ยน Limit Switch Home Y Axis ใหม่

=== END ===

=== MACHINE: BROTHER ===

อาการ: Alarm 5095 Pulse error (X Axis)

สาเหตุ:  Servo motor X Axis ชำรุด

การแก้ไข:  เปลี่ยน Servo motor X Axis และ Set origin X Axis

=== END ===

=== MACHINE: BROTHER ===

อาการ: CTS Presser Has Not Lower

สาเหตุ:  Filter coolant อุดตันจากเศษ Chip

การแก้ไข:  ทำความสะอาด Filter coolant

=== END ===

=== MACHINE: BROTHER TAPPING CENTER ===

อาการ: Door Error

สาเหตุ:  Safety Door สกปรก

การแก้ไข:  ทำความสะอาด Safety Door

=== END ===

=== MACHINE: BROTHER TAPPING CENTER ===

อาการ: A-Axis Position Shifted

สาเหตุ:  เม็ด Insert เสื่อมสภาพ ทำให้ Index ทำงานผิดตำแหน่ง

การแก้ไข:  เปลี่ยน Tool Insert และเปลี่ยน Cover Axis Y

=== END ===

=== MACHINE: BROTHER TAPPING CENTER ===

อาการ: Alarm Thermal Coolant

สาเหตุ:  Pump Coolant Overload เนื่องจาก Bearing เสีย

การแก้ไข:  เปลี่ยน Bearing Pump Coolant และทดสอบกระแส

=== END ===

=== MACHINE: BROTHER TAPPING CENTER ===

อาการ: Alarm Pulse Error (A-Axis)

สาเหตุ:  Servo Motor Index ชำรุด

การแก้ไข:  เปลี่ยน Servo Motor Index และตั้งค่า Zero

=== END ===

=== MACHINE: BROTHER TAPPING CENTER ===

อาการ: Servo Error

สาเหตุ:  มีน้ำเข้าในสาย Motor Index

การแก้ไข:  ทำความสะอาดสาย Motor Index

=== END ===

=== MACHINE: BROTHER TAPPING CENTER ===

อาการ: Y Servo Overload

สาเหตุ:  เศษ Chip สะสมในร่องแกน Y และ Cover ชำรุด

การแก้ไข:  ทำความสะอาดร่องแกน Y และเปลี่ยน Cover Axis Y

=== END ===

=== MACHINE: BROTHER TAPPING CENTER ===

อาการ: แกน Y สั่น ทำให้งาน Stop

สาเหตุ:  Servo Amplifier Y Axis ผิดปกติ

การแก้ไข:  สลับ Servo Amplifier ใหม่และทดสอบเครื่อง

=== END ===

=== MACHINE: Brother ===

อาการ: Alarm 5516 Deviation Error1A

สาเหตุ:  Index ทำงานแล้ว Alarm 5516 Deviation Error1A

การแก้ไข:  ทำการถอดเช็ค INDEX / ถอด Flushing น้ำมันภยาในทั้งหมด

เปลี่ยน O-ring ของ Brake Index ทั้งหมด

เปลี่ยนน้ำมันใหม่หมด

=== END ===

=== MACHINE: Brother ===

อาการ: Alarm 5596 *A-Axis Pos Shifted

สาเหตุ:  ชุด O-Ring Index เสื่อมสภาพ ทำให้เบรคทำงานได้ไม่เต็มประสิทธิภาพ

การแก้ไข:  ตรวจสอบระบบลม ระบบน้ำมันของ Index ทดลองเดินงาน 1 ตัวแล้วเกิด Alarm

-ถอดชุด Index ตรวจสอบ เบรค ตรวจสอบชุด O-Ring

Oil Seal พบมี O-Ring แข็งกระด้าง เสื่อมสภาพในการใช้งานทำให้เบรคทำงานได้ไม่เต็มประสิทธิภาพ

-เปลี่ยน ชุด O-Ring .ใหม่ และประกอบเติมน้ำมัน Index ทดลองเดินงาน 10 ตัว ไม่เกิด Alarm

=== END ===

=== MACHINE: BROTHER (-T14iF) ===

อาการ: EX1068 Pressure Rise Error (Lubrication)

สาเหตุ: สาย Lubrication แตกรั่ว

การแก้ไข: เปลี่ยนสาย Lubrication ใหม่

=== END ===

=== MACHINE: BROTHER (TC-S2D) ===

อาการ: X-Axis Servo Error CSE

สาเหตุ: Servo Motor X เสีย

การแก้ไข: เปลี่ยน Servo Motor X

=== END ===

=== MACHINE: BROTHER  (TC-S2D / TC-S2Dn) ===

อาการ: กด Start แล้วเครื่องไม่ทำงาน / Alarm Door / Tool Wash Level Error

สาเหตุ: Proximity, Cylinder Door หรือ Check Valve ผิดปกติ

การแก้ไข: ตรวจสอบ Proximity, Cylinder Door และเปลี่ยน Check Valve

=== END ===

=== MACHINE: BROTHER (TC-S2D / TC-S2Dn) ===

อาการ: เครื่องดับ / Main Breaker Trip / Chuck Clamp Error

สาเหตุ:

Motor Axis หรือ Motor Index เสื่อม

สายไฟ Solenoid ขาด

Motor Coolant ลงกราวด์

การแก้ไข:

เปลี่ยน Motor / Servo Amp

ย้ำสายไฟ Solenoid ใหม่

เปลี่ยน Motor Coolant และตรวจค่าฉนวน

=== END ===

=== MACHINE: BROTHER TAPPING CENTER (TC-S2A) ===

อาการ: *6165 ORG POS ERROR Z

สาเหตุ:  Servo Motor Z Axis ชำรุด

การแก้ไข:  เปลี่ยน Servo Motor Z Axis และ Set Origin ใหม่

=== END ===

=== MACHINE: BROTHER TAPPING CENTER (TC-S2A) ===

อาการ: *6165 ORG POS ERROR Z

สาเหตุ:  แกน Z จำตำแหน่งไม่ได้

การแก้ไข:  Set Origin ใหม่โดยปรับตำแหน่ง Coupling

=== END ===

=== MACHINE: BROTHER TAPPING CENTER (TC-S2Dn) ===

อาการ: Door Error

สาเหตุ:  Safety Door สกปรก

การแก้ไข:  ทำความสะอาด Safety Door

=== END ===

=== MACHINE: BROTHER TAPPING CENTER (SPEEDIO S500X1) ===

อาการ: A-Axis Servo Error (Position Deviation Large)

สาเหตุ:  ชุด Brake สกปรก และมีน้ำเข้าทางลม

การแก้ไข:  ถอด Index ล้างทำความสะอาดชุด Brake

=== END ===

=== MACHINE: BROTHER TAPPING CENTER (SPEEDIO S500X1) ===

อาการ: The Tool Washing Liquid Surface is Abnormal

สาเหตุ:  Pressure Coolant Washing ต่ำ

การแก้ไข:  ถอดทำความสะอาด Filter Coolant และทดสอบการผลิต

=== END ===

=== MACHINE: BROTHER TAPPING CENTER (TC-S2A) ===

อาการ: Can Not Clamp A Alarm

สาเหตุ:  Solenoid Valve Index ชำรุด

การแก้ไข:  เปลี่ยน Solenoid Valve Index ใหม่

=== END ===