=== MACHINE: Servo ===

อาการ: Alarm SV0001

สาเหตุ: sync alignment error.

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0002

สาเหตุ: sync excess error ALM 2

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0003

สาเหตุ: synchronous/composite/superimposed control mode can't be continued

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0004

สาเหตุ: g31 excess error

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0005

สาเหตุ: sync excess error (mcn)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0006

สาเหตุ: illegal tandem axis

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0007

สาเหตุ: SV ALM ANOTHER PATH(MULTI AMP.)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV00010

สาเหตุ: SV OVERHEAT

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO SOFTWARE) ===

อาการ: Alarm SV0011

สาเหตุ: SV MOTOR OVER CURRENT(SOFT)

การแก้ไข: The digital servo control software has detected an abnormal command value. An abnormality such as unconnected power cable, disconnection (open-phase), or short circuit is suspected.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 4 (SV0012)

สาเหตุ: SV DRIVE OFF CIRCUIT FAILURE

การแก้ไข:

(a) Check the parameter settings.

(b) Check if the STO-A and STO-B cables are disconnected.

(c) Check if the input time is off by more than 10 s between the STO-A and the STO-B

(d) Replace the servo amplifier.

(e) Replace the safety PLC or other component.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 3 (SV0013)

สาเหตุ: SV CPU BUS FAILURE

การแก้ไข: Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code J (SV0014)

สาเหตุ: SV CPU WATCH DOG

การแก้ไข: Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 2 (SV0015)

สาเหตุ: SV LOW VOLT DRIVER

การแก้ไข:

(a) Check the 24 V input voltage.

(b) Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER) ===

อาการ: Alarm Code b., c., d. (SV0016)

สาเหตุ: SV CURRENT DETECT ERROR

การแก้ไข:

(a) Check the voltage of the control power supply (24 VDC).

(b) Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 11 (SV0017)

สาเหตุ: SV INTERNAL SERIAL BUS FAILURE

การแก้ไข:

(a) Check that the control printed circuit board is properly inserted.

(b) Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER) ===

อาการ: Alarm Code 11 (SV0018)

สาเหตุ: SV ROM DATA FAILURE

การแก้ไข: Replace the servo amplifier

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 15 (SV0024, SP9204)

สาเหตุ: PS SOFT THERMAL

การแก้ไข: Overloaded operation → Review the operation of the motor.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 23 (SV0031, SP9211)

สาเหตุ: PS ILLEGAL PARAMETER

การแก้ไข:

(a) Parameters may not have been automatically updated.

→ Turn off the CNC and then turn it on.

(b) The setting parameters for the Power Supply are abnormal.

→ Set the parameters according to Chapter 15 in the FANUC SERVO AMPLIFIER ai-B series DESCRIPTIONS (B-65412EN).

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 24 (SV0034, SP9212)

สาเหตุ: PS HARDWARE ERROR

การแก้ไข:

(a) The setting parameters for the Power Supply are abnormal.

→ Set the parameters according to Chapter 15 in the FANUC SERVO AMPLIFIER ai-B series DESCRIPTIONS (B-65412EN).

(b) The selected Power Supply and submodule cannot be used together.

→ Connect the Power Supply and submodule with the correct ordering numbers according to Appendix I in the FANUC SERVO AMPLIFIER ai-B series DESCRIPTIONS (B-65412EN).

(c) Failure in the Power Supply

→ Replace the Power Supply.

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0035

สาเหตุ: SV NO FAILURE

การแก้ไข:

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER) ===

อาการ: Alarm Code A (SV0036)

สาเหตุ: PHASE OPEN

การแก้ไข:

(a) Check that the power leads are connected.

(b) Check whether a power lead is disconnected. If there is a disconnected power lead, replace it.

(c) Check whether a motor winding is disconnected. If there is a disconnected motor winding, replace the motor.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER) ===

อาการ: Alarm Code 9 (SV0037)

สาเหตุ: SV FAILURE (OPEN)

การแก้ไข: Replace the servo amplifier.

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0038

สาเหตุ: FAILURE OF CURRENT CTRL.

การแก้ไข:

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER) ===

อาการ: Alarm Code 8 (SV0039)

สาเหตุ: SV FAILURE (SHORT)

การแก้ไข: Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 25 (SV0040, SP9213)

สาเหตุ: PS EXT-COMP. ERROR

การแก้ไข:

(a) The setting parameters for the Power Supply are abnormal.

→ Set the parameters according to Chapter 15 in the FANUC SERVO AMPLIFIER ai-B series DESCRIPTIONS (B-65412EN).

(b) Abnormal condition of external input equipment (transformer or input filter)

→ Check external input equipment. (such as overheating or connection)

(c) Disconnection of communication path

→Check the connector and cable (CX61, CX62).

(d) Fault in the Power Supply

→ Replace the Power Supply.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 26 (SV0041, SP9214)

สาเหตุ: PS PFB-R ERROR

การแก้ไข: See the Appendix I in the FANUC SERVO AMPLIFIER ai-B series DESCRIPTIONS (B-65412EN).

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm code 27 (SV0042, SP9215)

สาเหตุ: PS PFB-C ERROR

การแก้ไข: See the Appendix in the FANUC SERVO AMPLIFIER ai-B series DESCRIPTIONS (B-65412EN).

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 28 (SV0043, SP9216)

สาเหตุ: PS SUB MODULE ERROR

การแก้ไข:

(a) Check if more than eight PFB-Rs are connected.

(b) Check if more than two PFB-Cs are connected.

(c) Check if more than 15 slaves for communication between the amplifiers are connected. (d) Check the connection order of PFB-Rs and PFB-Cs.

(e) Check whether PFB-Rs or PFB-Cs in the power supply voltage series that are different from the aiPS-B are connected.

(f) PFB-R or PFB-C communication error between the amplifiers Check the connector and cable (СХА2А, CXA2B). When multiple PFB-Rs are connected, check the connector and cable (CXA2A, CXS5).

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0301

สาเหตุ: APC ALARM: COMMUNICATION ERROR

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0302

สาเหตุ: APC ALARM: OVERTIME ERROR

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0303

สาเหตุ: APC ALARM: FRAMING ERROR

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0304

สาเหตุ: APC ALARM: PARITY ERROR

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0305

สาเหตุ: APC ALARM: PULSE ERROR

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0306

สาเหตุ: APC ALARM: OVERFLOW

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0307

สาเหตุ: APC ALARM: MOVEMENT EXCESS ERROR

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0360

สาเหตุ: PULSECODER CHECK SUM ERROR (INT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0361

สาเหตุ: PULSECODER PHASE ERROR (INT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0363

สาเหตุ: CLOCK ERROR (INT)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0364

สาเหตุ: SOFT PHASE ALARM (INT.)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0365

สาเหตุ: LED ERROR (INT.)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0366

สาเหตุ: PULSE ERROR (INT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0367

สาเหตุ: COUNT ERROR (INT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0368

สาเหตุ: SERIAL DATA ERROR (INT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0369

สาเหตุ: DATA TRANSFER ERROR (INT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0380

สาเหตุ: LED ERROR (EXT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0381

สาเหตุ: PULSECODER PHASE ERROR (EXT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0382

สาเหตุ: COUNT ERROR (EXT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0383

สาเหตุ: PULSE ERROR (EXT.)

การแก้ไข:

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0384

สาเหตุ: SOFT PHASE ALARM (EXT.)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0385

สาเหตุ: SERIAL DATA ERROR (EXT.)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0386

สาเหตุ: DATA TRANSFER ERROR (EXT.)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0387

สาเหตุ: ABNORMAL ENCODER (EXT.)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0401

สาเหตุ: V READY-OFF

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0403

สาเหตุ: CARD/SOFT MISMATCH

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0404

สาเหตุ: V READY-ON

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0407

สาเหตุ: EXCESS ERROR

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0409

สาเหตุ: DETECT ABNORMAL TORQUE

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0410

สาเหตุ: EXCESS ERROR (STOP)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0411

สาเหตุ: EXCESS ERROR (MOVING)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0413

สาเหตุ: LSI OVERFLOW

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0415

สาเหตุ: MOTION VALUE OVERFLOW

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO SOFTWARE) ===

อาการ: Alarm SV0417

สาเหตุ: INVALID SERVO PARAMETER

การแก้ไข: The setting value of the digital servo parameter is not correct. When diagnosis data (bit 4 of No. 0203) is 1, the servo software has detected an invalid parameter. → See Note 1.

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0420

สาเหตุ: SYNC TORQUE EXCESS

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0421

สาเหตุ: EXCESS ERROR (SEMI-FULL)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0422

สาเหตุ: EXCESS VELOCITY IN TORQUE

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0423

สาเหตุ: EXCESS ERROR IN TORQUE

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0430

สาเหตุ: SERVO MOTOR OVERHEAT

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 03 (SV0431, SP9058)

สาเหตุ: PS OVERLOAD

การแก้ไข:

(a) Failure in the heat sink cooling fan Check the rotation state of the heat sink cooling fan. → Replace the heat sink cooling fan.

(b) Degradation of the cooling capacity due to dust in the heat sink

→ Clean the heat sink and cooling system using factory air or a vacuum cleaner.

(c) Overloaded operation

→ Review the operating conditions.

(d) The control printed circuit board is not correctly attached. Push in the control printed circuit board in the front section of the Power Supply as far as it will go. If it does not go in easily, pull it out and then push it in again.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 06 (SV0432,SP9111)

สาเหตุ: PS LOW VOLT. CONTROL

การแก้ไข:

(a) Abnormal condition of the power supply → The 24 V power input to the connector (CXA2D) is abnormal. Check the output voltage and output capacity of the power supply.

(b) Failure in the Power Supply. → Replace the Power Supply.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 04 (SV0433, SP9051)

สาเหตุ: PS LOW VOLT. DC LINK

การแก้ไข:

(a) A small power dip has occurred. → Check the power supply.

(b) Drop in input power → Check the power supply specification.

(c) The main circuit was disconnected with emergency → Check the sequence, cable or magnetic contactor.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 2 (SV0434)

สาเหตุ: SV LOW VOLT CONTROL

การแก้ไข:

(a) Check the voltage of the control power supply (24 VDC).

(b) Check the communication cable between the amplifiers.

(c) Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 5 (SV0435)

สาเหตุ: SV LOW VOLT DC LINK

การแก้ไข:

(a) Check that the connection cable (bar) for DC link is tightened with the screw.

(b) If a DC link low voltage alarm condition occurs in more than one module, see Subsection 4.1.4, "Alarm code 4" for explanations about how to troubleshoot the Power Supply.

(c) If a DC link low voltage alarm condition occurs in only one servo amplifier, be sure to push the control printed circuit board of that servo amplifier in as far as it will go.

(d) Replace the servo amplifier corresponding to the alarm.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO SOFTWARE) ===

อาการ: Alarm SV0436

สาเหตุ: SOFT THERMAL (OVC)

การแก้ไข: The digital servo control software has detected a software thermal error (OVC). → See Note 2.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 01 (SV0437,SP9030)

สาเหตุ: PS OVERCURRENT

การแก้ไข:

(a) Unbalanced input power supply voltage → Check the specifications of the input power supply.

(b) Different AC reactor specifications ty of the power supply → Check the specifications of the Power Supply and AC reactor.

(c) Fault in the power module Replace the Power Supply → Replace the Power Supply.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code b, Code c, Code d (SV0438)

สาเหตุ: L,M,N AXIS SV CURRENT ERROR

การแก้ไข:

(a) Checking the servo parameters

Check whether the following parameters have the default values described in FANUC AC SERVO MOTOR alphaI-B/alphaI series PARAMETER MANUAL (B-65270EN).

Series 30i-B, 31i-B, 32i-B, 35i-B, 0i-D, 0i-F / Power Motion i-A: No.2004, No.2040, No.2041

Alternatively, if an abnormal motor current alarm condition occurs only on rapid acceleration/deceleration, it is likely that the motor is being used under too harsh a condition. Increase the acceleration/deceleration time constant, and see what will occur.

(b) Be sure to push the control printed circuit board as far as it will go.

(c) Remove the motor power leads from the servo amplifier and cancel the emergency stop.

<1> If no abnormal motor current occurs → Go to (d).

<2> If an abnormal motor current occurs → Replace the servo amplifier.

(d) Remove the motor power leads from the servo amplifier and check the isolation between the U, V, or W motor power lead and the PE.

<1> If the insulation is deteriorated →Go to (e).

<2> If the insulation is normal →Replace the servo amplifier.

(e) Disconnect the motor from its power leads, and check whether the insulation of the motor or power leads is deteriorated.

<1> If the insulation of the motor is deteriorated → Replace the motor.

<2> If the insulation of any power lead is deteriorated → Replace the power lead.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 07 (SV0439,SP9011)

สาเหตุ: PS OVER VOLT. DC LINK

การแก้ไข

(a) Excessive regenerative power, The capacity of the Power Supply is insufficient.

→ Check the specifications of the Power Supply and review the operation of the motor.

(b) The impedance of the power supply is high.

→ Check the impedance of the power supply. (Normal if the voltage variation at output time is within 7%)

(c) The main circuit was disconnected with emergency stop canceled

→ Check the sequence or magnetic contactor

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0441

สาเหตุ: CURRENT OFFSET ERROR

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 05 (SV0442, SP9033)

สาเหตุ: PS PRE-CHARGE FAILURE

การแก้ไข:

(a) No voltage is supplied to the power supply monitor (connector CX48) and main circuit of the Power Supply.

→ Check the power supply monitor (connector CX48) and its cabling, magnetic contactor, or breaker.

A phase of the power supply monitor (connector CX48) is disconnected.

→ Check the power supply monitor (connector CX48) and its cabling.

(b) A voltage outside of the specified range is input into the input power supply (three-phase AC).

An open-phase, unbalanced status, or low voltage is suspected. Check the power supply voltage.

A phase of the power supply monitor (connector CX48) is disconnected.

→ Check the power supply monitor (connector CX48) and its cabling.

(c) A voltage beyond the specification is input into the input power supply (three-phase AC).

→ Check the power supply voltage.

(d) The magnetic contactor is OFF.

→ See "4.1.4 If the Magnetic Contactor Is Not Turned ON" in Part I.

(e) Pre-charge was not completed within the specified time because the delay time from emergency stop cancellation to power-on of magnetic contactor was too long.

→ The ON sequence of the magnetic contactor is inappropriate. Check the PMC ladder.

(f) Pre-charge was not completed within the specified time because many spindle and servo

amplifiers were connected.

→ Review the number of connected amplifiers.

(g) The voltage value of the power supply monitor (connector CX48) in the Power Supply does not match the input of the main circuit (L1, L2, L3).

→ Check the power supply monitor (connector CX48) and its cabling.

(h) The phase sequence of the power supply monitor (connector CX48) in the Power Supply does not match the input of the main circuit (L1, L2, L3).

→ Check the power supply monitor (connector CX48) and its cabling.

(i) The DC link is short-circuited.

→ Remove the cause of the short circuit of the DC link.

using the SERVO GUIDE

The SERVO GUIDE allows you to identify the cause of this alarm by observing the internal data of the Power Supply immediately after emergency stop is canceled.

For details on how to observe the internal data of the Power Supply using the SERVO GUIDE, see Appendix C, "MEASURING INTERNAL DATA FOR THE POWER SUPPLY USING THE SERVO GUIDE."

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 02 (SV0443, SP9059)

สาเหตุ: PS INTERNAL FAN FAILURE

การแก้ไข:

(a) Failure in the internal cooling fan Check the rotation of the internal cooling fan.

If the internal cooling fan has stopped rotating, replace it.

(b) The internal cooling fan is not correctly attached.

Push the internal cooling fan into the top of the Power Supply as far as it will go.

If it does not go in easily, pull it out and then push it in again.

(c) The control printed circuit board is not correctly attached.

Push in the control printed circuit board in the front section of the Power Supply as far as it will go.

If it does not go in easily, pull it out and then push it in again.

(d) The Power Supply is defective.

Replace the Power Supply.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 1, 1 (SV0444)

สาเหตุ: SV INTERNAL FAN FAILURE (FAN DECELERATION)

การแก้ไข:

(a) Check whether there is any foreign material in the fan.

(b) Be sure to push the faceplate (control printed circuit board) as far as it will go.

(c) Check that the fan motor. connector is attached correctly.

(d) Replace Check that the fan motor.

(e) Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO SOFTWARE) ===

อาการ: Alarm SV0445

สาเหตุ: SOFT DISCONNECT ALARM

การแก้ไข: The digital servo control software has detected disconnection of the pulse coder. → See Note 3.

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0446

สาเหตุ: HARD DISCONNECT

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO SOFTWARE) ===

อาการ: Alarm SV0447

สาเหตุ: HARD DISCONNECT (EXT)

การแก้ไข: The hardware has detected disconnection of the separate detector. This alarm is issued when a separate phase A/B scale is used. Check if the phase A/B detector is connected correctly.

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0448

สาเหตุ: UNMATCHED FEEDBACK ALARM

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 8, Code 9, Code A (SV0449)

สาเหตุ: L,M,N AXIS SV IPM ALARM

การแก้ไข:

(a) Be sure to push the control printed circuit board as far as it will go.

(b) Remove the motor power leads from the servo amplifier and cancel the emergency stop.  <1> If no IPM alarm condition has occurred → Go to (c).

<2> If an IPM alarm condition has occurred → Replace the servo amplifier.

(c) Remove the motor power leads from the servo amplifier and check the isolation between the U, V, or W motor power lead and the PE.

<1> If the insulation is deteriorated → Go to (d).

<2> If the insulation is normal → Replace the servo amplifier.

(d) Disconnect the motor from its power leads, and check whether the insulation of the motor or power leads is deteriorated.

<1> If the insulation of the motor is deteriorated → Replace the motor.

<2> If the insulation of any power lead is deteriorated → Replace the power lead.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO SOFTWARE) ===

อาการ: Alarm SV0453

สาเหตุ: SPC SOFT DISCONNECT ALARM

การแก้ไข: αPulsecoder soft disconnection alarm. Turn off the power to the CNC, detach the pulsecoder cable, and then attach it again. If this alarm is still issued, replace the pulsecoder. → See Note 4.

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0454

สาเหตุ: ILLEGAL ROTOR POS DETECT

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm SV0456

สาเหตุ: ILLEGAL PHASE ROTOR POS DETECT

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0458

สาเหตุ: CURRENT LOOP ERROR

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0459

สาเหตุ: HI HRV SETTING ERROR

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0460

สาเหตุ: FSSB DISCONNECT

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0462

สาเหตุ: SEND CNC DATA FAILED

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0463

สาเหตุ: SEND SLAVE DATA FAILED

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0465

สาเหตุ: READ ID DATA FAILED

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0466

สาเหตุ: MOTOR/AMP. COMBINATION

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0468

สาเหตุ: HI HRV SETTING ERROR (AMP)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0474

สาเหตุ: EXCESS ERROR (STOP:SV)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0475

สาเหตุ: EXCESS ERROR (MOVE:SV)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0476

สาเหตุ: ILLEGAL SPEED CMD. (SV)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0477

สาเหตุ: ILLEGAL MACHINE POS. (SV)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0478

สาเหตุ: ILLEGAL AXIS DATA (SV)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0481

สาเหตุ: SAFETY PARAM ERROR (SV)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0484

สาเหตุ: SAFETY FUNCTION ERROR (SV)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0488

สาเหตุ: SELF TEST OVER TIME

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0489

สาเหตุ: SAFETY PARAM ERROR (CNC)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0490

สาเหตุ: SAFETY FUNCTION ERROR (CNC)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0494

สาเหตุ: ILLEGAL SPEED CMD. (CNC)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0496

สาเหตุ: ILLEGAL AXIS DATA (CNC)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0498

สาเหตุ: AXIS NUMBER NOT SET (CNC)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: SV0600

สาเหตุ: SV DC LINK OVER CURRENT

การแก้ไข:

(a) Remove the motor power leads from the servo amplifier and cancel the emergency stop.

<1> If no abnormal DC link current alarm condition has occurred → Go to (b).

<2> If an abnormal DC link current alarm condition has occurred in the amplifier. → Replace the servo amplifier.

(b) Remove the motor power leads from the servo amplifier and check the isolation between the U, V, or W motor power lead and the PE.

<1> If the insulation is deteriorated → Go to (c).

<2> If the insulation is normal → Replace the servo amplifier.

(c) Disconnect the motor from its power leads, and check whether the insulation of the motor or power leads is deteriorated.

<1> If the insulation of the motor is deteriorated → Replace the motor.

<2> If the insulation of any power lead is deteriorated → Replace the power lead.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code F, F (SV0601)

สาเหตุ: SV HEAT SINK FAN FAILURE (FAN DECELERATION)

การแก้ไข:

(a) Check whether there is any foreign material in the heat sink cooling fan.

(b) Be sure to push the control printed circuit board as far as it will go.

(c) Check that the heat sink cooling fan connector is attached correctly.

(d) Replace the heat sink cooling fan.

(e) Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 6 (SV0602)

สาเหตุ: SV HEAT SINK OVERHEAT

การแก้ไข:

(a) Check that the motor is being used at or below its continuous rating.

(b) Check that the cooling capacity of the power magnetics cabinet is sufficient (inspect the fans and filters).

(c) Check that the ambient temperature is not too high.

(d) Be sure to push the control printed circuit board in as far as it will go.

(e) Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 8, Code 9, Code A. (SV0603)

สาเหตุ: L,M,N AXIS SV IPM ALARM (OH)

การแก้ไข:

(a) Be sure to push the control printed circuit board as far as it will go.

(b) Check that the heat sink cooling fan is running.

(c) Check that the motor is being used at or below its continuous rating.

(d) Check that the cooling capacity of the power magnetics cabinet is sufficient (inspect the fans and filters).

(e) Check that the ambient temperature is not too high.

(f) Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code P (SV0604)

สาเหตุ: AMP COMMUNICATION ERROR

การแก้ไข:

(a) Check the communication cable between the amplifiers.

(b) Replace the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code U

สาเหตุ: SV FSSB communication error (COP10B)

วิธีแก้ไข:

(a) Replace the optical cable (COP10B) of the servo amplifier that is the closest to the CNC of those displaying U. (In Fig. 4.2.21, the cable between UNIT2 and UNIT3.)

(b) Replace the servo amplifier that is the closest to the CNC of those displaying U. (In Fig. 4.2.21, UNIT3.)

(c) Replace the servo amplifier on the COP10B side that is the closest to the CNC of those displaying U. (In Fig. 4.2.21, UNIT2.)

(d) Replace the CNC servo card.

NOTE: When the CNC power is turned on, "U" blinks momentarily, and then "-" steadily lights. This is not a failure, though.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER) ===

อาการ: Alarm Code L

สาเหตุ: SV FSSB communication error (COP10A)

การแก้ไข:

(a) Replace the optical cable (COP10A) of the servo amplifier that is the furthest from the CNC of those displaying L. (In Fig. 4.2.22, the cable between UNIT2 and UNIT3.)

(b) Replace the servo amplifier that is the furthest from the CNC of those displaying L. (In Fig. 4.2.22, UNIT2.)

(c) Replace the servo amplifier on the COP10A side that is the furthest from the CNC of those displaying L. (In Fig. 4.2.22, UNIT3.)

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Code 10 (SV0606, SP9113)

สาเหตุ: PS EXTERNAL FAN FAILURE

การแก้ไข:

(a) Failure in the heat sink cooling fan

Check the rotation state of the heat sink cooling fan.

→ Replace the heat sink cooling fan. The control printed circuit board is not correctly attached.

(b) The control printed circuit board is not correctly attached.

→ Push in the control printed circuit board in the section of the Power supply as fair as it will go. If it does not go in easily, pull out and then push it in again.

(c) The Power Supply is defective. → Replace the Power Supply.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (Power supply)===

อาการ: Alarm Code 14 (SV0607,SP9004)

สาเหตุ: PS IMPROPER INPUT POWER

การแก้ไข:

(a) The input power supply is abnormal → Check the power supply voltage and connection.

=== END ===

=== MACHINE: Servo ===

อาการ: SV0646

สาเหตุ: ABNORMAL ANALOG SIGNAL (EXT)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV0652

สาเหตุ: TEMP.ERROR

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Code 7 (SV0654)

สาเหตุ: DB RELAY FAILURE

การแก้ไข:

(a) Check if the power lead is short-circuited.

(b) Replace the servo amplifier or dynamic brake module.

(c) When using a dynamic brake module, replace the connection cable between it and the servo amplifier.

=== END ===

=== MACHINE: FANUC SERVO ai-B SERIES (SERVO AMPLIFIER)===

อาการ: Alarm Cold 7 (SV0659)

สาเหตุ: SSM FAILURE

การแก้ไข:

(a) Check the 24 V input voltage.

(b) Check the connection between the servo amplifier and SSM.

(c) Replace the SSM.

(d) Replace the servo amplifier.

=== END ===

=== MACHINE: Servo ===

อาการ: SV1025

สาเหตุ: V_READY ON (INITIALIZING)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV1026

สาเหตุ: ILLEGAL AXIS ARRANGE

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV1055

สาเหตุ: ILLEGAL TANDEM AXIS

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV1067

สาเหตุ: FSSB: CONFIG ERROR (SOFT)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV1068

สาเหตุ: DUAL CHECK SAFETY ALARM

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV1069

สาเหตุ: EXCESS ERROR (SERVO OFF:CNC)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV1070

สาเหตุ: EXCESS ERROR (SERVO OFF:SV)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV1071

สาเหตุ: EXCESS ERROR (MOVE:CNC)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV1072

สาเหตุ: EXCESS ERROR (STOP:CNC)

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV1100

สาเหตุ: S-COMP. VALUE OVERFLOW

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV5134

สาเหตุ: FSSB:OPEN READY TIME OUT

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV5136

สาเหตุ: FSSB:NUMBER OF AMP. IS INSUFFICIENT

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV5137

สาเหตุ: FSSB:CONFIG ERROR

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV5139

สาเหตุ: FSSB:ERROR

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV5197

สาเหตุ: FSSB:OPEN TIME OUT

การแก้ไข: -

=== END ===

=== MACHINE: Servo ===

อาการ: SV5311

สาเหตุ: FSSB:ILLEGAL CONNECTION

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 01 (SP9001)

สาเหตุ: MOTOR OVERHEAT

การแก้ไข:

(1) If this alarm is issued during cutting (the motor temperature is high)

(a) The cooling status of the motor is abnormal.

<1> If the fan motor for cooling the spindle motor has stopped, check that the power supply cable for the fan motor is connected correctly and that the cable is free of flaws such as scratches. If this does not resolve the situation, replace the fan motor.

<2> If the spindle motor is liquid-cooled, check that the cooling system is free of faults.

<3> If the ambient temperature of the spindle motor is high (higher than the specified range), make improvements so that the ambient temperature falls within the specified range.

(b) Check the processing conditions.

(2) If this alarm is issued under a light load (the motor temperature is high)

(a) The spindle motor frequently accelerates and decelerates. Make improvements so that the average output of the spindle motor including output during acceleration or deceleration is within the continuous rating output.

(b) Motor-specific parameters are incorrect. Check the motor-specific parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(3) If this alarm is issued immediately after power-on or even if the motor temperature is low

(a) Motor-specific parameters are incorrect. Check the motor-specific parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN). (Parameter No. 4134)

(b) The feedback cable of the spindle motor is abnormal. Replace the feedback cable.

(c) The temperature sensor inside the spindle motor is defective. Replace the spindle motor.

(d) Cable connection of the temperature sensor is incorrect. (Connect to pin Nos. 3 and 4 of connector JYA2.) For details, see the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN).

(e) The spindle amplifier may be defective. Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 02 (SP9002)

สาเหตุ: VELOCITY ERROR EXCESS

การแก้ไข: It has been detected that the detected motor speed value was very different from the specified speed.

(1) If this alarm is issued during motor acceleration

(a) The parameter setting of an acceleration/deceleration time is not acceptable. Set the acceleration/deceleration time of the actual machine with some margin to parameter No. 4082.

(b) Setting parameters for the speed detector are incorrect. Set correct values while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) If this alarm is issued at a heavy cutting load

(a) The cutting load is more than the maximum output of the motor. Check the load meter display and review the use conditions.

(b) An incorrect output limit parameter is set. Check that parameters No. 4028 and No. 4029 match the specifications of the machine and motor.

(c) Motor-specific parameters are incorrect. Check the motor-specific parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(3) If this alarm is issued even during unloaded rotation

(a) A motor winding is disconnected. Check the conductivity of motor windings.

(b) A power lead is disconnected. Check the conductivity of power leads.

(c) If the alarm is issued after checking (a) and (b) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 03 (SP9003)

สาเหตุ: DC-LINK FUSE IS BROKEN

การแก้ไข:

(1) If this alarm is displayed while the spindle is operating (rotating)

A fuse in the DC link in the spindle amplifier is likely to have blown. Replace the spindle amplifier. Perform the following steps before turning on the power again after replacing the spindle amplifier:

(a) Check that the power lead is not grounded.

(b) Check that motor windings are not grounded.

(2) When you turn on the magnetic contactor in the input section of the Power Supply and then turn it off with this alarm when the emergency stop is canceled (The magnetic contactor may not be turned off in the configuration where two or more axes of the spindle amplifiers are connected.)

(a) A cable is not connected in the DC link. Check and correct the cabling in the DC link.

(b) The control printed circuit board is not correctly attached. Push in the control printed circuit board in the front section of the spindle amplifier as far as it will go. If it does not go in easily, pull it out and push it in again.

(c) If the alarm is issued after checking (a) and (b) above, replace the spindle amplifier.

=== END ===

=== MACHINE: Servo ===

อาการ: SP9004

สาเหตุ: PS IMPROPER INPUT POWER

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 06 (SP9006)

สาเหตุ: TEMP. SENSOR DISCONNECTED

การแก้ไข:

(1) Motor-specific parameters are incorrect. Check the motor-specific parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) The feedback cable is defective. Replace the feedback cable.

(3) The temperature sensor inside the spindle motor is abnormal. Replace the internal temperature sensor or spindle motor.

(4) The ambient temperature of the spindle motor is outside of the specified range (low). Set the ambient temperature of the spindle motor within the specified range during use.

(5) The temperature sensor inside the spindle motor and winding are short-circuited. Cutting fluid or some other foreign matter may have entered into the spindle motor, short-circuiting the internal temperature sensor and winding. Check if cutting fluid or other foreign matter has gotten into the spindle motor.

(6) The spindle amplifier may be defective. Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 07 (SP9007)

สาเหตุ: OVER SPEED

การแก้ไข:

(1) If this alarm is issued during spindle synchronization If you deactivate a motor on either side (SFR or SRV) and reactivate it during spindle synchronization, the spindle motor may accelerate to eliminate the position error accumulated during this period and consequently reach the highest speed, causing the alarm. Correct the ladder to avoid this sequence.

(2) The spindle amplifier may be defective. Replace the spindle amplifier.

(3) The detector may be defective. Replace the detector.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 09 (SP9009)

สาเหตุ: OVERHEAT MAIN CIRCUIT

การแก้ไข:

(1) If this alarm is issued during cutting (the heat sink temperature is high)

(a) If this alarm is issued when the load meter indicates a value that is less than or equal to the continuous rating of the amplifier, check the cooling status of the heat sink.

<1> If the filter of the air inlet port in the power magnetics cabinet is dirty or there is an obstacle that interrupts the air flow to the heat sink, the heat sink is not sufficiently cooled. Make improvements to ensure sufficient air flow to the heat sink.

<2> If the ambient temperature of the spindle amplifier is outside of the specified range (high), make improvements so that the temperature falls within the specified range.

(b) If this alarm is issued when the load meter indicates a value that is greater than or equal to the continuous rating of the amplifier, review the cutting conditions.

(c) If the heat sink in the spindle amplifier is very dirty, clean it with air or some other tool. In addition, consider the structure of the power magnetics cabinet that prevents cutting fluid or other foreign matter from getting directly on the heat sink.

(2) If this alarm is issued under a light load (the heat sink temperature is high)

(a) The acceleration/deceleration frequency is high. Make improvements so that the average output of the spindle motor including output during acceleration or deceleration is within the continuous rating output.

(b) Motor-specific parameters are incorrect. Check the motor-specific parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(3) If this alarm is issued immediately after power-on

(a) The control printed circuit board is not correctly attached. Push in the control printed circuit board in the front section of the spindle amplifier as far as it will go. If it does not go in easily, pull it out and push it in again.

(b) If the alarm is issued after checking (a) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 10 (SP9010)

สาเหตุ: LOW INPUT POWER VOLT

การแก้ไข:

(1) Check the voltage of the control power supply (24 VDC).

(2) Check if there is a problem with the cable for the communication interface between the amplifiers.

(3) If the alarm is issued after checking (1) and (2) above, replace the spindle amplifier.

=== END ===

=== MACHINE: Servo ===

อาการ: SP9011

สาเหตุ: PS OVER VOLT. DC LINK

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: SP9012

สาเหตุ: OVER CUR. DC LINK

การแก้ไข:

(1) If this alarm is issued immediately after power-on

(a) The control printed circuit board is not correctly attached. Push in the control printed circuit board in the front section of the spindle amplifier as far as it will go.

(b) If the alarm is issued after checking (a) above, replace the spindle amplifier.

(2) If this alarm is issued immediately after the spindle rotation command is input

(a) A power lead of the spindle motor is defective. Check for a short circuit between power leads or grounding of a power lead. If there is a fault, replace the power lead.

(b) The spindle motor has poor isolation. Check for grounding of a motor winding. If there is a fault, replace the spindle motor. (The αiSP-B is equipped with the isolation deterioration detection function for the spindle motor. For details, see the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN).)

(c) Motor-specific parameters are incorrect. Check the motor-specific parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(d) If the alarm is issued after checking (a) to (c) above, replace the spindle amplifier.

(3) If this alarm is issued during spindle rotation

(a) Motor-specific parameters are incorrect. Check the motor-specific parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(b) The spindle sensor signal is abnormal. Check the signal waveform of the spindle sensor while referring to "4.3.6 Checking the Feedback Signal Waveform" in Part I. If there is an error, adjust the sensor assembly or replace the spindle sensor.

(c) A power lead or motor winding may be disconnected. Check the conductivity of power leads and motor windings. If there is an abnormality, replace the motor.

(d) The belt between the spindle and the motor may have slipped. (This alarm may be issued due to this cause in the spindle motor βiiI series. Clean the pulleys and adjust the belt tension.)

(e) If the alarm is issued after checking (a) to (d) above, replace the spindle amplifier.

(If the installation conditions of the spindle amplifier do not satisfy the specifications or if the heat sink is not sufficiently cooled due to contamination, a power element in the spindle amplifier may fail. In this case, replace the spindle amplifier and review the installation conditions.)

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 13 (SP12XX)

สาเหตุ: CPU INT DATA MEMORY ERROR

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 14 (SP9014)

สาเหตุ: INVALID SOFTWARE SERIES

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: SP9015

สาเหตุ: SPINDLE/OUTPUT SWITCHING ALARM

การแก้ไข:

(1) The magnetic contactor for power lead switching is defective. If the contact of the magnetic contactor in the power lead switching unit or the magnetic contactor that the customer prepared does not operate, check the power supply of the magnetic contactor. If this does not resolve the situation, replace the power lead switching unit or the magnetic contactor that the customer prepared.

(2) The I/O unit or wiring for checking the contact of the magnetic contactor is defective. Check the cabling. If this does not resolve the situation, replace the I/O unit.

(3) The sequence (ladder) is incorrect. Correct the processing to complete switching within one second.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 16 (SP9016)

สาเหตุ: RAM ERROR

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 17 (SP9017)

สาเหตุ: ID NO. PARITY ERROR

การแก้ไข:

(1) The control printed circuit board is not correctly attached. Push in the control printed circuit board in the front section of the spindle amplifier as far as it will go. If it does not go in easily, pull it out and push it in again. (This alarm may be displayed due to poor contact of the connector that connects the control printed circuit board and the power printed circuit board.)

(2) If the alarm is issued after checking (1) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 18 (SP12XX)

สาเหตุ: SUMCHECK ERROR PROGRAM ROM

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 19, 20 (SP9019 U, SP9020 V)

สาเหตุ: EXCESS OFFSET CURRENT DETECTION U,V

การแก้ไข:

(1) The control printed circuit board is not correctly attached. Push in the control printed circuit board in the front section of the spindle amplifier as far as it will go. If it does not go in easily, pull it out and push it in again. (This alarm may be displayed due to poor contact of the connector that connects the control printed circuit board and the power printed circuit board.)

(2) If the alarm is issued after checking (1) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 21 (SP9021)

สาเหตุ: INCORRECT POS SENSOR POLARITY SETTING

การแก้ไข:

(1) The setting of a parameter is incorrect. Check the parameter to set the polarity of the position sensor (bit 4 of No. 4001). For details, see the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) The feedback cable of the position sensor is not correctly wired. If the phase A/B signals are connected in reverse, correct the cabling.

(3) If the spindle is connected with the V belt, check if the belt slips.

(4) Check whether the signal from the position sensor is normal.

(5) If the alarm is issued after checking (1) to (4) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 22 (SP9022)

สาเหตุ: SP AMP OVER CURRENT

การแก้ไข:

(1) The operating conditions of the spindle are stringent. Possible causes include a high acceleration/deceleration frequency of the spindle and a high cutting load. Review the operating conditions of the spindle.

(2) Motor-specific parameters are incorrect. Check the motor-specific parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 24 (SP12XX)

สาเหตุ: SERIAL TRANSFER DATA ERROR

การแก้ไข:

(1) The optical communication cable between the CNC and the spindle amplifier is defective. Replace the cable.

(2) If the alarm is issued after replacing the cable, replace the spindle amplifier.

(3) If the alarm is issued after replacing the cable or spindle amplifier, replace the CNC board or module related to optical communication.

NOTE: This alarm is issued also if the CNC power is off. This is not a failure, though.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: SP9027

สาเหตุ: DISCONNECT POSITION CODER

การแก้ไข:

(1) If an alarm is issued while the motor is inactive

(a) The setting of a parameter is incorrect. Check the detector setting parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(b) The connection position on the spindle amplifier side of the feedback cable of the αi POSITIONCODER is incorrect. Check the connection position while referring to the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN).

(c) The feedback cable of the αi POSITIONCODER is disconnected. Replace the feedback cable.

(d) If the alarm is issued after checking (a) to (c) above, replace the spindle amplifier.

(2) If an alarm is issued when the feedback cable is moved (such as when the spindle is moved)

(a) The control printed circuit board is not correctly attached. Push in the control printed circuit board in the front section of the spindle amplifier as far as it will go. If it does not go in easily, pull it out and then push it in again.

(b) The feedback cable of the αi POSITIONCODER is disconnected. Replace the feedback cable.

(c) Cutting fluid has entered into the connector. If cutting fluid has entered into the connector, wash it off.

(d) If the alarm is issued after checking (a) to (c) above, replace the spindle amplifier.

(3) If this alarm is issued when the motor rotates

(a) The feedback cable is not sufficiently shielded. Read the chapter about connections in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and check the shield processing of the feedback cable.

(b) The feedback cable is bundled with the motor power leads. If the feedback cable is bundled with the motor power leads, read the chapter about installation in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and separate them.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 29 (SP9029)

สาเหตุ: SHORT-PERIOD OVERLOAD

การแก้ไข:

(1) If the alarm is issued during cutting

(a) Check the load meter and consider the cutting conditions.

(2) If this alarm is issued during a stop

(a) The spindle is locked. Check the sequence to see if the spindle is locked when a command for very slow movement is specified or orientation is specified for the spindle.

(3) If the spindle does not rotate as specified (the spindle rotates at a very low speed) and this alarm is issued

(a) The setting of a parameter is incorrect. Check the detector setting parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(b) The phase sequence of the motor power lead is incorrect. Check the phase sequence of the motor power leads again.

(c) The connection position of the feedback cable of the motor is incorrect. Make sure that the phase A/B signal of the spindle sensor is not connected in reverse.

(d) The feedback cable of the motor or spindle sensor (or motor) is defective. Manually turn the motor and check the motor speed on the CNC diagnosis screen or whether the spindle check board displays the speed. If the speed is not displayed, replace the feedback cable or spindle sensor (or motor).

(4) If the spindle does not rotate as specified (the spindle does not rotate at all) and this alarm is issued

(a) A power lead is abnormal. Check that the power lead is correctly connected from the spindle amplifier to the motor. When switching the output or spindle, check whether the magnetic contactor on the side to be used is on.

(b) If the alarm is issued after checking (1) to (4-a) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 31 (SP9031)

สาเหตุ: MOTOR LOCK

การแก้ไข:

(1) If the motor rotates at a very low speed and this alarm is issued

(a) The setting of a parameter is incorrect. Check the detector setting parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(b) The phase sequence of the motor power lead is incorrect. Check whether the phase sequence of the motor power lead is correct.

(c) The feedback cable of the motor has a problem. Make sure that the phase A/B signal of the spindle sensor is not connected in reverse.

(d) The feedback cable of the motor or spindle sensor (or motor) is defective. Manually turn the motor and check the motor speed on the CNC diagnosis screen or whether the spindle check board displays the speed. If the speed is not displayed, replace the feedback cable or spindle sensor (or motor).

(2) If the motor does not rotate at all and this alarm is issued

(a) The sequence for locking the spindle is incorrect. Check the sequence to see if the spindle is locked.

(b) The power lead is defective. Check that the power lead is correctly connected from the spindle amplifier to the motor. When switching the output or spindle, check whether the magnetic contactor on the side to be used is on.

(c) If the alarm is issued after checking (1) to (2-b) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 32 (SP12XX)

สาเหตุ: SERIAL LSI RAM ERROR

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 34 (SP9034)

สาเหตุ: ILLEGAL PARAMETER

การแก้ไข:

(1) Connect the spindle check board (A06B-6078-H001) to the connector JY1. The spindle check board (A06B-6078-H001) alternately displays AL-34 and F-xxx. F-xxx indicates the parameter number outside the specifiable range. For the correspondence between the CNC parameter numbers and F-xxx, see "Internal number F-xxx" in Appendix "SPINDLE PARAMETER LIST" in the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN). Correct the setting value of the relevant parameter.

(2) A parameter outside the specifiable range can be identified with the status error number. The status error number in the diagnosis number No. 710 indicates the internal number of the relevant parameter. For the correspondence between the internal numbers and actual parameters, see "Internal number F-xxx" in Appendix "SPINDLE PARAMETER LIST" in the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN). Correct the setting value of the relevant parameter.

If the alarm is issued after checking (1) and (2) above, replace the spindle amplifier.

=== END ===

=== MACHINE: Servo ===

อาการ: Alarm Code 35 (SP9035)

สาเหตุ: EXCESS GEAR RATIO SETTING DATA

การแก้ไข:

(1) If an alarm is issued when a rotation command is entered

(a) Error in the position coder setting parameter. Correctly specify the bits representing the relationships between the direction of position coder rotation and that of spindle rotation and between the direction of spindle rotation and that of motor rotation.

Parameter No. 4000#0: Spindle and spindle motor rotation directions

Parameter No. 4001#4: Spindle sensor (position coder) mounting direction

(b) Invalid gear ratio parameter setting. Check to see if an incorrect gear ratio has been specified. This value is used to convert the position coder speed to the motor speed. Be sure to specify the correct value.

Parameter No. 4056~4059: Gear ratio between the spindle and motor

(c) Clutch/gear signal error. Make sure that the entered clutch/gear signals (CTH1A, CTH2A) are correct with respect to the actually selected gear.

First spindle (G070): CTH1A (Bit #3), CTH2A (Bit #2)

Second spindle (G074): CTH1B (Bit #3), CTH2B (Bit #2)

(d) Belt slippage between the spindle and spindle motor. Make adjustments so that the belt between the spindle and the spindle motor does not slip.

(2) If the alarm is issued during cutting The load is excessive and the motor speed decelerates. Review the cutting conditions.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 36 (SP9036)

สาเหตุ: OVERFLOW ERROR COUNTER

การแก้ไข:

(1) The setting of a parameter is incorrect.

(a) The gear ratio set in a parameter is incorrect. Check if an excessively large gear ratio is set.

(b) The setting of a position gain is incorrect. If the gear ratio data is correct, increase the position gain.

Parameter No. 4056~4059: Gear ratio between the spindle and motor

Parameter No. 4060~4063: Position gain at orientation

Parameter No. 4065~4068: Position gain in the servo mode/spindle synchronization

Parameter No. 4069~4072: Position gain in Cs contour control

(2) The sequence is incorrect.

(a) Check if the motor is deactivated (by turning off SFR/SRV) in a position control mode (rigid tapping, Cs contour control, or spindle synchronization control).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 37 (SP9037)

สาเหตุ: SPEED DETECTION PARAMETER ERROR

การแก้ไข:

(1) The parameter setting of the speed detector is incorrect. Set correct values while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) The parameter setting of an acceleration/deceleration time is not acceptable. Check the parameter-set value and actual acceleration/deceleration time, then set an actual acceleration/deceleration time plus some margin.

Parameter No. 4082: Setting of acceleration/deceleration time

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 41 (SP9041)

สาเหตุ: 1-ROT PC ERROR

การแก้ไข:

(1) The setting of a parameter is incorrect. Check the detector setting parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) The αi POSITIONCODER is defective. If you observe the phase Z signal for the αi POSITIONCODER but the signal does not occur at each rotation, replace the αi POSITIONCODER.

(3) The frame ground is not sufficiently grounded. Ground the equipment flange, cable shield or other component with a low impedance. Keep a distance of 10 cm or more between the ground of the signal system and the ground of the power system. Connect the grounding wire to an unpainted surface.

(4) The feedback cable is not sufficiently shielded. Read the chapter about connections in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and check the shield processing of the feedback cable.

(5) The feedback cable is routed parallel to the motor power leads over a long distance or is bundled with them. Noise may be induced in the feedback cable by the power lead. Read the chapter about installation in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and separately wire the power leads and the feedback cable.

(6) The specifications of the feedback cable are incorrect. Use the twisted pair line for the sensor signal. Use the shielded cable for the feedback cable.

(7) The spindle amplifier is defective. If the alarm is issued after checking (1) to (6) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 42 (SP9042)

สาเหตุ: NO 1-ROT PC

การแก้ไข:

(1) The setting of a parameter is incorrect. Check the detector setting parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) The feedback cable or αi POSITIONCODER is defective. If you observe the phase Z signal for the αi POSITIONCODER but the signal does not occur at each rotation, replace the feedback cable or αi POSITIONCODER.

(3) If the alarm is issued after checking (1) and (2) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 43 (SP9043)

สาเหตุ: DISCONNECT POSITION CODER DEF. SPEED

การแก้ไข: The position coder signal of the master spindle used in differential spindle speed control is disconnected. For troubleshooting when this alarm is issued, see the subsection about alarm code 27.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: SP9046

สาเหตุ: 1-ROT PC ERROR DURING THREAD CUTTING

การแก้ไข: The one-rotation signal of the position coder cannot be detected normally during thread cutting. For troubleshooting when this alarm is issued, see the subsection about alarm code 41.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 47 (SP9047)

สาเหตุ: POSITION CODER SIGNAL ERROR

การแก้ไข:

(1) If the alarm is issued when the cable is moved (such as when the spindle is moved)

(a) The feedback cable of the αi POSITIONCODER is disconnected. Replace the feedback cable.

(b) Cutting fluid has gotten into the connector. If cutting fluid has entered into the connector, wash it off.

(2) In other cases

(a) The setting of a parameter is incorrect. Check the detector setting parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(b) The frame ground is not sufficiently grounded. Ground the equipment flange, cable shield or other component with a low impedance. Keep a distance of 10 cm or more between the ground of the signal system and the ground of the power system. Connect the grounding wire to an unpainted surface.

(c) The feedback cable is not sufficiently shielded. Read the chapter about connections in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and check the shield processing of the feedback cable.

(d) The feedback cable is routed parallel to the motor power leads over a long distance or is bundled with them. Noise may be induced in the feedback cable by the power lead. Read the chapter about installation in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and separately wire the power leads and the feedback cable.

(e) The specifications of the feedback cable are not sufficient. Use the twisted pair line for the sensor signal. Use the shielded cable for the feedback cable.

(f) The spindle amplifier is defective. If the alarm is issued after checking (a) to (e) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 49 (SP9049)

สาเหตุ: EXCESS CUMULATIVE DIFFERENTIAL SPEED

การแก้ไข: In differential spindle speed control, the speed of the master spindle converted to the motor speed of the slave spindle exceeded the allowable range. The master spindle speed is converted to the motor speed of the slave spindle by multiplying the master spindle speed by the gear ratio between the slave spindle and motor. Check whether the converted spindle speed exceeds the maximum motor speed and operate the machine so that the maximum motor speed is not exceeded.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 50 (SP9050)

สาเหตุ: SPINDLE CONTROL EXCESS VELOCITY

การแก้ไข:

1. The gear ratio set in a parameter is incorrect. Check if an excessively large gear ratio is set.

2. The parameter setting of a position gain is incorrect. If the gear ratio data is correct, decrease the position gain during spindle synchronization.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 52, 53 (SP9052, SP9053)

สาเหตุ: ITP FAULT I

การแก้ไข:

1. The spindle amplifier is defective. Replace the spindle amplifier.

2. The CNC is defective. Replace the board or module related to the serial spindle.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 54 (SP9054)

สาเหตุ: AMP OVER CURRENT

การแก้ไข: For troubleshooting when this alarm is issued, see the subsection about alarm code 29.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 55 (SP9055)

สาเหตุ: POWER LEAD ERROR AT SWITCHING

การแก้ไข:

1. The magnetic contactor for power lead switching is defective. If the contact of the magnetic contactor in the power lead switching unit or the magnetic contactor that the customer prepared does not operate, check the power supply of the magnetic contactor. If this does not resolve the situation, replace the power lead switching unit or the magnetic contactor that the customer prepared.

2. The I/O unit or wiring for checking the contact of the magnetic contactor is defective. Check the cabling. If this does not resolve the situation, replace the I/O unit.

3. The sequence (ladder) is incorrect. Correct the processing to complete switching within 1 second.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 56 (SP9056)

สาเหตุ: INTERNAL FAN FAILURE

การแก้ไข:

1. The internal cooling fan is not correctly attached. Push in the internal cooling fan in the top section of the spindle amplifier as far as it will go. If it does not go in easily, pull it out and then push it in again.

2. The control printed circuit board is not correctly attached. Push in the control printed circuit board in the front section of the spindle amplifier as far as it will go. If it does not go in easily, pull it out and then push it in again.

3.  The internal cooling fan is abnormal. Read "REPLACEMENT PARTS AND REPLACEMENT PROCEDURE" in this manual and replace the internal cooling fan.

4.  If the alarm is issued after checking (1) to (3) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ:  Alarm Code 61 (SP9061)

สาเหตุ: SEMI-FULL ERROR EXCESS

การแก้ไข:

1. When this alarm is issued at the start of the machine, there may be a setting error in a detector-related parameter or motor-end position feedback conversion coefficient parameter. Check whether the settings for the arbitrary gear ratio between the motor sensor and the spindle, No. 4171 and No. 4172 (when in high gear) and No. 4173 and No. 4174 (when in low gear) are correct. Check the detector setting parameters while referring to B-65280EN.

2.  If the arbitrary gear ratio between the motor sensor and the spindle is correct, the setting level of No. 4354 is too low.

3.  When this alarm is issued for a system which has been operating normally, there may be slippage between the motor and spindle. Check the machine.

4. When this alarm is issued intermittently for a system which has been operating normally, the position feedback signals may be miscounted at the motor end or spindle end due to noise or other cause. Take appropriate countermeasures against noise.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 65 (SP9065)

สาเหตุ: SERIAL SPINDLE ALARM

การแก้ไข:

1. There is a setting error in a parameter (number of teeth of a motor sensor or number of phases of a motor). Check settings (bits 2, 1, 0 of No. 4011 and No. 4334 / bits 7, 3 of No. 4011 and No. 4368).

2.  The phase sequence of the motor power lead differs from that of connection of motor sensor feedback.

3.  The motor power lead is not connected.

4.  The motor is locked and inoperable. Or, the motor cannot operate due to friction because the excitation current is too small. Detect the magnetic pole when the motor can move smoothly.

5.  The motor sensor or rotor shaft slipped and the phase relationship between the motor sensor and the rotor changed.

6.  A count error occurs in the motor feedback signal due to the effect of noise. Take appropriate countermeasures against noise.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 66 (SP9066)

สาเหตุ: SPINDLE AMP COMMUNICATION ERROR

การแก้ไข:

1.  The communication cable between the spindle amplifiers (K88) is disconnected. Push in the connector as far as it will go.

2.  The communication cable between the spindle amplifiers (K88) is too long or installed in a different cabinet. Read B-65412EN.

3.  The communication cable between the spindle amplifiers (K88) is defective. Replace it.

4.  If the alarm is issued after checking (1) to (3) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 67 (SP9067)

สาเหตุ: FSC/EGB COMMAND ERROR

การแก้ไข: Reference position return is specified for the slave axis on which the spindle electric gear box (EGB) is being executed. Reference position return cannot be performed in the spindle EGB mode (G81). Before performing reference position return, turn the EGB mode off (G80).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 68 (SP9068)

สาเหตุ: ILLEGAL SPINDLE PARAMETER

การแก้ไข: The setting of a parameter is abnormal. Details of the abnormal condition can be identified with the status error number. Correct the relevant parameter setting while referring to the status error number in the diagnosis number No. 710 and "4.3.3 Status Error Display" in Part I.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 69 (SP9069)

สาเหตุ: SAFETY SPEED EXCEEDED

การแก้ไข:

1.  If the protective door is open, operate the spindle motor at the safety speed or slower.

2.  Check the safety speed parameter.

3.  If the alarm is issued after checking (1) and (2) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 70 (SP9070)

สาเหตุ: ABNORMAL AXIS DATA

การแก้ไข:

1. Check the connection and settings of the spindle amplifier.

2.  Replace the CPU card in the CNC.

3.  The spindle amplifier ID is not written. If the amplifier ID cannot be read on the CNC screen, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 71 (SP9071)

สาเหตุ: SAFETY PARAM ERROR

การแก้ไข:

1. Set the safety parameter again.

2.  Replace the CPU card in the CNC.

3.  If the alarm is issued after checking (1) and (2) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 72 (SP9072)

สาเหตุ: MOTOR SPEED MISMATCH

การแก้ไข: replace the CPU card or spindle amplifier in the CNC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 73 (SP9073)

สาเหตุ: MOTOR SENSOR DISCONNECTED

การแก้ไข:

1. If this alarm is issued when the motor is deactivated:

(a) Parameter setting incorrect. Check B-65280EN.

(b) Feedback cable disconnected. Replace cable.

(c) Motor sensor (αiBZ or αiMZ) not correctly adjusted. Adjust or replace sensor/cable.

(d) Replace spindle amplifier if (a)-(c) checked.

2. If issued when cable is moved:

(a) Cutting fluid in connector. Wash off.

(b) Feedback cable disconnected. Replace cable.

3. If issued when motor rotates:

(a) Frame ground insufficiently grounded.

(b) Feedback cable insufficiently shielded.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 74 (SP9074)

สาเหตุ: CPU TEST ERROR

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 75 (SP9075)

สาเหตุ: CRC ERROR

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 76 (SP9076)

สาเหตุ: SAFETY FUNCTION NOT EXECUTED

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 77 (SP9077)

สาเหตุ: AXIS NO. MISMATCH

การแก้ไข: Replace the CPU card or spindle amplifier in the CNC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 78 (SP9078)

สาเหตุ: SAFETY PARAM MISMATCH

การแก้ไข: Replace the CPU card or spindle amplifier in the CNC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 79 (SP9079)

สาเหตุ: INITIAL TEST OPERATION ERROR

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 80 (SP9080)

สาเหตุ: DESTINATION SPINDLE AMP ERROR

การแก้ไข: Check the meaning of the alarm in the counterpart spindle amplifier and take action.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 81 (SP9081)

สาเหตุ: 1-ROT MOTOR SENSOR ERROR

การแก้ไข:

1.  When using external one-rotation signal (proximity switch):

(a) Incorrect parameters (Check No. 4171, 4172, 4173, 4174).

(b) Slippage between spindle and motor.

2. Troubleshooting in other cases:

(a) Incorrect parameter setting.

(b) Motor sensor (αiBZ or αiMZ) not correctly adjusted.

(c) Frame ground insufficiently grounded.

(d) Feedback cable insufficiently shielded.

(e) Feedback cable routed parallel or bundled with power leads.

(f) Cable specs incorrect (must use twisted pair for sensor and thermistor, route thermistor signal line between unused lines—do not place adjacent to Phase A signal line).

(g) Spindle amplifier defective (replace if (a)-(f) checked).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 82 (SP9082)

สาเหตุ: NO 1-ROT MOTOR SENSOR

การแก้ไข:

1. When using external one-rotation signal (proximity switch):

(a) Signal defective. Check pin EXTSC1 on check board. If no signal, replace cable or position coder.

2. Troubleshooting in other cases:

(a) Parameter setting incorrect.

(b) Motor sensor (αiBZ or αiMZ) not correctly adjusted.

(c) Frame ground insufficiently grounded.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 83 (SP9083)

สาเหตุ: MOTOR SENSOR SIGNAL ERROR

การแก้ไข:

(1) If the alarm is issued when the cable is moved (such as when the spindle is moved)

(a) Cutting fluid has entered into the connector.

If cutting fluid has entered into the connector, wash it off.

(b) The feedback cable is disconnected.

Replace the feedback cable.

(2) Troubleshooting in other cases

(a) The setting of a parameter is incorrect.

Check the detector setting parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(b) The motor sensor (αiBZ SENSOR or αiMZ SENSOR) is not correctly adjusted.

Adjust the motor sensor signal. If adjustment is impossible or the signal is not observed, replace the connection cable and motor sensor.

(c) The frame ground is not sufficiently grounded.

Ground the equipment flange, cable shield or other component with a low impedance. Keep a distance of 10 cm or more between the ground of the signal system and the ground of the power system. Connect the grounding wire to an unpainted surface.

(d) The feedback cable is not sufficiently shielded.

Read the chapter about connections in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and securely connect the shield for sensor or thermistor. Pay extra attention so that there are no breaks in the shield in the relay section. Ensure that the unshielded ends of the cable are as short as possible.

(e) The feedback cable is routed parallel to the motor power leads over a long distance or is bundled with them.

Noise may be induced in the feedback cable by the power lead. Read the chapter about installation in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and separately wire the power leads and the feedback cable.

(f) The specifications of the feedback cable are incorrect.

Use the twisted pair line for the sensor signal and thermistor signal. Use the shielded cable for the feedback cable. If the shield of the sensor or thermistor is not grounded and the sensor and thermistor cables are connected adjacent to each other in a bundled cable like that for the αiBZ SENSOR, noise superimposed over the thermistor signal may affect the sensor signal in the cable. Route the sensor signal and thermistor signal cables so that they are not adjacent to each other in the bundled cable to the extent possible as shown in the figure below.

(g) The spindle amplifier is defective. If the alarm is issued after checking (a) to (f) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 84 (SP9084)

สาเหตุ: SPNDL SENSOR DISCONNECTED

การแก้ไข: The spindle sensor signal is abnormal. For troubleshooting when this alarm is issued, see the subsection about alarm code 73.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 85 (SP9085)

สาเหตุ: 1-ROT SPNDL SENSOR ERROR

การแก้ไข: The one-rotation signal of the spindle sensor occurred in an incorrect location. For troubleshooting when this alarm is issued, see the subsection about alarm code 81.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 86 (SP9086)

สาเหตุ: NO 1-ROT SPNDL SENSOR

การแก้ไข: The one-rotation signal of the motor sensor is not generated. For troubleshooting when this alarm is issued, see the subsection about alarm code 82.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 87 (SP9087)

สาเหตุ: SPNDL SENSOR SIGNAL ERROR

การแก้ไข: The spindle sensor signal is abnormal. For troubleshooting when this alarm is issued, see the subsection about alarm code 83.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 88 (SP9088)

สาเหตุ: EXTERNAL FAN FAILURE

การแก้ไข:

(1) The heat sink cooling fan is abnormal. Read "REPLACEMENT PARTS AND REPLACEMENT PROCEDURE" in this manual and replace the heat sink cooling fan.

(2) If the alarm is issued after checking the above item, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 89 (SP9089)

สาเหตุ: SUBMODULE SM (SSM) ERROR

การแก้ไข: An error related to the submodule SM (SSM) used when driving the synchronization spindle motor has been detected. When this alarm is issued at the start, the possible causes are (1) to (4) below. When the machine has been operating normally, the possible causes are (3) and (4).

(1) The SSM is not connected.

Action: Check the connection of the interface signal cable between JYA4 (spindle amplifier) and CX31 (SSM).

(2) The cable is disconnected.

Action: Replace the interface signal cable between JYA4 (spindle amplifier) and CX31 (SSM).

(3) The SSM is defective.

Action: Replace the SSM.

(4) If the alarm is issued after checking (1) to (3) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 90 (SP9090)

สาเหตุ: UNEXPECTED ROTATION ALARM

การแก้ไข: An unexpected rotation state (state in which the polarity in the torque command differs that of acceleration) was detected. This spindle alarm is issued for the BiS series spindle motors (synchronous spindle motors). When this alarm is issued at the start, the possible causes are (1) to (4) below. When the machine has been operating normally, the possible causes are (3) and (4).

(1) There is a setting error in a parameter (number of teeth of a motor sensor or number of phases of a motor). Check the parameter while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) The setting of the AMR offset-related parameter (No. 4084 or 4085) is incorrect. Parameter No. 4084 (AMR offset) is adjusted for each machine. If the phase relationship between the rotor magnetic pole 0° position and the one-rotation signal position of the detector changes due to detector replacement or because the shaft slipped or if you load parameters from another machine, readjustment is required. Parameter No. 4085 is provided for AMR offset adjustment. Normally, this parameter must be set to "0".

(3) If this alarm is issued for example when the spindle hits against something, the motor sensor or rotor shaft may have slipped, shifting the phase relationship between the motor sensor and the rotor.

(4) A count error occurs in the motor sensor due to noise. Read the chapter on installation in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and take appropriate countermeasures against noise.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 91 (SP9091)

สาเหตุ: POLE POSITION COUNT MISS

การแก้ไข: A count error occurred for the magnetic pole position (a large difference between the magnetic pole position at the one-rotation signal position and the setting of parameter No. 4084 (AMR offset) was detected). This spindle alarm is issued for the BiS series spindle motors (synchronous spindle motors). When this alarm is issued at the start, possible causes are both (1) and (2) below. When the machine has been operating normally, check (2) and take appropriate action.

(1) There is a setting error in a parameter (number of teeth of a motor sensor or number of phases of a motor). Check the parameter while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) An unnecessary one-rotation signal is generated or a count error occurs for feedback pulses due to slippage of the sensor ring or noise. Check if the motor sensor has slipped. If there is no problem, read the chapter on installation in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and take appropriate countermeasures against noise.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code 92 (SP9092)

สาเหตุ: OVERSPEED (SPEED COMMAND STANDARDS)

การแก้ไข: The actual motor speed exceeded the overspeed level corresponding to the velocity command. When this alarm is issued at the start, possible causes are all of (1) to (3) below. When the machine has been operating normally, it is highly possible that the cause is (3).

(1) There is a setting error in a parameter (number of teeth of a motor sensor or number of phases of a motor). Check parameters while referring to the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) When a BiS series spindle motor (synchronous spindle motor) is driven, there is a setting error in an AMR offset-related parameter (No. 4084 or 4085). Parameter No. 4084 (AMR offset) is adjusted for each machine. If the phase relationship between the rotor magnetic pole 0° position and the one-rotation signal position of the detector changes due to detector replacement or because the shaft slipped or if you load parameters from another machine, readjustment is required. Parameter No. 4085 is provided for AMR offset adjustment. Normally, this parameter must be set to "0".

(3) A count error occurs in the motor sensor due to noise. Read the chapter on installation in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and take appropriate countermeasures against noise.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Codes A, A1, and A2

สาเหตุ: PROGRAM ROM ERROR

การแก้ไข: The control program is not running. An error was detected when the control program was running.

(1) If this alarm is issued when the spindle amplifier power is switched on: Replace the spindle amplifier.

(2) If this alarm is issued when the motor is active:

(a) Effects of noise: Read the chapter on installation in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN) and check the ground-related cabling.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code b0 (SP9110)

สาเหตุ: AMP COMMUNCATION ERROR

การแก้ไข: An error occurred in communication between amplifiers (spindle amplifier, servo amplifier, and Power Supply).

(1) Check the connector to which the communication cable between the amplifiers is connected. Normally, CXA2A and CXA2B must be coupled.

(2) The communication cable between the amplifiers is defective. Replace the communication cable between the amplifiers.

(3) Effects of noise. Check that the communication cable between the amplifiers is not wired in parallel with the DC link short bar and power leads.

(4) If the alarm is issued after checking (1) to (3) above, replace the spindle amplifier, servo amplifier or Power Supply.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code b4 (SP9114)

สาเหตุ: PS CONTROL AXIS ERROR 1

การแก้ไข: A non-zero value is set to No. 4657 even though the spindle axis is not the PS control axis. Set 0 to No. 4657 when the spindle axis is not the PS control axis.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code b5 (SP9115)

สาเหตุ: PS CONTROL AXIS ERROR 2

การแก้ไข: 0 is set to No. 4657 even though the spindle axis is the PS control axis. Set the amplifier group number (non-zero value) to No. 4657 for the PS control axis when the spindle axis is the PS control axis.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Codes C0, Codes C1, Codes C2 (SP9120, SP9121, SP9122)

สาเหตุ: COMMUNICATION DATA ALARM

การแก้ไข: Serial communication data transferred between the CNC and Spindle Amplifier contains an error.

(1) The spindle amplifier is defective. Replace the spindle amplifier.

(2) The CNC is defective.

(3) Replace the board or module related to the serial spindle.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code C3 (SP9123)

สาเหตุ: SPNDL SWITCHING CIRCUIT ERROR

การแก้ไข: In spindle switching, a mismatch is found between the switching request signal (SPSL) and the internal status of the motor/spindle sensor signal switching circuit (submodule SW: SSW).

(1) The SSW is defective. Replace the SSW.

(2) The cable is defective. Replace the cable that connects the spindle amplifier and SSW.

(3) If the alarm is issued after checking (1) and (2) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code C4 (SP9124)

สาเหตุ: LEARNING CTRL ROTATION CMD ERROR

การแก้ไข: An invalid repetition cycle is set under repetition control with the time synchronization method.

(1) Check whether the specified speed value is appropriate.

(2) Check that a value between 20 and 4800 is set as the repetition cycle (No. 4425).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code C5 (SP9125)

สาเหตุ: LEARNING CTRL INVALID DYNAMIC CHARACTERISTIC COMPENSATION DEGREE

การแก้ไข: An invalid value is set for the highest or lowest degree of a dynamic characteristic compensation element in repetition control. Set a smaller value to the highest or lowest degree of a dynamic characteristic compensation element Gx (No. 4427, 4428).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code C6 (SP9126)

สาเหตุ: SPNDL SPEED EXCEEDED

การแก้ไข: The spindle speed exceeds the spindle monitoring speed five times in a row.

(1) The sequence is incorrect. Check the sequence and command speed.

(2) The setting of a parameter is incorrect. Check No. 4512.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code C7 (SP9127)

สาเหตุ: INVALID LEARNING CYCLE

การแก้ไข: The reference angle cycle is inappropriate under repetition control with the angle synchronization method. Increase the setting value of the parameter for the number of divisions per cycle under repetition control (No. 4425).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code C8 (SP9128)

สาเหตุ: SP SYNC CTRL VELOCITY ERROR EXCESS

การแก้ไข: In spindle synchronization, the speed deviation (difference between the specified speed at the spindle end calculated based on the position error and position gain and the actual speed) exceeded the alarm detection level (parameter No. 4515).

(1) When this alarm is issued immediately after the motor excitation (SFR, SRV) is turned off, then on again in spindle synchronization: This alarm may be issued because the spindle motor is accelerated to eliminate the accumulated position error while the motor excitation is off. After canceling spindle synchronization control, correct the sequence to deactivate the motor.

(2) When this alarm is issued during cutting:

(a) A possible cause is overload. Review the cutting conditions.

(b) When no overload occurs, review whether the alarm detection level setting is appropriate.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code C9 (SP9129)

สาเหตุ: SP SYNC CTRL POSITION ERROR EXCESS

การแก้ไข: In spindle synchronization, the position error exceeded the alarm detection level (parameter No. 4516). For troubleshooting when this alarm is issued, see the subsection about alarm code C8.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d0 (SP9130)

สาเหตุ: POLARITY ERROR BETWEEN TORQUE TANDEM MASTER AND SLAVE

การแก้ไข: In torque tandem control, the speed polarity relationship between the master motor and slave motor is abnormal. Check the setting related to the rotation direction (bit 2 of No. 4353).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d1 (SP9131)

สาเหตุ: SPNDL ADJUSTMENT FUNCTION ALARM

การแก้ไข: An error occurred during spindle adjustment function operation.

See the description of the error and action to be taken that are displayed on the SERVO GUIDE.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d2 (SP9132)

สาเหตุ: SER.SENSOR DATA ERROR

การแก้ไข: An error occurred in communication between the (serial) detector and the spindle amplifier.

When this alarm is issued at the start, possible causes are (1) to (5) below. When the machine has been operating normally, possible causes are (3) and (5).

(1) The setting of a parameter is incorrect.

Check the detector setting parameters while referring to the section on startup in the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) The spindle amplifier does not support your detector.

Read the section on the detector in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN).

(3) The cable is disconnected or incorrectly connected.

Replace the cable or check connection.

(4) If the alarm is issued after checking (1) to (3) above, the detector could be defective.

Replace the detector.

(5) If the alarm is issued after checking (1) to (4) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d3 (SP9133)

สาเหตุ: SER.SENSOR TRANSFER ERROR

การแก้ไข: It has been detected that serial data between the (serial) detector and the spindle amplifier received on the spindle amplifier side became abnormal due to noise.

Check the shield processing of the cables of the (serial) detector and spindle amplifier.

Read the section on the detector in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d4 (SP9134)

สาเหตุ: SER.SENSOR SOFT PHASE ERROR

การแก้ไข: A change to position data from the (serial) detector was detected exceeding the expected range.

When this alarm is issued at the start, possible causes are (1) and (2) below. When the machine has been operating normally, the possible cause is (2).

(1) The setting of a parameter is incorrect.

Check the detector setting parameters while referring to the section on startup in the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) Check the shield processing of the cables of the (serial) detector and spindle amplifier.

Read the section on the detector in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d5 (SP9135)

สาเหตุ: SAFE SPEED ZERO ALARM

การแก้ไข: This alarm can be issued only when Dual Check Safety is in use.

With the safety speed zero monitor request signal input, the move amount of the spindle motor exceeded the setting value.

(1) The sequence is incorrect. Check the sequence.

(2) The setting of a parameter is incorrect. Check No. 4460 and No. 4462.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d6 (SP9136)

สาเหตุ: SAFE SPEED ZERO MONITOR MISMATCH

การแก้ไข: The safety speed zero monitor status of the spindle amplifier does not match the safety speed zero monitor status of the CNC.

Replace the CPU card or spindle amplifier in the CNC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d7 (SP9137)

สาเหตุ: SP DEVICE COMMUNICATION ERROR

การแก้ไข:

(1) The control printed circuit board is not correctly attached.

Push in the control printed circuit board in the front section of the spindle amplifier as far as it will go. If it does not go in easily, pull it out and then push it in again.

(2) If the alarm is issued after checking the above item, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d8 (SP9138)

สาเหตุ: CURRENT LIMIT SETTING ERROR

การแก้ไข: Check the parameter setting (No. 4526).

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code d9 (SP9139)

สาเหตุ: SER.SENSOR PULSE MISS

การแก้ไข: An error occurred in the interpolation circuit in the (serial) detector.

An error occurred in the detection circuit in the (serial) detector. Replace the detection circuit.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code E0 (SP9140)

สาเหตุ: SER.SENSOR COUNT MISS

การแก้ไข: The number of pulses between one-rotation signals in the (serial) detector goes outside of the specified range.

An error occurred in the detection circuit in the (serial) detector. Replace the detection circuit.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code E1 (SP9141)

สาเหตุ: NO 1-ROT SERIAL SENSOR

การแก้ไข: A one-rotation signal for the (serial) detector did not occur within five rotations after power-on.

When this alarm is issued at the start, possible causes are (1) to (3) below. When the machine has been operating normally, possible causes are (2) and (3).

(1) The setting of a parameter is incorrect.

If the number of teeth for the detector is not correctly set, this alarm may be issued because the actual speed does not match the speed calculated with the spindle amplifier. Check the detector setting parameters while referring to the section on startup in the FANUC AC SPINDLE MOTOR αi/βi series PARAMETER MANUAL (B-65280EN).

(2) The detector is not correctly adjusted.

Adjust the sensor signal while referring to the section on the detector in the FANUC SERVO AMPLIFIER αi-B series DESCRIPTIONS (B-65412EN).

(3) An error occurred in the detection circuit in the (serial) detector.

Replace the detection circuit.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code E2 (SP9142)

สาเหตุ: SER.SENSOR ABNORMAL

การแก้ไข: An error occurred in the (serial) detector. This alarm is output if a third-party (serial) detector is used.

For details on the error, contact the detector manufacturer.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code E3 (SP9143)

สาเหตุ: CS HIGH SPEED CHANGE CMD ERROR

การแก้ไข: The Cs contour control high-speed switching function starts dividing the command with one-rotation signal not detected.

The sequence is incorrect. Correct the sequence.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code E4 (SP9144)

สาเหตุ: CURRENT DETECT CIRCUIT ERROR

การแก้ไข: The current detection circuit section in the spindle amplifier is abnormal.

Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code E5 (SP9145)

สาเหตุ: LOW VOLTAGE DRIVER

การแก้ไข: The power in the driver circuit in the spindle amplifier has dropped.

(1) The contact between the control printed circuit board and the spindle amplifier unit is poor.

Push in the control printed circuit board in the front section of the spindle amplifier as far as it will go. If it does not go in easily, pull it out and then push it in again.

(2) An error such as momentary drop occurred in the 24 VDC power input to the Power Supply.

Check the 24 VDC power supply.

(3) The communication cable (connector CXA2A, CXA2B) is not connected correctly.

Correct the incorrect connection of the communication cable. If this does not solve the problem, replace the communication cable.

(4) If the alarm is issued after checking (1) to (3) above, replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code E8 (SP9148)

สาเหตุ: AXIS NUMBER NOT SET

การแก้ไข: The spindle control software has detected that the spindle axis number is not set. Turn off the power supply and then turn it on.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code E9 (SP9149)

สาเหตุ: EXCESS EXTERNAL CURRENT DETECTION PHASE U OFFSET

การแก้ไข: Replace the cable between the filter module and the spindle amplifier, filter module, or spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code F0 (SP9150)

สาเหตุ: EXCESS EXTERNAL CURRENT DETECTION PHASE V OFFSET

การแก้ไข: Replace the cable between the filter module and the spindle amplifier, filter module, or spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code F1 (SP9151)

สาเหตุ: FILTER MODULE ERROR

การแก้ไข: Replace the cable between the filter module and the spindle amplifier, filter module, or spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code F2 (SP9152)

สาเหตุ: EXTERNAL CURRENT SENSOR DISCONNECTED

การแก้ไข: Replace the cable between the filter module and the spindle amplifier, filter module, or spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code F3 (SP9153)

สาเหตุ: SP NO FAILURE

การแก้ไข:

(1) The speed feedback cable is disconnected, has poor contact, or is not connected correctly.

Correct the connection of the speed feedback cable or replace the cable.

(2) The speed feedback signal is abnormal.

Replace the detector.

(3) An external force is being applied to the spindle motor.

Remove or decrease the external force.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code F4 (SP9154)

สาเหตุ: PHASE OPEN

การแก้ไข:

(1) The magnetic contactor is defective.  Check and replace the magnetic contactor.

(2) A power lead is disconnected.  Replace the power lead.

(3) The spindle motor winding is disconnected.  Replace the spindle motor.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code F5 (SP9155)

สาเหตุ: SP FAILURE (OPEN)

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code F6 (SP9156)

สาเหตุ: FAILURE OF CURRENT CTRL.

การแก้ไข: The current control of the spindle amplifier is defective. After checking the location of the program where the alarm was issued and the operation procedure, save the parameters, and contact FANUC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code F7 (SP9157)

สาเหตุ: SP FAILURE (SHORT)

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code F8 (SP9158)

สาเหตุ: MODE MISMATCH (DCS)

การแก้ไข: The acceptance test mode status of the spindle amplifier does not match the acceptance test mode of the CNC. Replace the CPU card or spindle amplifier in the CNC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G0 (SP9160)

สาเหตุ: THERMISTOR DISCONNECTION

การแก้ไข: Replace the spindle amplifier.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G1 (SP9161)

สาเหตุ: POW. CABLE SHORT CIRCUIT

การแก้ไข:

(1) Check the connection of the power lead of the spindle motor.

2) If connection of the power lead is normal, a motor may be defective. Replace the motor.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G2 (SP9162)

สาเหตุ: DELAY OF SP POS. DATA

การแก้ไข: After checking the location of the program where the alarm was issued and the operation procedure, save the parameters, and contact FANUC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G3 (SP9163)

สาเหตุ: ILLEGAL SP POS. DATA

การแก้ไข: After checking the location of the program where the alarm was issued and the operation procedure, save the parameters, and contact FANUC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G4 (SP9164)

สาเหตุ: MASTER AXIS SENSOR SIGNAL ERROR

การแก้ไข: The master axis sensor signal input to the slave axis amplifier is abnormal in the master axis sensor signal direct input method EGB. For input of the master axis sensor signal (JYA4), troubleshoot while referring to the subsection about alarm code 83.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G5 (SP9165)

สาเหตุ: CONTROL METHOD CHANGED

การแก้ไข: Check bit 7 of No. 4000 and turn OFF the CNC and amplifier and turn them ON again.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G6 (SP9166)

สาเหตุ: SPINDLE SYNC FUNCTION FOR GUIDE BUSH: SYNC EXCESS ERROR

การแก้ไข:

(1) Overload.  Review the operating conditions.

(2) The spindle amplifier is defective.  Replace the spindle amplifier.

(3) A power lead or the spindle motor is defective.  Replace the power lead or spindle motor.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G7 (SP9167)

สาเหตุ: SPINDLE CTRL SEQUENCE ERROR

การแก้ไข: Details of the abnormal condition can be identified with the status error number. Correct the ladder sequence while referring to the status error number in the diagnosis number No. 710 and "4.3.3 Status Error Display" in Part I. If the displayed status error number is not specified, contact FANUC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G8 (SP9168)

สาเหตุ: CONNECTION ERROR BETWEEN SP AND MOTOR

การแก้ไข: Check a connection error.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code G9 (SP9169)

สาเหตุ: DRIVE OFF CIRCUIT FAILURE (STO)

การแก้ไข: The safe torque-off (STO) function has detected an error in the interruption circuit.

(1) Check whether the settings for setting switch SW1 are correct.

(a) Setting switch SW1 for the spindle amplifier is off (use the STO function) even though the STO function is not used (bit 1 of parameter No. 4542=0).

(b) Setting switch SW1 for the spindle amplifier is on (do not use the STO function) even though the STO function is used (bit 1 of parameter No. 4542=1).

(2) Check if the STO-A and STO-B input cables are disconnected.

(3) Check if the input time is off by more than 10 s between the STO-A and the STO-B.

(4) The spindle amplifier is defective.

Replace the spindle amplifier.

(5) External equipment used for the STO function such as the safety PLC is abnormal.

Check external equipment.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code H0 (SP9170)

สาเหตุ: EXCESS ERROR AT SPINDLE STOP (SMART RIGID TAP)

การแก้ไข: Overrun has been detected on the hole bottom or at return to the R position during rigid tapping. After checking the location of the program where the alarm was issued and the operation procedure, save the parameters, and contact FANUC.

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code UU

สาเหตุ: FSSB MASTER PORT WIRE BREAK

การแก้ไข: FSSB communication error (connector COP10B).

For details, see "4.2.21 Alarm Code U."

=== END ===

=== MACHINE: FANUC SPINDLE bi-B SERIES (SPINDLE AMPLIFIER) ===

อาการ: Alarm Code LL

สาเหตุ: FSSB SLAVE PORT WIRE BREAK

การแก้ไข: FSSB communication error (connector COP10A).

For details, see "4.2.22 Alarm Code L."

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 000

สาเหตุ: A parameter which requires the power off was input, turn off power.

การแก้ไข: Turn off power.

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 001

สาเหตุ: TH alarm (A character with incorrect parity was input).

การแก้ไข: Correct the tape.

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 002

สาเหตุ: TV alarm (The number of characters in a block is odd). This alarm will be generated only when the TV check is effective.

การแก้ไข: Correct the tape.

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 003

สาเหตุ: Data exceeding the maximum allowable number of digits was input.

การแก้ไข: Refer to Appendix 4 (item-Max. programmable dimension).

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 004

สาเหตุ: A numeral or the sign "-" was input without an address at the beginning of a block.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 005

สาเหตุ: The address was not followed by the appropriate data but was followed by another address or EOB code.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 006

สาเหตุ: Sign "-" input error (Sign "-" was input after an address with which it cannot be used. Or two or more "-" signs were input.)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 007

สาเหตุ: Decimal point "." input error (A decimal point was input after an address with which it can not be used. Or two decimal points were input.)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 008

สาเหตุ: EOR was read without a program end command. (When parameter No. 396 EORRE=0)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 009

สาเหตุ: Unusable character was input in significant area.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 010

สาเหตุ: An unusable G code was commanded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 011

สาเหตุ: Feedrate was not commanded to a cutting block or the feedrate was inadequate.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 014

สาเหตุ:  1. In variable lead thread cutting, the lead increment and decrement value assigned to address K exceeds the maximum command value, or a command was given which would make the a negative value. (T only)

2. A synchronous feed is specified without the option for threading/synchronous feed. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 015

สาเหตุ: The number of commanded axes in a block exceed the number of simultaneously controlled axes allowed. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 020

สาเหตุ: An arc is specified so that the difference between the radiuses at the start point and the end point exceeds the allowable range (parameter No. 876). (When parameter No. 393 RADCHK=1)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 021

สาเหตุ: An axis not included in the selected plane (by using G17, G18, G19) was commanded in circular interpolation.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 023

สาเหตุ: In circular interpolation by radius designation, a negative value was commanded for address R. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 025

สาเหตุ: A 1digit -F command was specified in the circular interpolation mode.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 027

สาเหตุ: No axis is specified in G43 and G44 blocks for the tool length compensation type C. Offset is not canceled and another axis is instructed to be offset for the tool length compensation type C. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 028

สาเหตุ: In the plane selection command, two or more axes in the same direction are commanded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 029

สาเหตุ:  1.  The offset value specified by H code is too large. (M only)

2. The offset value specified by T code is too large. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 030

สาเหตุ:  1. The offset number specified by H code for tool length offset or cutter compensation is too large. (M only)

2. The offset number in T function specified for tool offset is too large. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 031

สาเหตุ: In setting an offset amount by G10, the offset number following address P was excessive or it was not specified.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 032

สาเหตุ: In setting an offset amount by G10, the offset amount was excessive.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 033

สาเหตุ:  1. A point of intersection cannot be determined for cutter compensation C. (M only)

2. A point of intersection cannot be determined for tool nose radius compensation. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 034

สาเหตุ:   1. The start up or cancel was going to be performed in the G02 or G03 mode in cutter compensation C. (M only)

2. The start up or cancel was going to be performed in the G02 or G03 mode in tool nose radius compensation. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 035

สาเหตุ:  1. G39 is commanded in cutter compensation B cancel mode or on the plane other than the offset plane. (M only)

2. Skip cutting (G31) was specified in tool nose radius compensation mode. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 036

สาเหตุ: Skip cutting (G31) was specified in cutter compensation mode. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 037

สาเหตุ:   G40 is commanded at the plane other than the offset plane in cutter compensation B.

The plane selected by using G17, G18 or G19 is changed in cutter compensation C mode. (M only)

The offset plane is switched in tool nose radius compensation. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 038

สาเหตุ:  1. Overcutting will occur in cutter compensation C because the arc start point or end point coincides with the arc center. (M only)

2. Overcutting will occur in tool nose radius compensation because the arc start point or end point coincides with the arc center. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 039

สาเหตุ: Chamfering or corner R was specified with a start-up, a cancel, or switching between G41 and G42 in tool nose radius compensation. The program may cause overcutting to occur in chamfering or corner R. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 040

สาเหตุ: Overcutting will occur in tool nose radius compensation in a canned cycle G90 or G94. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 041

สาเหตุ:  1. Overcutting will occur in cutter compensation C. (M only)

2. Overcutting will occur in tool radius compensation. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 042

สาเหตุ: Tool position compensation is commanded in tool radius compensation. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 043

สาเหตุ: An unusable T code was specified in an M06T ◻◻ command.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 044

สาเหตุ: One of G27 to G30 is commanded in canned cycle mode. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 046

สาเหตุ: Other than P2, P3 and P4 are commanded for 2nd, 3rd and 4th reference point return command.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 050

สาเหตุ: Chamfering and corner R are commanded in the thread cutting block.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 051

สาเหตุ: Improper movement or the move distance of the block next to that for which chamfering and corner R are commanded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 052

สาเหตุ: The block next to the block for which chamfering and corner R are commanded is not G01.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 053

สาเหตุ: In the chamfering and corner R commands, two or more of I, K and R are specified. Otherwise, the character after a comma (",") is not C or R in direct drawing dimensions programming.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 054

สาเหตุ: A block in which the chamfering or the corner R was specified includes a taper command.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 055

สาเหตุ: In the block for which chamfering and corner R are commanded, the move distance is commanded less than the corner R amount.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 056

สาเหตุ: Neither the end point nor angle is specified in the command for the block next to that for which only the angle is specified (A). In the chamfering command, I (K) is commanded for the X (Z) axis.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 057

สาเหตุ: Block end point is not calculated correctly in direct dimension drawing programming.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 058

สาเหตุ: Block end point is not found in direct dimension drawing programming.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 059

สาเหตุ: The program with the selected number cannot be searched, in external program number search.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 060

สาเหตุ: Commanded sequence number was not found in the sequence number search.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 061

สาเหตุ: Address P or Q is not specified in G70, G71, G72, or G73 command. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 062

สาเหตุ:

1. The depth of cut in G71 or G72 is zero or negative value.

2. The repetitive count in G73 is zero or negative value.

3. The negative value is specified to Δi or Δk in G74 or G75.

4. A value other than zero is specified to address U or W, though Δi or Δk is zero in G74 or G75.

5. A negative value is specified to Δd though the relief direction in G74 or G75 is determined.

6. Zero or a negative value is specified to the height of thread or depth of cut of 1st time in G76.

7. The specified minimum depth of cut in G76 is greater than the height of thread.

8. An unusable angle of tool tip is specified in G76.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 063

สาเหตุ: The sequence number specified by address P in G70, G71, G72, or G73 command cannot be searched. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 064

สาเหตุ: For the multiple repetitive cycle function, a pocket shape was specified in G71 or G72.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 065

สาเหตุ: G00 or G01 is not commanded at the block with the sequence number which is specified by address P in G71, G72, or G73 command. (T only) / Address Z (W) or X (U) was commanded in the block with a sequence number which is specified by address P in G71 or G72, respectively.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 066

สาเหตุ: An unallowable G code was commanded between two blocks specified by address P and Q in G71, G72 or G73. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 067

สาเหตุ: G70, G71, G72, or G73 command with address P and Q was specified in MDI mode. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 068

สาเหตุ: For the multiple repetitive cycle function, the number of pockets exceeded the limit (10) in type II of stock removal in turning.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 069

สาเหตุ: The final move command in the blocks specified by P and Q of G70, G71, G72 and G73 ended with chamfering or corner R. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 070

สาเหตุ: The memory area is insufficient.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 071

สาเหตุ: The address to be searched was not found. Or the program with specified program number was not found in program number search.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 072

สาเหตุ: The number of programs to be stored exceeded 63 or 125 (option).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 073

สาเหตุ: The commanded program number has already been used.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 074

สาเหตุ: The program number is other than 1 to 9999.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 076

สาเหตุ: Address P was not commanded in the block which includes an M98 command or a G65 command.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 077

สาเหตุ: The subprogram was called in three or five folds.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 078

สาเหตุ: A program number or a sequence number which was specified by address P in the block which includes an M98, M99, G65, or G66 was not found.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 079

สาเหตุ: The contents of the program stored in memory did not agree with the tape during collation.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 080

สาเหตุ: When the axis reached the area specified by parameter nos. 0731 and 0732, the PMC signals XAE and ZAE were not output from the machine tool. (Automatic tool compensation function) (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 081

สาเหตุ: Automatic tool compensation was specified without a T code. (Automatic tool compensation function) (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 082

สาเหตุ: T code and automatic tool compensation were specified in the same block. (Automatic tool compensation function) (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 083

สาเหตุ: In automatic tool compensation, an invalid axis was specified or the command is incremental. (Automatic tool compensation function) (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 085

สาเหตุ: When entering data in the memory by using ASR or Reader / Puncher interface, an overrun, parity or framing error was generated. The number of bits of input data or setting of baud rate is incorrect.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 086

สาเหตุ: While entering data into memory, using the Reader/Puncher interface, the ready signal (DR) was turned off.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 087

สาเหตุ: While entering data into memory, using the Reader/Puncher interface, the data input is not stopped, after 10 characters are read, when a stop command is specified.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 090

สาเหตุ: The reference point return cannot be performed normally because the reference point return start point is too close to the reference point or the speed is too slow.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 092

สาเหตุ: The commanded axis by G27 (Reference point return check) did not return to the reference point.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 094

สาเหตุ: The P type program restart cannot be used. (After program interruption, the coordinate system setting operation was performed.)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 095

สาเหตุ: The P type program restart cannot be used. (After program interruption, the external work offset amount changed.)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 096

สาเหตุ: The P type program restart cannot be used. (After program interruption, the work offset amount changed.)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 097

สาเหตุ: The P type program restart cannot be used. After power on, emergency stop or P/S 94 to 97 is reset, no automatic operation can be performed.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 098

สาเหตุ: Program restart was specified before the reference point return operation. Reference point return must be completed after: 1. Power is turned on 2. Emergency stop 3. A G28 was found during sequence number search

การแก้ไข: Reference point return must be completed after 1. Power is turned on 2. Emergency stop 3. A G28 was found during sequence number search

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 099

สาเหตุ: After completion of search in program restart, a move command is given with MDI.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 100

สาเหตุ: PWE is set to "1".

การแก้ไข: Turn it to 0 and reset the system.

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 101

สาเหตุ: While writing to part program memory, the power was turned off.

การแก้ไข: Set PWE to "1" then turn the power off. Turn the power on while depressing the delete button. This will clear part program memory only.

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 110

สาเหตุ: The absolute value of fixed decimal point display data exceeds the allowable range.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 111

สาเหตุ: The calculation result of the macro instruction exceeds the allowable range (-2^31 to 2^31 - 1).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 112

สาเหตุ: Division by zero was specified. (including tan 90°)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 113

สาเหตุ: A function which cannot be used in custom macro is commanded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 114

สาเหตุ: An undefined H code is designated in the G65 block. For custom macro A / There is an error in the format other than <formula>. A variable is "vacant". For custom macro B

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 115

สาเหตุ: A value not defined as a variable number is designated. Or the header contents are improper. This alarm is given in the following cases: High speed cycle machining

1. The header corresponding to the specified call machining cycle number is not found.

2. The cycle connection data value is out of the allowable range (0-999).

3. The number of data in the header is out of the allowable range (1-32767).

4. The storing start data variable number of executable format data is out of the allowable range (#20000-#85535).

5. The last storing data variable number of executable format data is out of the allowable range (#85535).

6. The storing start data variable number of executable format data is overlapped with the variable number used in the header.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 116

สาเหตุ: The variable number designated with P is forbidden for assignment. The left side of the substitution statement is a variable whose substitution is inhibited.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 118

สาเหตุ: The nesting of brackets exceeds the upper limit (5).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 119

สาเหตุ: The argument of SQRT or BCD is negative. The SQRT statement or the BCD statement is negative. Values other that 0 to 9 are present on each line of the BIN argument. For custom macro B.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 122

สาเหตุ: The macro modal call is specified twice. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 123

สาเหตุ: Macro control command is used during DNC operation.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 124

สาเหตุ: DO END does not correspond to 1: 1.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 125

สาเหตุ: An unusuable address is used in G65 block. For custom macro A / <Formula> format is erroneous. For custom macro B.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 126

สาเหตุ: In DOn, 1 <= n <= 3 is not established.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 127

สาเหตุ: NC and macro commands are confused.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 128

สาเหตุ: The sequence number specified in the branch command was not 0 to 9999. Or, it cannot be searched.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 129

สาเหตุ: An address which is not allowed in <Argument Designation> is used. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 130

สาเหตุ: In axis control, a 3rd axis control command was given by PMC during Cf control. On the contrary, an attempt was made for Cf control from PMC during axis control.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 131

สาเหตุ: Five or more alarms have been generated in the external alarm message.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 132

สาเหตุ: No alarm No. concerned exists in external alarm message clear.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 133

สาเหตุ: Small section data is erroneous in external alarm message or external operator message.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 135

สาเหตุ: An attempt was made to index the spindle before the spindle was oriented. (T series only) / In spindle indexing by index table, a positioning angle which is not a multiple of the minimum angle was specified. (M series only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 136

สาเหตุ: Another axis was specified to move in the same block as spindle indexing addresses C and H. (T series only) / In spindle indexing by index table, another control axis was specified as well as the B axis. (M series only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 137

สาเหตุ: Other axes were commanded to move in the same block as the spindle indexing M code. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 139

สาเหตุ: An axis is selected in commanding by PMC axis control.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 140

สาเหตุ: An attempt was made in the background to select or delete a program being selected in the foreground.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 141

สาเหตุ: G51 (Scaling ON) is commanded in the tool offset mode. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 142

สาเหตุ: Scaling magnification is commanded in other than 1-999999. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 143

สาเหตุ: The scaling results, move distance, coordinate value and circular radius exceed the maximum command value. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 144

สาเหตุ: The coordinate rotation plane and arc or tool offset C plane differ from each other. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 145

สาเหตุ: The condition of the polar coordinate interpolation start or cancel is not correct. G112/G113 is commanded in other mode than G40. There is a plane selection error. (Error of parameter setting (No.291, 292)) (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 146

สาเหตุ: A G code, which should not be used with polar coordinate interpolation, was specified. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 148

สาเหตุ: Automatic corner override deceleration rate is out of the settable range of judgement angle. Check parameter No.0213, 0214, 0215. (M only)

การแก้ไข: Check parameter No.0213, 0214, 0215.

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 150

สาเหตุ: The Tool Group No. exceeds the maximum allowable value. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 151

สาเหตุ: The tool group commanded in the machining program is not set. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 152

สาเหตุ: The number of tools within one group exceeds the maximum value registerable. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 153

สาเหตุ: A T-code is not stored in the next block. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 154

สาเหตุ: When the group is not commanded, H99 or D99 was commanded. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 155

สาเหตุ: In the machining program, M06 and T code in the same block do not correspond to the group in use. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 156

สาเหตุ: The P and L commands are missing at the head of the program in which the tool group is set. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 157

สาเหตุ: The number of tool groups to be set exceeds the maximum allowable value. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 158

สาเหตุ: The tool life to be set is too excessive. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 159

สาเหตุ: The power was turned off during the execution of the setting program. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 160

สาเหตุ: A different M code is commanded in heads 1 and 2 as the waiting M code. (0TT only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 163

สาเหตุ: Both G68 and G69 were specified in balance cut. (TT only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 165

สาเหตุ: An attempt was made to run an even numbered program for head 1 or an odd numbered program for head 2. (0TT only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 169

สาเหตุ: Tool figure data was found to be incorrect in the interference check. (TT only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 175

สาเหตุ: Conditions when performing circular interpolation start or cancel not correct.

1. G107 is not commanded simultaneously with rotation axis radius.

2. G107 is commanded simultaneously with two axes.

3. G107 is commanded during cutter radius R compensation. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 176

สาเหตุ: A G code is commanded during circular interpolation mode when commands cannot be performed. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 177

สาเหตุ: A check sum error occurred in a high-speed remote buffer.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 178

สาเหตุ: Commanded during the G41/G42 mode.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 179

สาเหตุ: The number of controlled axes set by the parameter 597 exceeds the maximum number.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 180

สาเหตุ: A line alarm occurred in a high-speed remote buffer.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 190

สาเหตุ: In the constant surface speed control, the axis specification is wrong. (Misprogram) (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 194

สาเหตุ: The contouring mode, Cs axis control, or rigid tapping mode was specified in the synchronous control mode for serial spindle. (Release the synchronous control mode before specifying the above.) Alternatively, synchronous control was specified in the simplified synchronous control mode. (Release the simplified synchronous control mode before specifying synchronous control.)

การแก้ไข: Release the synchronous control mode before specifying the above. / Release the simplified synchronous control mode before specifying synchronous control.

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 195

สาเหตุ: Switching to the contouring mode, Cs axis control, rigid tapping mode, or spindle control mode has not correctly completed. (This occurs when the spindle control unit does not correctly respond to the switching command from the NC. This alarm is not a warning of an operation error, but a P/S alarm. This is because continuing the operation in this state may be dangerous.)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 197

สาเหตุ: The program specified the machine to move along the Cf axis or Cs contour axis when the COFF signal was on. (T series only) / The program specified the machine to move along the Cs contour axis when the CON signal was off. (M series only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 199

สาเหตุ: A macro cannot be executed.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 200

สาเหตุ: In the rigid tap mode, an S value is out of the range or is not specified. (programming error)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 201

สาเหตุ: In the rigid tap mode, no F value is specified. (programming error)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 202

สาเหตุ: In the rigid tap mode, spindle distribution value is too large (system error)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 203

สาเหตุ: In the rigid tap mode, position for M29 or an S command is incorrect.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 204

สาเหตุ: In the rigid tap mode, an axis movement is specified between the M29 and G84 (G74) blocks. (programming error)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 205

สาเหตุ: M29 was entered in the rigid tap mode. However, the rigid mode DI signal is not turned on when the G84 (G74) block is executed. (PMC error)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 206

สาเหตุ: Plane switching was specified in the rigid tapping mode.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 210

สาเหตุ: M198 and M099 are executed in the schedule operation. M198 is executed in the DNC operation.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 211

สาเหตุ: G31 is commanded in the per revolution command when the high-speed skip option is provided.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 212

สาเหตุ: The direct drawing dimensions programming is commanded for the plane including an additional axis. (M only) / The direct drawing dimensions programming is commanded for the plane other than the Z-X plane. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 213

สาเหตุ: The 4th axis was specified in the program in synchronous control. Jog, handle, or step feed was specified for the 4th axis in synchronous control. Low-speed reference position return was specified for the master axis in synchronous control. The position deviation between the master axis and 4th axis exceeded the limit in synchronous control.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 214

สาเหตุ: Coordinate system is set or tool compensation of the shift type is executed in the synchronous control. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 217

สาเหตุ: G251 is commanded again while still in the G251 mode. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 218

สาเหตุ: P or Q is not commanded in the G251 block, or the command value is out of the range. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 219

สาเหตุ: G251 and G250 are not independent blocks. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 220

สาเหตุ: In the synchronous operation, movement is commanded by the NC program or the PMC axis control interface for the synchronous axis. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 221

สาเหตุ: Polygon machining synchronous operation and axis control or balance cutting are executed at the same time. (T only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 222

สาเหตุ: Input and output are executed at the same time in background edit. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 224

สาเหตุ: A move command was attempted before reference position return after the power was turned on. (Parameter No. 74; CRFX, CRFZ, CRF3, and CRF4)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 225

สาเหตุ: In mixed synchronous control for the feed axis, 1. Parameters for axis numbers (No. 269 to 274) contain incorrect setting. 2. A control command contains an error. (0TT only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 226

สาเหตุ: In mixed synchronous control for the feed axis, an axis to be synchronized was specified to move in the synchronous mode.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 232

สาเหตุ: T0 was specified in special tool compensation.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 233

สาเหตุ: The skip function triggered by the torque limit-reached signal failed to return to the due position.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Program errors (P/S alarm)) ===

อาการ: Alarm 250

สาเหตุ: The Z-axis was specified in a simultaneous positioning command when changing tools. Moving along Z-axis cannot be specified together with the tool change command (M06).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n0

สาเหตุ: Manual return to the reference position is necessary for the n axis.

การแก้ไข: Manual return to the reference position is necessary for the n axis.

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n1

สาเหตุ: n-axis APC communication error. Failure in data transfer.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n2

สาเหตุ: n-axis APC overtime error. Failure in data transfer.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n3

สาเหตุ: n-axis APC framing error. Failure in data transfer.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n4

สาเหตุ: n-axis APC parity error. Failure in data transfer.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n5

สาเหตุ: n-axis APC missing pulse alarm. APC alarm.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n6

สาเหตุ: The n-axis APC battery voltage has dropped to the level at which the data can no longer be held. APC alarm.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n7

สาเหตุ: The n-axis APC battery voltage is at the level where the battery must be replaced. APC alarm.

การแก้ไข: Replace the battery.

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n8

สาเหตุ: The n-axis APC battery voltage has reached the level where the battery must be replaced (including when the power is off). APC alarm.

การแก้ไข: Replace the battery.

=== END ===

=== MACHINE: FANUC SERIES O (Alarm on the absolute pulse coder (APC)) ===

อาการ: Alarm 3n9

สาเหตุ: Abnormality of the n-axis serial pulse coder or failure in communication. (Failure in data transfer) See No. 760 to 767 and No. 770 to 777 in the diagnostic display (p.44) for details.

การแก้ไข: See No. 760 to 767 and No. 770 to 777 in the diagnostic display for details.

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 400

สาเหตุ: The overload signal was turned on.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 401

สาเหตุ: The velocity control READY signal (VRDY) was turned off.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 402

สาเหตุ: The 4th-axis overload signal was turned on.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 403

สาเหตุ: The 4th-axis velocity control READY signal (VRDY) was turned off.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 404

สาเหตุ: The position control READY signal (PRDY) was turned off when the velocity control READY signal (VRDY) was not turned off. Alternatively, the velocity control READY signal (VRDY) was turned on, although the power-on READY signal (PRDY) had not yet been turned on.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 405

สาเหตุ: A position control system error. Reference position return may have failed due to a problem in the NC or the servo system.

การแก้ไข: Restart operations with manual reference position return.

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 406

สาเหตุ: The overload signal for the 7th or 8th axis was turned on. Alternatively, the velocity control READY signal (VRDY) was turned off.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 408

สาเหตุ: The spindle control unit is not correctly started when the power is turned on in a system with a serial spindle. The cause may be one of the following: Connections of optical cables are erroneous or the power is not supplied to the spindle control unit. Serial spindle parameters are set incorrectly or are not set. Other problems, such as a hardware configuration error. After the system and the spindle control unit are started, this alarm does not occur.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 409

สาเหตุ: This alarm indicates on the NC unit that an alarm occurred in the spindle unit in a system with a serial spindle. The nature of the alarm is displayed on the spindle control unit as AL-XX (where XX is a number).

การแก้ไข: Refer to the documents related to the spindle control unit for details.

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 4n0

สาเหตุ: The motor or servo amplifier is overloaded. n axis.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 4n1

สาเหตุ: The position deviation while the machine is stopped is greater than the set value for the n axis.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 4n3

สาเหตุ: The position deviation while the machine is moving is greater than the set value for the n axis.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 4n4

สาเหตุ: The contents of the n-axis error register exceeded ±32767 or the velocity specified for the DA converter is not between -8192 and +8191. Generally, this error is caused by setting errors.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 4n5

สาเหตุ: An error occurred in the digital servo system for the n axis. Details of the error are output to DGNOS No. 720 to 727. Digital servo system alarm.

การแก้ไข: Check DGNOS No. 720 to 727.

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 4n6

สาเหตุ: An attempt was made to specify a velocity exceeding 511875 detection units per second for the n axis. This error is caused by a CMR setting error. There is an error in the position detection system for the n-axis pulse coder. (Disconnection alarm) This alarm can be detected by the hardware or software. (Digital servo only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES 0 ===

อาการ: Alarm 4n7

สาเหตุ: This alarm occurs when the n axis enters any of the following states. Digital servo system alarm.

(1) A value out of the specified range is set in parameter No. 8n20 for the motor type.

(2) A proper value (111 or -111) is not set in parameter No. 8n22 for the rotating direction of the motor.

(3) An improper value such as 0 or less is set in parameter No. 8n23 for the number of velocity feedback pulses per motor rotation.

(4) An improper value such as 0 or less is set in parameter No. 8n24 for the number of position feedback pulses per motor rotation.

(5) An improper value such as 0 or less is set in parameter No. 8n23 for the number of position feedback pulses per motor rotation.

(6) An overflow occurred in a parameter operation.

(1) The following formula yielded a value exceeding 32767 when a high-resolution pulse coder is used.

POA1×Np2000×11+LDINT/256

(2) The following formula yielded a value exceeding 32767 when position detection is controlled in units of 1 μm and velocity detection in units of 0.1 μm.

655×PG20×Value of No. 8n23Value of No. 8n24×4DMR

(3) Options for 0.1 μm control are not specified when position detection is controlled in units of 0.1 μm.

(7) Parameters for axis selection are incorrect. Parameters for axis selection are No. 269 to 274. Options for the 4th-axis interface are not specified for M series.

(8) Any bit 4 of No. 770 to 777 is 1 on the DGN display when the serial pulse coder is used. The cause may be one of the following

1. The flexible feed gear is not set. Parameter 8n84 (numerator of DMR) Parameter 8n85 (denominator of DMR)

2. The velocity loop period of the serial pulse coder is set to 2 ms.

3.cAn improper value is set for the flexible feed gear, number of velocity pulses, or number of position pulses. Parameter 8n23 (Number of velocity pulses) Parameter 8n24 (Number of position pulses)

การแก้ไข: For (6)(1): When the above formula yields a value exceeding 32767, set POA1 to 0.

For (6)(2): When the above formula yields a value exceeding 32767, reduce the parameter value for the position gain. Alternatively make the system semi-closed, change the high-resolution pulse coder, and use the DMR 1/5 function.

For (8): Set bit 1 of parameter 8n04 to 1 (when the velocity loop period of the serial pulse coder is set to 2 ms).

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 491

สาเหตุ: Speed control READY signal (VRDY) on the sub-side is OFF.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 494

สาเหตุ: Although the position control READY signal (PRDY) on the sub-side is OFF, the speed control READY signal (VRDY) is not OFF. Or, although the READY signal (PRDY) is not ON when the power was turned on, the speed control READY signal (VRDY) is ON.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Servo alarms) ===

อาการ: Alarm 495

สาเหตุ: Position control system abnormality on the sub-side. Considering the reference point return, it is possible that there is an abnormality internally in the CNC or the sub-system and the reference point could not return correctly.

การแก้ไข: Correct from the manual reference point return (by PMC axis control).

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 510

สาเหตุ: Overtravel to exceed the (+) stroke limit of X-axis.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 511

สาเหตุ: Overtravel to exceed the (-) stroke limit of X-axis.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 512

สาเหตุ: 2nd stroke limit at "+" side of X-axis was exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 513

สาเหตุ: 2nd stroke limit at "-" side of X-axis was exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 514

สาเหตุ: The hardware OT in the "+" side of X-axis is exceeded. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 515

สาเหตุ: The hardware OT in the "-" side of X-axis is exceeded. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 520

สาเหตุ: Overtravel to exceed the (+) stroke limit of Y-axis (M) or Z-axis (T).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 521

สาเหตุ: Overtravel to exceed the (-) stroke limit of Y-axis (M) or Z-axis (T).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 522

สาเหตุ: The 2nd stroke limit in the "+" side of Y-axis (M) or Z-axis (T) is exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 523

สาเหตุ: The 2nd stroke limit in the "-" side of Y-axis (M) or Z-axis (T) is exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 524

สาเหตุ: The hardware OT in the "+" side of Y-axis is exceeded. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 525

สาเหตุ: The hardware OT in the "-" side of Y-axis is exceeded. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 530

สาเหตุ: Overtravel to exceed the (+) stroke limit of Z-axis. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 531

สาเหตุ: Overtravel to exceed the (-) stroke limit of Z-axis. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 532

สาเหตุ: The 2nd stroke limit in the "+" side of Z-axis is exceeded. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 533

สาเหตุ: The 2nd stroke limit in the "-" side of Z-axis is exceeded. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 534

สาเหตุ: The hardware OT in the "+" side of Z-axis is exceeded. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 535

สาเหตุ: The hardware OT in the "-" side of Z-axis is exceeded. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 540

สาเหตุ: Overtravel to exceed the (+) stroke limit of the 4th-axis. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 541

สาเหตุ: Overtravel to exceed the (-) stroke limit of the 4th-axis. (M only)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 550

สาเหตุ: The 5th axis + side stroke limit was exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 551

สาเหตุ: The 5th axis - side stroke limit was exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 560

สาเหตุ: The 6th axis + side stroke limit was exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 561

สาเหตุ: The 6th axis - side stroke limit was exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 570

สาเหตุ: The stroke limit in the "+" side of the 7th axis is exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 571

สาเหตุ: The stroke limit in the "-" side of the 7th axis is exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 580

สาเหตุ: The stroke limit in the "+" side of the 8th axis is exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overtravel alarms) ===

อาการ: Alarm 581

สาเหตุ: The stroke limit in the "-" side of the 8th axis is exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (ALARM IN PMC) ===

อาการ: Alarm 600

สาเหตุ: An illegal command interruption has occurred in the PMC.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (ALARM IN PMC) ===

อาการ: Alarm 601

สาเหตุ: A PMC RAM parity error has occurred.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (ALARM IN PMC) ===

อาการ: Alarm 602

สาเหตุ: A PMC serial transmission error has occurred.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (ALARM IN PMC) ===

อาการ: Alarm 603

สาเหตุ: A PMC watchdog error has occurred.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (ALARM IN PMC) ===

อาการ: Alarm 604

สาเหตุ: A PMC ROM parity error has occurred.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (ALARM IN PMC) ===

อาการ: Alarm 605

สาเหตุ: The ladder storage area of the PMC has been exceeded.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (ALARM IN PMC) ===

อาการ: Alarm 606

สาเหตุ: Allocation data for a conventional I/O module is erroneous. (Not an alarm for the FANUC I/O LINK)

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (ALARM IN PMC) ===

อาการ: Alarm 607

สาเหตุ: SLC ERROR (xxx) Alarm for the FANUC I/O LINK

Description of SLC errors (XXX)

010 Communication error (Internal register error of the SLC (master))

020 SLC RAM bit error (verification error)

030 SLC RAM bit error (verification error)

040 I/O units are not connected or more I/O units are connected.

050 Data transmission error (no response from the slave unit)

060 Communication error (no response from the slave unit)

070 Communication error (no response from the slave unit)

080 NMI occurred. (NMI with an alarm code other than 110 to 160)

090 SLC (master) RAM parity error (detected by hardware)

130 SLC (slave) RAM parity error (detected by hardware)

140 SLC (slave) communication error

160 ALO: Watchdog alarm DO clear reception / IR1: CRC or framing error / Watchdog alarm / Parity error (indicates a hardware error).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overheat alarm) ===

อาการ: Alarm 700

สาเหตุ: The master PCB is overheated.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Overheat alarm) ===

อาการ: Alarm 704

สาเหตุ: Spindle overheat is detected by the spindle fluctuation detection feature.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (M-NET interface alarm) ===

อาการ: Alarm 899

สาเหตุ: Alarm (xxx) from the serial interface with the external PLC. xxx indicates the detail number:

0001 Abnormal character received (character other than the codes used for transmission)

0002 EXT code error

0003 Connection time monitoring error (parameter No. 464)

0004 Polling time monitoring error (parameter No. 465)

0005 Vertical parity or framing error detected

0257 Transmission time-out (parameter No. 466)

0258 ROM parity error

0259 Overrun detected

Other than the above: CPU interrupt detected

การแก้ไข: Check parameters No. 464, 465, 466.

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 910

สาเหตุ: There is a RAM parity error (low byte).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 911

สาเหตุ: There is a RAM parity error (high byte).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 912

สาเหตุ: There is a parity error of the RAM that is shared with the digital servo (low).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 913

สาเหตุ: There is a parity error of the RAM that is shared with the digital servo (high).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 914

สาเหตุ: There is a parity error of the digital servo local RAM.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 915

สาเหตุ: RAM parity error (low-order bytes) of the ladder program editing cassette

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 916

สาเหตุ: RAM parity error (high-order bytes) of the ladder program editing cassette

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 920

สาเหตุ: There is a watchdog alarm.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 921

สาเหตุ: Servo alarm for 5/6 axis

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 922

สาเหตุ: Servo alarm for 7/8 axis

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 930

สาเหตุ: There is a CPU error (abnormal interrupt).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 940

สาเหตุ: A printed circuit board which cannot be used with the system software is mounted.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 941

สาเหตุ: The memory printed circuit board is not in the correct position. (For MODEL C).

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 945

สาเหตุ: Hardware for serial spindle control does not operate correctly. Alternatively, a communication error occurred in serial communication with the serial spindle.

There are two cases:

1. If the alarm occurs immediately after the power is turned on, the memory card on the NC unit may not be a card which can be used with a serial spindle. Check the drawing number of the card (A16B-1212-0215 for SIC-1 and A16B-1212-0216 for SIC-2). If the drawing number is correct, the memory card needs to be replaced because the printed circuit board is defective.

2. If the alarm occurs while the system is being activated after the power is turned on, a communication error may have occurred. Generally, the cause is a disconnected or broken optical cable.

When the SICs in the NC unit and spindle control unit are not the same LSI chip, a communication error occurs, causing the alarm.

การแก้ไข: 1. Check the drawing number of the card (A16B-1212-0215 for SIC-1 and A16B-1212-0216 for SIC-2). If the drawing number is correct, the memory card needs to be replaced because the printed circuit board is defective.

2. Check for disconnected or broken optical cable.

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 946

สาเหตุ: In serial spindle control, communication with the second serial spindle amplifier is impossible although the second amplifier is specified to be connected. (Check that the second amplifier is connected and activated.)

การแก้ไข: Check that the second amplifier is connected and activated.

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 950

สาเหตุ: Fuse disconnection alarm.

การแก้ไข: Replace the +24E Fuse.

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 960

สาเหตุ: NMI occurred in the sub-CPU.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (System alarm) ===

อาการ: Alarm 998

สาเหตุ: There is a ROM parity error.

การแก้ไข: -

=== END ===

=== MACHINE: FANUC SERIES O (Background edit alarm (BP/S)) ===

อาการ: Alarm BP / S

สาเหตุ: BP/S alarm is produced with the same number as P/S alarm taking place in normal program editing. (070, 071, 072, 073, 074, etc.)

การแก้ไข: Reset it by any subsequent MDI key operation.

=== END ===

=== MACHINE: FANUC SERIES O (Background edit alarm (BP/S)) ===

อาการ: Alarm 140

สาเหตุ: An attempt was made to select or delete a program being selected in foreground.

การแก้ไข: Reset it by any subsequent MDI key operation.

=== END ===