=== MACHINE: TOYODA GCS32 ===

อาการ: alarm a000

สาเหตุ:

1. maintenance parameters were written.

2. auxiliary axis parameters were written.

3. a parameters was written at the initialization of sercans.

การแก้ไข: exit the cnc. stop the windows. shut off the power then turn it back on.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A001

สาเหตุ : The internal absolute position data was input at the AXIS POSITION DATA SETTING screen.

การแก้ไข: Turn the power OFF according to the procedure as described in <Shutting OFF the power> in Page 6-1

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A012

สาเหตุ :

1. sercans was not initialized after replacing the pci-sercans board.

2. sercans has not been initialized after changing the maintenance parameter setting.

3. sercens parameter was change from the serctop ***

การแก้ไข:

1. initialize sercans

2. Return the setting of the parameter changed By the SERCTOP.

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm A400

สาเหตุ:

(a) The PLC is in the halt state.

(b) The sequence program is incorrect.

(c) Data communications between the MCUL and the PLC is abnormal.

(d) An error occurred in the servo system.

การแก้ไข:

(a) Start the PLC.

(b) Correct the sequence program.

(c) Replace the MCUL unit or the PLC unit. (It is necessary to save the CNC data or the sequence program before replacing the unit and recover the saved data or the program after the replacement.)

(d) Check the alarm status information display of the CNC and take corrective action.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A401

สาเหตุ :

1. The PLC is in the halt state.

2. The sequence program is incorrect.

3. Data communications between the MCUL and the PLC is abnormal.

การแก้ไข:

1. Start the PLC.

2. Correct the sequence program. (5.10.3 “Checking the Circuit Using LdNT”)

3. Replace the MCUL unit or the PLC unit. It is necessary to save the CNC data or the sequence program before replacing the unit and recover the saved data or the program after the replacement.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A402

สาเหตุ:

1. The emergency stop button was pressed or the power was shut off while an axis was moving.

2. An axis was moved while the power was off (example: the ball screw was rotated with a wrench).

3. The servomotor was replaced.

4. The position encoder of the servomotor is faulty.

5. The CNC memory was initialized.

การแก้ไข:

- Clear the alarm by following the procedure below. (5.4.4 "Canceling the Position-offset Safety Protection Function")

- Press the [CNC RESET] button.

- Record the data at the upper field (encoder) of INT ABS POSI of the axis with which the alarm took place.

- Input the recorded value at the axis position setting screen.

- Shut off the power and then turn it on according to the procedure explained in page 6-1.

- Turn on MASTER ON and check the coordinate values.

- Follow the same operation as explained in (a).

- Follow the procedure explained in the servomotor replacement procedure.

- Replace the servomotor following the servomotor replacement procedure.

10.1 Record the internal absolute position of all axes before initializing the memory.

2 Press the [CNC RESET] button.

3 Record the data at the upper field (encoder) of INT ABS POSI of the axis with which the alarm took place.

4 Input the recorded value at the axis position setting screen.

5 Shut off the power and then turn it on according to the procedure explained in page 6-1.

6 Turn on MASTER ON and check the coordinate values.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A430

สาเหตุ:

1. Optical fiber cable connection is incorrect or the optical fiber cable is broken.

2. Setting of the data transmission speed at the drive side is incorrect.

การแก้ไข:

  - Replace the optical fiber cable.

  - Set the correct data transmission speed at the SERCOS interface at the driver side.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A434

สาเหตุ:

  - Intensity of the light is not set correctly.

  - Optical fiber cable connection is incorrect.

  - SERCOS interface at the CNC side or the driver side is faulty.

การแก้ไข:

  - Set the light intensity at the driver side SERCOS interface.

  - Connect the optical fiber cable correctly.

  - Replace the faulty SERCOS interface.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A447

สาเหตุ :SERVO ALARM (SERVO DRIVE AMPLIFIER ALARM)

การแก้ไข: REMOVE ALARM IN ACCORDANCE WITH DRIVE ALARM

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A449

สาเหตุ:

- The parameters necessary in SERCOS phase 3 are not set correctly.

- Software module is faulty.

การแก้ไข:

  - Input the SERCOS parameters.

  - Replace the software module.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A457

สาเหตุ:

  - The cooling fan is faulty.

  - The cooling unit of the electric cabinet is faulty.

การแก้ไข:

  - Replace the servo amplifier.

  - Replace the cooling unit.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A459

สาเหตุ:

  - The cable supplying the power for the control is faulty.

  - The power supply unit is faulty.

การแก้ไข:

  - Check and replace the cable or the connector.

  - Check and replace the power supply unit.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A460

สาเหตุ:

  - The encoder cable is broken.

  - The encoder interface is faulty.

  - The feedback cable and the power cable run too closely with each other.

การแก้ไข:

  - Replace the encoder cable.

  - Replace the encoder interface.

  - Run the feedback cable apart from the power cable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A462

สาเหตุ:

  - The optical fiber cable is broken.

  - Damping of light in the fiber optical cable is excessively large.

  - SERCOS interface is faulty.

  - Loose fiber optical cable connector.

การแก้ไข:

  - Replace the optical fiber cable.

  - Replace a SERCOS interface if damping of light is large.

  - Replace the SERCOS interface.

  - Securely tighten the fiber optical cable connector.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A463

สาเหตุ : Maintenance or SERCOS parameter is not set correctly.

การแก้ไข: CHECK GRINDING DATA OR NC PROGRAM CAUSED ALARM

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A615

สาเหตุ: Amount of variation of encoder's feedback data exceeded 1.5 times the variation amount that would be detected in the operation at the maximum feedrate.

วิธีการแก้ไข:

- Check the mounting and connection of the motor encoder.

- Replace the motor encoder.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A616

สาเหตุ:

(a) The encoder is faulty.

(b) Connection of the motor encoder is faulty.

(c) Load inertia is excessively high.

การแก้ไข:

(a) Replace the motor encoder.

(b) Check the cable connection.

(c) Lower the load.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A621

สาเหตุ:

(a) The encoder is faulty.

(b) Connection of the motor encoder is faulty.

(c) Load inertia is excessively high.

การแก้ไข:

(a) Replace the motor encoder.

(b) Check the cable connection.

(c) Lower the load.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A657

สาเหตุ:

1. The alarm occurred when the control power was turned on: The CNC unit or the servo amplifier is faulty.

2. The alarm occurred when the ready signal was turned on: The servo amplifier is faulty.

3. The alarm occurred when the servo power was turned on:

  - Motor grounding is faulty.

  - The power module is faulty.

  - The CNC unit is faulty.

  - Wiring of the motor main circuit is incorrect.

4. The alarm occurred at the start or stop of the motor: Selection of the motor and amplifier combination is inadequate.

การแก้ไข:

1. If the alarm occurred when the control power was turned on: Replace the CNC unit or the servo amplifier.

2. If the alarm occurred when the control on signal was turned on: Replace the servo amplifier.

3. The alarm occurred when the servo power was turned on:

- Replace the motor.

- Replace the servo amplifier.

- Replace the CNC unit.

- Correct motor wiring.

4. The alarm occurred at the start or stop of the motor: Set the motor type (maintenance parameter) again. (Please contact the Service Department of TOYODA)

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A658

สาเหตุ:

1) The alarm occurred when the power was turned on: The servo amplifier is faulty.

2) The alarm occurred during the operation of the motor:

- The motor is operated under the torque exceeding the rated torque.

- The holding brake has not been released.

(3) Although the motor rotates, required torque is not output. The alarm occurred after operating the motor for a while:

  - Incorrect wiring in the motor main circuit or one of the motor main circuit lines (U, V, W) is open.

  - Selection of the motor and amplifier combination is inadequate.

(d) The alarm occurred even during operation below the rated torque: The CNC unit or the servo amplifier is faulty.

การแก้ไข:

(a) The alarm occurred when the power was turned on:  Replace the servo amplifier.

(b) The alarm occurred during the operation of the motor:

  - Review motor load.

  - Release the brake by exciting it.

(c) Although the motor rotates, required torque is not output. The alarm occurred after operating the motor for a while:

- Correct the motor wiring.

- Review the motor and servo amplifier combination.

(d) The alarm occurred even during operation below the rated torque: Replace the CNC unit or the servo amplifier.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A659

สาเหตุ:

(a) The alarm occurred when the power was turned on: The servo amplifier is faulty.

(b) The alarm occurred during motor operation:

- The cooling fan is faulty.

- Ambient temperature is high.

- The motor has been operated at the state exceeding the maximum continuous torque.

การแก้ไข:

(a) The alarm occurred when the power was turned on: Replace the servo amplifier.

(b) The alarm occurred during motor operation:

- Replace the servo amplifier.

- Lower ambient temperature.

- Lower load.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A661

สาเหตุ:

(a) The alarm occurred when the control power was turned on: The servo amplifier is faulty.

(b) The alarm occurred when the main power was turned on: Input voltage is too high or input voltage waveform is distorted.

(c) The alarm occurred during motor operation:

- Load inertia is too high.

- The regeneration transistor/resistor is faulty.

- External regeneration resistor is faulty.

การแก้ไข:

(a) The alarm occurred when the control power was turned on: Replace the servo amplifier.

(b) The alarm occurred when the main power was turned on: Adjust input voltage (200 ~ 220V AC).

(c) The alarm occurred during motor operation:

- Adjust load inertia within the specified value.

- Replace the servo amplifier.

- Replace the external regeneration resistor.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A665

สาเหตุ:

(a) The alarm occurred when the control power was turned on: The servo amplifier is faulty.

(b) The alarm occurred when the main power was turned on:

- Source voltage is too low.

- The rectifier is broken.

(c) The alarm occurred during motor operation:

- Input voltage fluctuates excessively.

- The servo amplifier is faulty.

การแก้ไข:

(a) The alarm occurred when the control power was turned on: Replace the servo amplifier.

(b) The alarm occurred when the main power was turned on:

- Adjust source voltage (200 ~ 220V AC).

- Replace the servo amplifier.

(c) The alarm occurred during motor operation:

- Adjust source voltage (200 ~ 220V AC).

- Replace the servo amplifier.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A666

สาเหตุ:

(a) The alarm occurred when the control power was turned on: The servo amplifier is faulty.

(b) The alarm occurred when the main power was turned on:

- Open phase in the thee phases of power supply.

- Loose input terminal connection of the servo amplifier.

การแก้ไข:

(a) The alarm occurred when the control power was turned on: Replace the servo amplifier.

(b) The alarm occurred when the main power was turned on:

- Correct the power source.

- Tighten the servo amplifier terminal.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A673

สาเหตุ:

(a) The alarm occurred when the control power was turned on:

- Voltage of the backup battery is low.

- The encoder is faulty.

(b) The alarm occurred during motor operation:

- Noise on the signal transmitted between the encoder and the servo amplifier.

- The encoder is faulty.

การแก้ไข:

(a) The alarm occurred when the control power was turned on:

- Replace the battery and after performing multi-turn clear processing, adjust the motor position.

- Replace the motor.

(b) The alarm occurred during motor operation:

- Check the wiring between the encoder and the servo amplifier.

- Replace the motor encoder.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A741

สาเหตุ:

(a) The emergency stop button was pressed during axis movement.

(b) The SVRDY signal was turned off during axis movement.

การแก้ไข:

(a) Press the NC reset button.

(b) Replace the servo amplifier or the PLC.

Check the servo ready signal circuit of the PLC.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A747

สาเหตุ:  Although the ready signal was turned on, power is not supplied to the main circuit of the servo amplifier.

การแก้ไข: Check the ready signal circuit of the PLC. Check the ready signal circuit by referring to the circuit diagram.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A861

สาเหตุ:

(a) The PLC unit is faulty.

(b) The sequence program is incorrect.

(c) Jumper setting or parameter setting of the PLC unit is faulty.

การแก้ไข:

(a) Replace the PLC unit.

(b) Correct the sequence program.

(c) Correct the jumper setting or parameter setting of the PLC unit.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm A862

สาเหตุ:

(a) The PLC unit is faulty.

(b) The sequence program is incorrect.

(c) Jumper setting or parameter setting of the PLC unit is faulty.

การแก้ไข:

(a) Replace the PLC unit.

(b) Correct the sequence program.

(c) Correct the jumper setting or parameter setting of the PLC unit.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B900

สาเหตุ:

(a) The sizer setting position is not set correctly.

(b) The workpiece has been ground or has a small diameter.

(c) The sizer is faulty.

การแก้ไข:

(a) Change the sizer setting position correctly.

(b) Set up the next workpiece.

(c) Repair or replace the sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B901

สาเหตุ:

(a) The sizer setting position is not set correctly.

(b) The sizer is faulty.

การแก้ไข:

(a) Change the sizer setting position correctly.

(b) Repair or replace the sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B903

สาเหตุ:

(a) The setting for A71 data is not correct. (If the difference is less than A70 data, this alarm is not given.)

(b) The work coordinate system is changed. The wheel OD position is changed (the wheel position memory operation is performed). Work position offset data is altered.

(c) Thermal displacement that exceeds A71 data occurs between the previous and current grinding cycles.

(d) Diamond wearout that exceeds A71 data occurs between the previous and current grinding cycles.

(e) The sizer is not working properly.

การแก้ไข:

(a) Set the correct data for A71.

(b) Perform the position memory operation of the wheel OD and the truing device, and reset the work position offset data to “0”.

(c) Perform the wheel OD position memory operation again. (Operation manual 6.5.1 “Wheel Shape Position Memory”)

(d) Perform the truing position memory operation again.

(e) Check the sizer (contact, amplifier, etc.) and put it in the correct working condition. (Operation manual 7.9 “Setup Changes of Sizer and Locator”)

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B904

สาเหตุ:

(a) Setting for A72 is too small.

(b) The work coordinate system was changed. * The wheel OD position was changed. (Wheel shape position memory was executed.)

(c) Thermal displacement larger than the value set for A72

(d) Diamond tool wear larger than the value set for A72

(e) The auto-sizer is faulty.

การแก้ไข:

(a) Set the correct data for A72.

(b) Perform the position memory operation of the wheel shape and the truing device, and reset the work position offset data to “0”.

(c) Perform the wheel OD position memory operation again. (Operation manual 6.5.1 “Wheel Shape Position Memory”)

(d) Perform the truing position memory operation again.

(e) Check the sizer (contact, amplifier, etc.) and put it in the correct working condition. (Operation manual 7.9 “Setup Changes of Sizer and Locator”)

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B910

สาเหตุ:

(a) The contact detection pin is dislocated.

(b) The AE sensor of the contact detection pin is faulty.

การแก้ไข:

(a) Execute the position data memory operation (contact detection) again. (Operation manual 6.5.3 “Contact Detection Pin Position Memory”)

(b) Check the AE sensor of the contact detection pin.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B911

สาเหตุ:

(a) The contact detection pin is dislocated.

(b) The AE sensor of the contact detection pin is faulty.

การแก้ไข:

(a) Execute the position data memory operation (contact detection) again. (Operation manual 6.5.3 “Contact Detection Pin Position Memory”)

(b) Check the AE sensor of the contact detection pin.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B912

สาเหตุ:

(a) The contact detection pin is dislocated.

(b) The AE sensor of the contact detection pin is faulty.

การแก้ไข:

(a) Execute the position data memory operation (contact detection) again. (Operation manual 6.5.3 “Contact Detection Pin Position Memory”)

(b) Check the AE sensor of the contact detection pin.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B913

สาเหตุ:

(a) The contact detection pin is dislocated.

(b) The AE sensor of the contact detection pin is faulty.

การแก้ไข:

(a) Execute the position data memory operation (contact detection) again. (Operation manual 6.5.3 “Contact Detection Pin Position Memory”)

(b) Check the AE sensor of the contact detection pin.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B919

สาเหตุ:

(a) The cooler of the coolant is faulty.

(b) Coolant thermometer is faulty.

การแก้ไข:

(a) Repair or replace the cooler.

(b) Check the thermometer and replace it if faulty.  (6.5.3 “Replacing the Coolant Cooler”)

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B91A

สาเหตุ:

(a) The contact detection pin is dislocated.

(b) The AE sensor of the contact detection pin is faulty.

(c) Large wheel wear amount

(d) Large thermal displacement

การแก้ไข:

(a) Execute the position data memory operation (contact detection) again. (Operation manual 6.5.3 “Contact Detection Pin Position Memory”)

(b) Check the AE sensor of the contact detection pin.

(c) Change the kind of CBN wheel meeting the workpiece material.

(d) Check the coolant temperature. Check the coolant cooler if temperature varies excessively.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B19D

สาเหตุ:

(a) The contact detection pin is dislocated.

(b) The AE sensor of the contact detection pin is faulty.

(c) The diamond roll position is incorrect.

(d) The wheel position is incorrect.

การแก้ไข:

(a) Execute the position data memory operation (contact detection) again. (Operation manual 6.5.3 “Contact Detection Pin Position Memory”)

(b) Check the AE sensor of the contact detection pin.

(c) Execute diamond roll position memory operation again. (Operation manual 6.5.2 “Diamond Roll Position Memory”)

(d) Execute wheel shape position memory operation again. (Operation manual 6.5.1 “Wheel Shape Position Memory”)

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B920

สาเหตุ:

(a) Breakage of wiring, or the PLC input card is faulty.

(b) The inverter is faulty.

การแก้ไข:

(a) Check the wiring or the PLC input card.

(b) Check the inverter. (5.10 “Diagnosis Using TOYOPUC”)

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B921

สาเหตุ:

(a) Breakage of wiring, or the PLC input card is faulty.

(b) The inverter is faulty.

การแก้ไข:

(a) Check the wiring or the PLC input card.

(b) Check the inverter.  (5.10 “Diagnosis Using TOYOPUC”) , (6.13.1 “Replacing the Truing Motor Controlling Inverter”)

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B932

สาเหตุ:

(a) The wheelhead position is incorrect.

(b) The workpiece is a finished workpiece or a small-diameter workpiece.

(c) The auto-sizer is faulty.

การแก้ไข:

(a) Set the wheelhead position coordinate values.

(b) Set the next workpiece.

(c) Repair or replace the auto-sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B933

สาเหตุ:

(a) Poor roundness of the workpiece

(b) The auto-sizer is faulty.

การแก้ไข:

(a) Check if the workpiece is held correctly between the centers.

(b) Repair or replace the auto-sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B934

สาเหตุ:

(a) The wheelhead position is incorrect.

(b) The auto-sizer is faulty.

การแก้ไข:

(a) Set the wheelhead position coordinate values.

(b) Repair or replace the auto-sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B935

สาเหตุ:

(a) The wheelhead position is incorrect.

(b) The workpiece is a finished workpiece or a small-diameter workpiece.

(c) The auto-sizer is faulty.

การแก้ไข:

(a) Set the wheelhead position coordinate values.

(b) Set the next workpiece.

(c) Repair or replace the auto-sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B936

สาเหตุ:

(a) Poor roundness of the workpiece

(b) The auto-sizer is faulty.

การแก้ไข:

(a) Check if the workpiece is held correctly between the centers.

(b) Repair or replace the auto-sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B937

สาเหตุ:

(a) The wheelhead position is incorrect.

(b) The auto-sizer is faulty.

การแก้ไข:

(a) Set the wheelhead position coordinate values.

(b) Repair or replace the auto-sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B938

สาเหตุ:

(a) The wheelhead position is incorrect.

(b) The workpiece is a finished workpiece or a small-diameter workpiece.

(c) The auto-sizer is faulty.

การแก้ไข:

(a) Set the wheelhead position coordinate values.

(b) Set the next workpiece.

(c) Repair or replace the auto-sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B939

สาเหตุ:

(a) Poor roundness of the workpiece

(b) The auto-sizer is faulty.

การแก้ไข:

(a) Check if the workpiece is held correctly between the centers.

(b) Repair or replace the auto-sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm B93A

สาเหตุ:

(a) The wheelhead position is incorrect.

(b) The auto-sizer is faulty.

การแก้ไข:

(a) Set the wheelhead position coordinate values.

(b) Repair or replace the auto-sizer.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm C916

สาเหตุ:

(a) The proximity switch for pin insertion confirmation is not attached to the correct position.

(b) The proximity switch for pin insertion confirmation is faulty.

การแก้ไข:

(a) Adjust the position of the proximity switch for pin insertion confirmation.

(b) Check the proximity switch for pin insertion confirmation.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm C917

สาเหตุ:

(a) The proximity switch for pin insertion confirmation is not attached to the correct position.

(b) The proximity switch for pin insertion confirmation is faulty.

การแก้ไข:

(a) Adjust the position of the proximity switch for pin insertion confirmation.

(b) Check the proximity switch for pin insertion confirmation.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm C930

สาเหตุ: For all processes, the setting for C07 data (skip yes/no) is "1".

การแก้ไข: If the skip setting is "yes" for all processes, table indexing to the grinding position is not possible. Correct the setting for the C07 data (skip yes/no). (Operation manual 7.4.3 "Editing the Process Data")

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm D401

สาเหตุ:  (a) The M code finish conditions, specified in the circuit diagram, are not satisfied.

Example: M17 End face sizer advance command The M code finish signal is not returned to the CNC since the advance end proximity switch is not ON.

การแก้ไข: (a) Check the O and N numbers of the program involved with the alarm by displaying the DISPLAY ALARM screen and check the conditions that must be satisfied for the specified M code in the circuit diagram.

Check the M code finish conditions in the circuit diagram and check the unsatisfied condition by the maintenance PLC menu. (5.10 "Diagnosis Using TOYOPUC")

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm D401

สาเหตุ:(a) The M code finish conditions, specified in the circuit diagram, are not satisfied.

Example: M17 End face sizer advance command The M code finish signal is not returned to the CNC since the advance end proximity switch is not ON.

การแก้ไข: (a) Check the O and N numbers of the program involved with the alarm by displaying the DISPLAY ALARM screen and check the conditions that must be satisfied for the specified M code in the circuit diagram.

Check the M code finish conditions in the circuit diagram and check the unsatisfied condition by the maintenance PLC menu. (5.10 "Diagnosis Using TOYOPUC")

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm D407

สาเหตุ:

(a) The NC program start condition stays ON.

Example: The return NC program start condition is held and is not turned OFF.

(b) M02 code is ignored and the cycle start signal stays on.

Example: Value "1" was input for MB61 by mistake.

การแก้ไข:

(a) Check the O or N numbers indicated in the alarm code, and find the current cycle start condition in the electric circuit diagram. Reset the condition. (5.10 "Diagnosis Using TOYOPUC")

(b) Correct the parameter value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E900

สาเหตุ: Gap data between the wheel and the workpiece (A230) is not set correctly.

การแก้ไข: Enter a value of 10 mm (diameter) or greater for A230 (gap between the workpiece and the grinding wheel). (Operation manual 7.4.8 "Inputting the Machine Constant Data") (Operation manual 12.4 "Machine Constant Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E902

สาเหตุ: An incorrect value is set for C00 (grinding cycle pattern).

การแก้ไข: Check the present setting for C00 (grinding cycle pattern) and enter a correct value. (Operation manual 7.4.3 "Editing the Process Data") (Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E903

สาเหตุ: The profile number registration (C200) data of the grinding step has not been input.

การแก้ไข:  Check the profile number registration (C200) data and input the appropriate value. (Operation manual 7.4.3 "Editing the Process Data") (Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E903

สาเหตุ: The profile number registration (C200) data of the grinding step has not been input.

การแก้ไข:  Check the profile number registration (C200) data and input the appropriate value. (Operation manual 7.4.3 "Editing the Process Data") (Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E910

สาเหตุ: "0" is set for C40 (workpiece finish diameter).

การแก้ไข:  Check the present setting for C40 (workpiece finish diameter) and enter a correct value. (Operation manual 7.4.3 "Editing the Process Data") (Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E911

สาเหตุ: "0" is set for C42 (No. 2 step grinding start diameter, or coarse grinding start diameter).

การแก้ไข: Check the present setting for C42 (No. 2 step grinding start diameter, or rough grinding start diameter) and enter a correct value. (Operation manual 7.4.3 "Editing the Process Data") (Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E912

สาเหตุ: The relationship between the value set for C41 (No. 1 step grinding or air-cut grinding start diameter) and the value set for C42 (No. 2 step grinding or rough grinding start diameter) is incorrect.

การแก้ไข: Check and correct the values set for C41 and C42. (Operation manual 7.4.3 "Editing the Process Data") (Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E913

สาเหตุ: The relationship between the value set for C44 (fine grinding start diameter) and the value set for C42 (rough grinding start diameter) is incorrect.

การแก้ไข: Check and correct the values set for C42 and C44. (Operation manual 7.4.3 "Editing the Process Data") (Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E914

สาเหตุ: The relationship between the value set for C46 (micro grinding start diameter) and the value set for C44 (fine grinding start diameter) is incorrect.

การแก้ไข: Check and correct the values set for C44 and C46. (Operation manual 7.4.3 "Editing the Process Data") (Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E91B

สาเหตุ: The relationship between the value set for C42 (No. 2 step grinding start diameter) and the value set for C43 (No. 3 step grinding start diameter) is incorrect.

การแก้ไข: Check and correct the values set for C42 and C43. (Operation manual 7.4.3 "Editing the Process Data") (Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E91C

สาเหตุ: The relationship between the value set for C43 (No. 3 step grinding start diameter) and the value set for C44 (No. 4 step grinding start diameter) is incorrect.

การแก้ไข: Check and correct the values set for C43 and C44. (Operation manual 7.4.3 "Editing the Process Data") ,(Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E91D

สาเหตุ: The relationship between the value set for C44 (No. 4 step grinding start diameter) and the value set for C45 (No. 5 step grinding start diameter) is incorrect.

การแก้ไข: Check and correct the values set for C44 and C45.(Operation manual 7.4.3 "Editing the Process Data") ,(Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E91E

สาเหตุ: The relationship between the value set for C45 (No. 5 step grinding start diameter) and the value set for C46 (No. 6 step grinding start diameter) is incorrect.

การแก้ไข: Check and correct the values set for C45 and C46.ฃ

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E91F

สาเหตุ: The step infeed range data has not been input.

การแก้ไข:Check the step infeed range (C68, C69, C70) data and input the appropriate value.ฃ

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E930

สาเหตุ: The work spindle speed is not set.

การแก้ไข: Check work spindle speed data (C20) and enter a correct value. (Operation manual 7.4.3 "Editing the Process Data") ,(Operation manual 12.6 "Process Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E931

สาเหตุ: The work spindle speed is not set.

การแก้ไข: Check work spindle speed data (C21 or C22) and enter a correct value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E932

สาเหตุ: The work spindle speed is not set.

การแก้ไข: Check work spindle speed data (C22 or C24) and enter a correct value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E940

สาเหตุ:

(a) The next step indexing diameter (B30) data is not input.

(b) The relationship between the input workpiece finish diameter (C40) data and the next step indexing diameter (B30) data is incorrect.

การแก้ไข: Check the next step indexing diameter (B30) data and input an appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E941

สาเหตุ: The workpiece longitudinal datum position memory operation has not been conducted.

การแก้ไข: Execute the workpiece longitudinal datum position memory operation and make sure that the data is input to B32.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E942

สาเหตุ: Input error of the table oscillation stroke (C205 or C206) data

การแก้ไข: Check the table oscillation stroke (C205 or C206) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E950

สาเหตุ:

(a) Input error of the number of dressing steps (B147 or B170) data; set "0" if grinding for dressing is not made.

(b) The dressing speed ratio (B146 or B169) data is not input.

การแก้ไข:

(a) Check the number of dressing steps (B147 or B170) data and input an appropriate value.

(b) Check the dressing speed ratio (B146 or B169) data and input an appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E951

สาเหตุ:

(a) The workpiece rotating speed is too high or the work spindle speed override value is not appropriate.

(b) The automatically determined dividing angle is not appropriate.

(c) The automatically determined work spindle phase error allowable limit value (C249) is too small.

การแก้ไข:

(a) Check the work spindle phase error allowable limit (C249) data, and reduce the work spindle speed or change the override data.

Execute the top-search angle setting. (Operation manual 12.8.3 "Searching Angle")

(c) Input an appropriate value for work spindle phase error allowable limit (C249). (Operation manual 12.8.2 "Automatic Sampling Check Function")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E970

สาเหตุ: The minimum wheel diameter data has not been input.

การแก้ไข:  Check the minimum wheel diameter (B157) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E971

สาเหตุ: The maximum wheel diameter data has not been input.

การแก้ไข:  Check the maximum wheel diameter (A125) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E971

สาเหตุ:The maximum wheel diameter data has not been input.

การแก้ไข:  Check the maximum wheel diameter (A125) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E972

สาเหตุ: The minimum diamond roll diameter data has not been input.

การแก้ไข: Check the minimum diamond roll diameter (A147) data and input the appropriate value. Minimum diamond roll diameter (A147): 91 mm.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E973

สาเหตุ: The maximum diamond roll diameter data has not been input.

การแก้ไข: Check the maximum diamond roll diameter (A152) data and input the appropriate value. Maximum diamond roll diameter (A152): 100.5 mm. (Operation manual 7.4.8 "Inputting the Machine Constant Data") ,(Operation manual 12.4 "Machine Constant Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E974

สาเหตุ: The minimum detection pin length data has not been input.

การแก้ไข:  Check the minimum detection pin length (A174) data and input the appropriate value. Minimum detection pin length (A174): 10 mm.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E975

สาเหตุ: The maximum detection pin length data has not been input.

การแก้ไข: Check the maximum detection pin length (A179) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E976

สาเหตุ: The diamond roll diameter data has not been input.

การแก้ไข:  Perform diamond roll diameter position memory operation and confirm that the diamond roll diameter has been input to A140. (Operation manual 6.5.2 "Diamond Roll Position Memory")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E977

สาเหตุ: The diamond roll width data has not been input.

การแก้ไข: Check the diamond roll width (A141) data and input the appropriate value. (Operation manual 7.4.8 "Inputting the Machine Constant Data") ,(Operation manual 12.4 "Machine Constant Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E978

สาเหตุ: The detection pin length data has not been input.

การแก้ไข:  Perform detection pin length position memory operation and confirm that the detection pin length has been input to A170.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm

สาเหตุ: The memory was initialized or the data has been changed by mistake.

การแก้ไข: Input the data from the backup disk. (Operation manual 7.4.8 "Inputting the Machine Constant Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E97A

สาเหตุ: The memory was initialized or the data has been changed by mistake.

การแก้ไข: Input the data from the backup disk. (Operation manual 7.4.8 "Inputting the Machine Constant Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E97B

สาเหตุ: The memory was initialized or the data has been changed by mistake.

การแก้ไข:  Input the data from the backup disk. (Operation manual 7.4.8 "Inputting the Machine Constant Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E97E

สาเหตุ: The memory was initialized or the data has been changed by mistake.

การแก้ไข:  Input the data from the backup disk.(Operation manual 7.4.8 "Inputting the Machine Constant Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E97F

สาเหตุ: The memory was initialized or the data has been changed by mistake.

การแก้ไข: Input the data from the backup disk. (Operation manual 7.4.8 "Inputting the Machine Constant Data")

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E981

สาเหตุ: The wheel width 1 data has not been input.

การแก้ไข:  Check the wheel width 1 (B173) data and input the appropriate value. When a multi-wheel is used, input the width of the right wheel.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E982

สาเหตุ: The wheel-to-wheel distance data has not been input.

การแก้ไข: Check the wheel-to-wheel distance (B172) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E983

สาเหตุ: The wheel width 2 (B174) data is "0".

การแก้ไข:  Check the wheel width 2 (B174) data and input the appropriate value.

=== END ==

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E991

สาเหตุ: The truing infeed amount data has not been input.

การแก้ไข:  Check the truing infeed amount (A202) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E992

สาเหตุ: For the truing infeed amount data, a large value is input.

การแก้ไข:  Input a value smaller than 0.02 mm for the truing infeed amount (B111 or B112) data.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E993

สาเหตุ: The override data has not been input.

การแก้ไข: Check the override (A208) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E994

สาเหตุ: The rough truing override data has not been input.

การแก้ไข:  Check the rough truing override (B139) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E995

สาเหตุ: The back-off truing override data has not been input.

การแก้ไข:  Check the back-off truing override (B140) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E99A

สาเหตุ: Truing cycle start position setting error

การแก้ไข: Move the table so that the diamond roll comes to a position within the wheel width.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E9B0

สาเหตุ:  The X-axis grinding datum position is within the wheelhead traverse interference area.

การแก้ไข: Check the X-axis grinding datum position (B35) data or the X-axis PSW1 ON position (B400) data and input a value larger than the value set for MI9664.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E9B2

สาเหตุ:  The value set for the work spindle grinding datum position (B40) data is "360".

การแก้ไข: Check the work spindle grinding datum position (B40) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E9B5

สาเหตุ: The footstock spindle advance position data has not been input.

การแก้ไข:  Check the footstock spindle advance position (B53) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E9B6

สาเหตุ: In random workpiece flow production, the footstock spindle grinding origin position data has not been input.

การแก้ไข: Check the footstock spindle grinding origin position (A255) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E9B7

สาเหตุ: The footstock spindle grinding origin position data has not been input.

การแก้ไข:  Check the footstock spindle grinding origin position (B42) data and input the appropriate value.

=== END ==

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E9B8

สาเหตุ: The work spindle advance position data has not been input.

การแก้ไข:  Check the work spindle advance position (B54) data and input the appropriate value.

=== END ===

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E9BA

สาเหตุ: The data of zero position for machining in lateral position of work spindle is not input.

การแก้ไข:  Check the data (A260) of zero position for machining in lateral position of work spindle, input a proper value.

อาการ: Alarm E9B7

=== MACHINE: TOYODA GCS32  ===

อาการ: Alarm E9BB

สาเหตุ: The data of zero position for machining in lateral position of work spindle is not input.

การแก้ไข:  Check the data (B51) of zero position for machining in lateral position of work spindle, input a proper value.

อาการ: Alarm E9B7

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F000

สาเหตุ:

(a) The emergency stop button was pressed or the power was shut off while an axis was moving.

(b) An axis was moved while the power was off (example: the ball screw was rotated with a wrench).

(c) The servomotor was replaced.

(d) The position encoder of the servomotor is faulty.

(e) The CNC memory was initialized.

การแก้ไข:

(a) 1. Press the [CNC RESET] button.

2. Record the data at the upper field (encoder) of INT ABS POSI of the axis with which the alarm took place.

3. Input the recorded value at the axis position setting screen.

4. Shut off the power and then turn it on according to the procedure explained in page 6-1.

5. Turn on MASTER ON and check the coordinate values.

(b) Follow the same operation as explained in (a).

(c) Follow the procedure explained in the servomotor replacement procedure. (6.12 "Replacing the Servomotors")

(d) Replace the servomotor following the servomotor replacement procedure. (6.12 "Replacing the Servomotors")

(e) 1. Record the internal absolute position of all axes before initializing the memory.

2. Press the [CNC RESET] button.

3. Record the data at the upper field (encoder) of INT ABS POSI of the axis with which the alarm took place.

4. Input the recorded value at the axis position setting screen.

5. Shut off the power and then turn it on according to the procedure explained in page 6-1.

6. Turn on MASTER ON and check the coordinate values.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F002

สาเหตุ:

(a) The emergency stop button was pressed or the power was shut off while an axis was moving.

(b) An axis was moved while the power was off (example: the ball screw was rotated with a wrench).

(c) The servomotor was replaced.

(d) The position encoder of the servomotor is faulty.

(e) The CNC memory was initialized.

การแก้ไข:

(a) 1. Press the [CNC RESET] button.

2. Record the data at the upper field (encoder) of INT ABS POSI of the axis with which the alarm took place.

3. Input the recorded value at the axis position setting screen.

4. Shut off the power and then turn it on according to the procedure explained in page 6-1.

5. Turn on MASTER ON and check the coordinate values.

(b) Follow the same operation as explained in (a).

(c) Follow the procedure explained in the servomotor replacement procedure.(6.12 "Replacing the Servomotors")

(d) Replace the servomotor following the servomotor replacement procedure.(6.12 "Replacing the Servomotors")

(e) 1. Record the internal absolute position of all axes before initializing the memory.

2. Press the [CNC RESET] button.

3. Record the data at the upper field (encoder) of INT ABS POSI of the axis with which the alarm took place.

4. Input the recorded value at the axis position setting screen.

5. Shut off the power and then turn it on according to the procedure explained inpage 6-1.

6. Turn on MASTER ON and check the coordinate values.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F00A

สาเหตุ: In inching or pulse handle operation, the selected axis was moved most closely to the interference axis in the plus (+) side.

Actually, the alarm occurs if the selected axis is moved to the position indicated below. In this state, alarm F00B also occurs with the interference axis. (Present position of the plus side interference axis) - (Interference range of the selected axis in the plus side) + (Minus side interference range of the interference axis in the plus side)

การแก้ไข: Move the selected axis in the minus (-) side.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F00B

สาเหตุ:  In inching or pulse handle operation, the selected axis was moved most closely to the interference axis in the minus (-) side.

Actually, the alarm occurs if the selected axis is moved to the position indicated below. In this state, alarm F00A also occurs with the interference axis.  (Present position of the minus side interference axis) - (Interference range of the selected axis in the minus side) + (Plus side interference range of the interference axis in the minus side)

การแก้ไข: Move the selected axis in the plus (+) side.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F094

สาเหตุ:

(a) The command to move an axis beyond the overtravel limit in the negative direction is specified in NC program.

(b) The position data outside the overtravel limit in the negative direction is set in the machining data.

(c) An axis was moved beyond the overtravel limit in the negative direction in jog mode operation.

การแก้ไข:

(a) Move the axis in the positive direction into the axis movable range by using the pulse hand or in jog mode operation, then correct the NC program or machining data.

(b) Move the axis in the positive direction into the axis movable range by using the pulse hand or in jog mode operation, then correct the machining data.

(c) Move the axis in the positive direction into the axis movable range by using the pulse hand or in jog mode operation.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F095

สาเหตุ:

(a) The command to move an axis beyond the overtravel limit in the positive direction is specified in NC program.

(b) The position data outside the overtravel limit in the positive direction is set in the machining data.

(c) An axis was moved beyond the overtravel limit in the positive direction in jog mode operation.

การแก้ไข:

(a) Move the axis in the negative direction into the axis movable range by using the pulse hand or in jog mode operation, then correct the NC program or machining data.

(b) Move the axis in the negative direction into the axis movable range by using the pulse hand or in jog mode operation, then correct the machining data.

(c) Move the axis in the negative direction into the axis movable range by using the pulse hand or in jog mode operation.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F096

สาเหตุ:

(a) The wheel is too small.

(b) The minimum wheel diameter is too large.

การแก้ไข:

(a) Replace the wheel.

(b) Correct the minimum wheel diameter value.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F133

สาเหตุ:  An O number of the NC program that is not stored in the CNC memory was specified.

การแก้ไข:

- Correct the start number in the sequence program.

- Register the NC program corresponding to the start number.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F138

สาเหตุ: An illegal Chinese character code exists in the NC program.

การแก้ไข: Correct the NC program.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F143

สาเหตุ:  A value greater than the maximum value of a variable was going to be set.

การแก้ไข: Change the value so that it is smaller than the maximum value and larger than the minimum value of the variable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F144

สาเหตุ: A value smaller than the minimum value of a variable was going to be set.

การแก้ไข: Change the value so that it is smaller than the maximum value and larger than the minimum value of the variable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F145

สาเหตุ: A wrong variable name was specified.

การแก้ไข: Correct the variable name.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F150

สาเหตุ: An illegal Chinese character code exists in the part number.

การแก้ไข: Correct the part number.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F151

สาเหตุ:  An attempt was made to refer to the C data in the state the number processes is “0” or larger than the maximum process number.

การแก้ไข: Correct the number of processes (“1” to “maximum process number”).

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F152

สาเหตุ: The specified variable number does not correspond to a variable name.

การแก้ไข: Correct the variable number.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F603

สาเหตุ:

(a) Battery has been used to the life or it is faulty.

(b) Battery connection error

การแก้ไข:

(a) Replace the battery.  (6.6.1 "Replacing the CNC Memory Backup Battery")

(b) Tighten the battery terminals.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F609

สาเหตุ: The CNC unit is faulty.

การแก้ไข: Replace the CNC unit.  (6.7 "Replacing the CNC Unit")

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F611

สาเหตุ: Voltage of encoder battery is lower than the allowable value.

การแก้ไข: Replace the battery immediately.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F628

สาเหตุ:

(a) The PLC unit is faulty.

(b) Sequence program error

(c) Jumper setting or parameter setting of the PLC is incorrect.

การแก้ไข:

(a) Replace the PLC unit.

(b) Correct the sequence program.

(c) Change the jumper setting or correct the parameter setting for the PLC unit.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm F400

สาเหตุ: 081-N-3-CNC  CNC FAULT Refer to the description of the alarm displayed in the CNC.

การแก้ไข: Check the alarm displayed by the CNC on the display screen of the content of the HMI alarm. (5.4.1 "Displaying Error Information" (d) Displaying the Error Log)

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G401

สาเหตุ: 081-N-0-CNCOT CNC OVERTRAVEL An over travel has been generated.

การแก้ไข: Checking the axis in which the over-travel has been generated, move the axis to the reverse direction

against the direction of over-travel by the manual pulse generator.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G402

สาเหตุ: 040-H-1-M1PS1  AIR FAULT (Pressure below setting, air leak, filter clogged)

(a) Low air source pressure

(b) Breakage of the piping in the air circuit (air leakage)

(c) Faulty pressure switch

(d) Breakage of pressure switch cable, faulty TOYOPUC input card

การแก้ไข:

(a) Check the air intake valve in the machine, if it is closed.

Open the valve fully and press the [MASTER ON] button.

Carry out cleaning or replacement of the filter element when the air filter is clogging.

If the air filter is free of trouble, the pressure of air source will be low. Check the piping to the machine for leakage.

(b) If the alarm occurs after pressing the [MASTER ON] button, air may leak through the piping/hose.

Check the piping and the hose for loose connection and breakage.

(c) If there are no problems in the check made according to items (a) to (c) above, the pressure switch will be faulty.

Replace the pressure switch. For the pressure switch type, refer to the hydraulic/pneumatic circuit diagram supplied with the machine.

(d) If the alarm still occurs when the [MASTER ON] button after replacing the pressure switch, the cable may be broken or the TOYOPUC input card may be faulty.

Check the wiring referring to the electric circuit diagram.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G406

สาเหตุ: 090-W-0-QC  QUALITY CHECK(Grinding completed by presetting value with the counter)

สาเหตุ: Count-of the quality check counter

การแก้ไข: Check the preset value and the present value of the counter.

If the present value of the quality check counter has reached the preset value (count-up), check the quality of the ground workpiece and reset the counter.

< quality check counter > The counter counts the number of workpieces having been ground after resetting the quality check counter.

- If the count value reaches the value set for B026 (quality check counter preset value), the quality check indicating lamp lights.

< Resetting the quality check counter>

- On the operation panel screen, click the [△] or [▽] button to display the menu group 2/3.

- Click the [COUNTER] button at the screen.

- The counter screen opens.

- Click the CURRENT field of the quality check counter.

- The CURRENT field color changes.

- Click the [RESET] button.

- The current value of the quality check counter is reset to "0".

< Resetting the quality check counter>

- Click the SETUP field of the quality check counter.

- The key-panel window box opens.

- Change the present setting using the ten keys in the key-panel dialog box.

- Example: Changing the Fore. of the quality check

- counter: 100→120

- Click the Fore. field of the quality check counter.

- The key-panel dialog box opens.

- Click the keys as: [1] [2] [0] [Fixation].

- The key-panel dialog box closes and the setting for the Fore. of the quality check counter changes to "120".

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G407

สาเหตุ: 091-T-1-WHEEL  WHEEL CHANGE

(a) The wheel diameter became smaller than the minimum diameter (below the setting level).

(b) Setting error of the minimum wheel diameter

การแก้ไข:

(a) Replace the wheel.

(b) Check the value set for minimum wheel diameter (A122). If a value other than listed below is set, set a correct value for A122.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G408

สาเหตุ: 092-T-1-ROLL  ROLL CHANGE

(a) The diamond roll diameter became smaller than the minimum diameter (ϕ91 mm).

(b) Setting error of the minimum diamond roll diameter (A147).

การแก้ไข:

(a) Replace the diamond roll.

(b) Check the value set for A147. If a value other than "ϕ91.0" is set, change the presently set value to "91.0".

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G409

สาเหตุ: 093-T-1-ROLL  ROLL CHANGE

สาเหตุ:

(a) The contact pin length became smaller than 5 mm.

(b) Setting error of the minimum diamond roll diameter (A147).

การแก้ไข:

(a) Replace the contact pin.

(b) Check the value set for A147. If a value other than "5.0" is set, change the presently set value to "5.0".

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G440

สาเหตุ: 030-P-3-PLC  PLC BATTERY FAULT

(a) Battery life (approximately 5 years)

(b) Mounting or connection of the battery is incorrect.

(c) Failure of PC3J CPU card

การแก้ไข:

(a) Replace the rechargeable lithium battery (Toshiba), type TIP-5426.

(b) Check the connections.

(c) In cases other than (a) and (b), the PC3J-CPU card may be faulty. Replace the PC3J-CPU card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G441

สาเหตุ: 030-N-3-CNC  CNC BATTERY FAULT

(a) Expiration of battery life

(b) Battery contact error

(c) Faulty battery voltage detection circuit

การแก้ไข:

(a) Replace the battery.  (6.6.1. “Replacing the CNC Memory Backup Battery")

(b) Set the battery again correctly.

(c) If the same alarm occurs even after replacing the battery, please contact us.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G450

สาเหตุ: 063-H-0-M1FS1  NO LUB. OIL (Oil is less than preset value.)

(a) Low lubricating oil level.

(b) Setting error of the float switch.

(c) Faulty float switch.

(d) Disconnection of the float switch, or faulty TOYOPUC input card.

การแก้ไข:

(a) Check the amount of lubricating oil by the lubricating oil level gauge and replenish the specified lubricating oil if the oil level is low.

(b) If the float switch is not turned ON although lubricating oil level is normal, check the setting of the lower limit level of the float switch.

(c) If the alarm still occurs when the [MASTER ON] button is pressed although there are no problems in the check made according to items 1. and 2. above, the float switch will be faulty. Replace the float switch.

(d) If the alarm still occurs after replacing the float switch, the cable may be broken or the TOYOPUC input card may be faulty. Check the wiring referring to the electric circuit diagram.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G451

สาเหตุ: 043-H-1-M1PS3 LUB. PRESSURE FAULT

(a) Low lubrication pressure (below the setting level)

(b) Clogged static-pressure slideway lubricating oil filter

(c) Broken lubrication circuit piping

(d) Faulty pressure switch

(e) Setting error of the pressure switch

(f) Disconnection of the pressure switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the lubrication pressure with the pressure gauge. If the pressure is low, adjust the pressure using the pressure adjusting screw at the lubrication pump.

(b) Replace the element of the static-pressure slideway lubricating oil filter.

(c) Check the lubrication circuit piping and replace the piping if broken.

(d) Check that pressure switch is not clogged with dust, chips or other foreign matter. If clogged, clean the pressure switch. If the pressure switch fails to recover the function even after cleaning, replace it with new one.

(e) Check that the pressure switch is set to the level specified on the hydraulic/pneumatic circuit diagram. If the setting differs from the specified level, correct the setting.

(f) Check continuity of the pressure switch cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G452

สาเหตุ: 80-H-3-M1LB1  OIL/AIR LUB. UNIT FAULT

(a) Insufficient lubricating oil.

(b) Faulty lubricating pump of the oil-air lubrication system

(c) Failure or clogging of pressure relief valve or residual pressure breathing valve

(d) Faulty pressure switch or float switch

(e) Breakage or loose connection of cables to the oil-air lubrication system

(f) Faulty controller of the oil-air lubrication system

การแก้ไข:

(a) Check the lubricating oil level and replenish the specified lubricating oil if the oil level is low. Recommended lubricating oil: Mobil Velocite No. 6

If a plastic tank is used, the tank has no oil level gauge. In this case, observe the oil level directly at the tank.

(b) If the lubrication pump motor does not start even if the manual start button is pressed, the pump motor will be faulty (seized, etc.).

Replace the oil-air lubrication unit.

(c) If the lubricating oil pressure is not built up although the pump motor runs in response to the pressing of the manual start button, or if the pressure does not decrease even after the pump stopped, the pressure relief valve or the residual pressure breathing valve may be clogged or faulty. Remove and clean the tank and change the oil. If the same alarm occurs even after cleaning the tank, replace the oil-air lubrication unit.

(d) Although the pump operates when the manual start button is pressed and the pressure is correctly built up and decreases as the pump starts or stops, the pressure indicating lamp does not light or stays lit.

If this occurs, the pressure switch may be faulty.

If the low oil level indicating red lamp lights although the tank holds sufficient amount of lubricating oil, the float switch may be faulty.

Replace the oil-air lubrication unit.

(e) If the alarm still occurs after following the steps (a) to (d) above, check the wiring to the oil-air lubrication unit.

If there are loose connection or breakage of the wire, repair the cable connection.

(f) If the alarm still occurs after following the steps (a) to (e), the controller of the oil-air lubrication unit will be faulty.

Replace the oil-air lubrication unit.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G453

สาเหตุ: 066-H-0-M1LB1  OIL/AIR LUB. UNIT NO OIL. (Oil is less than preset value.)

(a) Low mist oil level of the oil-air lubrication unit (below the lower limit).

(b) Faulty float switch.

(c) Disconnection of the pressure switch cable, or faulty TOYOPUC input card.

การแก้ไข:

(a) Check the mist oil level of the oil-air lubrication unit. If the oil level is low, replenish the mist oil.

(b) If this alarm occurs although the mist oil level is correct, the float switch may be faulty. Repair or replace the float switch.

(c) Check continuity of the float switch cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G454

สาเหตุ: 114-H-1-********  R-WHEELHEAD FEED STATIC PRESSURE SCREW LUB. FILTER CLOGGING

(a) The left wheelhead hydrostatic slide lubricating oil filter is clogged.

(b) Trouble of the detection device in the filter.

(c) Breakage of wiring for the detection device in the filter, fault of corresponded TOYOPUC input device.

การแก้ไข:

(a) Clean or replace the left wheelhead hydrostatic slide lubricating oil filter element.

(b) Replace the left wheelhead hydrostatic slide lubricating oil filter.

(c) Checking continuity of wiring for the detection device in the filter, replace the wiring if there is breakage or the like. Checking the corresponded TOYOPUC input device, replace it if there is any abnormality.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G45D

สาเหตุ: 044-H-1-M1PS14 STATIC PRESSURE SLIDE LUB. FAULT

(a) Low lubricating pressure (below the setting level)

(b) Damage in lubricating circuit piping, or the clearance of the wheelhead static-pressure slideway is widened.

(c) Faulty wheelhead static-pressure slideway lubrication pressure confirmation pressure switch.

(d) Setting error of the wheelhead static-pressure slideway lubrication pressure confirmation pressure switch.

(e) Disconnection of the pressure switch cable, or faulty TOYOPUC input device.

การแก้ไข:

Check the wheelhead hydrostatic slide feed lubricating pressure with the pressure gauge. If it is lower than the level specified on the plate or the hydraulic/pneumatic circuit diagram, check the lubricating pump pressure. If it is lowered, adjust the pressure with the pressure adjusting screw at the lubrication pump. Check the lubricating oil filter for clogging, and if clogged, clean or replace the element. If both the pump and filter do not have any problem, turn the reducing valve and adjust the left wheelhead hydrostatic slide feed lubricating pressure to the specified pressure.

Check the wheelhead static-pressure slideway lubrication circuit for breaks and oil leaks of the piping. If abnormality is found, repair the piping. Check the lift amount of the wheelhead static-pressure slideway. Contact Toyoda if an abnormality is found.

If the error occurs with the normal pressure level, the pressure switch is faulty. Replace the pressure switch.

Check that the pressure switch is set to the level specified on the hydraulic/pneumatic circuit diagram and if not, correct the setting.

Check continuity of the pressure switch cable, and if disconnected, replace the cable. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G45F

สาเหตุ: 113-H-1-******** LUB. OIL FILTER CLOGGING

สาเหตุ:

(a) The hydrostatic slide lubricating oil filter is clogged.

(b) Trouble of the detection device in the filter.

(c) Breakage of wiring for the detection device in the filter, fault of corresponded TOYOPUC input device.

การแก้ไข:

(a) Clean or replace the left wheelhead hydrostatic slide lubricating oil filter element.

(b) Replace the left wheelhead hydrostatic slide lubricating oil filter.

(c) Checking continuity of wiring for the detection device in the filter, replace the wiring if there is breakage or the like. Checking the corresponded TOYOPUC input device, replace it if there is any abnormality.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G460

สาเหตุ: 065-H-0-M1FS3  NO HYD. OIL (Oil is less than preset value.)

สาเหตุ:

(a) Low hydraulic oil level

(b) Setting error of the float switch

(c) Faulty float switch

(d) Disconnection of the float switch, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the amount of hydraulic oil by the hydraulic oil level gauge and replenish the specified hydraulic oil if the oil level is low.

(b) If the float switch is not turned ON although hydraulic oil level is normal, check the setting of the lower limit level of the float switch.

(c) If the alarm still occurs when the [MASTER ON] button (although there are no problems in the check made according to items 1. and 2. above), the float switch will be faulty. Replace the float switch.

(d) If the alarm still occurs after replacing the float switch, the cable may be broken or the TOYOPUC input card may be faulty. Check the wiring referring to the electric circuit diagram.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G46D

สาเหตุ: 047-H-1-M1PS2  MIST PRESSURE FAULT

(a) Low mist lubricating pressure (below the setting level)

(b) Damage in air circuit piping (air leaks)

(c) Faulty pressure switch

(d) Setting error of the pressure switch

(e) Disconnection of the pressure switch cable, or faulty TOYOPUC input device

การแก้ไข:

(a) Check the wheel dresser mist lubrication oil pressure with the pressure gauge. If the oil pressure is low, adjust the pressure using the pressure reducing valve. If the pressure is too low, not allowing the adjustment using the pressure reducing valve, check the air filter for clogging as well as the air source pressure. Take appropriate corrective action if an abnormality is found.

(b) Check the piping of the wheel dresser mist-air circuit and repair the piping when breaks of piping or air hose, or air leak is found.

(c) If the alarm state cannot be cleared although the wheel dresser mist lubrication pressure is correct, or if the pressure switch is ON although the air source is OFF, the pressure switch may be faulty. Replace the pressure switch in this case.

(d) If the alarm state cannot be cleared even after replacing the pressure switch, check the continuity of the pressure switch cable and replace the cable if disconnected. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G470

สาเหตุ: 061-H-0-M1FS4 NO FLUID BEARING OIL (Oil is less than preset value.)

(a) Low wheel spindle bearing oil level (below the lower limit)

(b) Faulty float switch

(c) Setting error of the float switch

(d) Disconnection of the float switch cable, or faulty TOYOPUC input device

การแก้ไข:

(a) Check the amount of wheel spindle bearing oil, and if short, supply lubricating oil.

(b) If the error cannot be reset with the sufficient amount of wheel spindle bearing oil, the float switch is faulty. Repair or replace the float switch.

(c) If the float switch is not set to the required oil level, correct the setting.

(d) Check continuity of the float switch cable, and if disconnected, replace the cable. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G471

สาเหตุ: 041-H-1-M1PS11 FLUID BEARING P. FAULT

(a) Faulty wheel spindle bearing oil pump, clogged wheel spindle bearing oil filter, or pressure setting error

(b) Damage or leaks at the bearing oil circuit piping

(c) Faulty pressure switch

(d) Setting error of the pressure switch (if the switch is of the settable type)

(e) Disconnection of the pressure switch cable, or faulty TOYOPUC input device

การแก้ไข:

(a) Check the wheel spindle bearing pressure with the pressure gauge. If it is lowered, adjust the wheel spindle bearing oil pump pressure with the pressure adjusting screw. If the wheel spindle bearing oil filter is clogged, clean or replace the filter element. If the pressure is not increased, the wheel spindle bearing oil pump is faulty. Replace the pump.

(b) Check the wheel spindle bearing oil circuit piping and parts for damage or leaks and if any, repair or replace the piping.

(c) If the error cannot be reset under the normal wheel spindle bearing pressure, check the piping to the pressure switch for clogging with dust, etc. Check the operation of the pressure switch and if faulty, replace the pressure switch.

(d) Check that the pressure switch is set to the level specified on the hydraulic/pneumatic circuit diagram and if not, correct the setting.

(e) Check continuity of the pressure switch cable, and if disconnected, replace the cable. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G472

สาเหตุ: 111-H-1-******** WHEEL SPINDLE FLUID BEARING OIL FILTER CLOGGING

(a) The wheel spindle bearing oil filter is clogged

(b) The detector in the filter is faulty

(c) The detector in the filter is disconnected or the TOYOPUC input device is faulty

การแก้ไข:

(a) Clean or replace the wheel spindle bearing oil filter element

(b) Replace the wheel spindle bearing oil filter

(c) Check continuity of the detector cable, and if disconnected, replace the cable Check the TOYOPUC input device, and if faulty, replace the device

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G490

สาเหตุ: 180-E-2-CYCLE EACH CYCLE CONDITIONS OFF

(a) The signal of a device that detects unit positions, taken as the manual cycle conditions, is turned OFF

(b) Cable of a device that detects unit positions, taken as the manual cycle conditions, is broken or the related TOYOPUC input card is faulty

การแก้ไข:

(a) Check the conditions necessary for executing the cycle at the electric circuit diagram and check the ON/OFF status of the contact by starting the LdNT

(b) If the condition is turned OFF even if the checked conditions are all satisfied, check the conditions one by one to find which of the conditions is turned OFF by executing the cycle in the single block mode

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G491

สาเหตุ: 180-E-2-AUTO AUTO CYCLE CONDITIONS OFF

- During the execution of the cycles below, some of the conditions are turned OFF

- The signal of a device that detects unit positions, taken as the manual cycle conditions, is turned OFF

- Cable of a device that detects unit positions, taken as the manual cycle conditions, is broken or the related TOYOPUC input card is faulty

- The wheel spindle stopped during automatic operation

- The wheel spindle stopped rotating due to chattering of the wheel spindle bearing lubrication pressure switch, etc

- Breakage of the wheel spindle start magnet switch, or faulty TOYOPUC input card

- Wheel spindle drive motor inverter is faulty (In this case, inverter error is also displayed)

การแก้ไข:

- During the execution of the cycles below, some of the conditions are turned OFF

- Check the conditions necessary for executing the cycle at the electric circuit diagram and check the ON/OFF status of the contact by starting the LdNT

If there are no problems in the check above, Execute the cycle manually to check which of the conditions is turned OFF

- The wheel spindle stopped during automatic operation

- Check the wheel spindle bearing lubrication pressure If the pressure is low, adjust the pressure

- Check the pressure switch Replace the pressure switch if the pressure switch is faulty

- Check the ON/OFF status of the wheel spindle bearing lubrication pressure confirmation circuit and the wheel spindle rotation confirmation circuit If an error occurred with the inverter, check the details of the error

- Check the magnet switch and the TOYOPUC input card and replace them if faulty

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G493

สาเหตุ: 100-E-1-********  EMERGENCY STOP IN GRINDING CYCLE

(a) The [EMERGENCY STOP] button was pressed during grinding

(b) The thermal relay or breaker tripped during grinding

การแก้ไข:

- Unlock the [EMERGENCY STOP] button by turning it clockwise Press the [MASTER ON] button After that press the [RETURN] button Check the workpiece having been ground and restart the cycle if the workpiece is not damaged If the workpiece cannot be ground continuously, change the workpiece to the new one

- Check the thermal relay or breaker that has tripped and check the device protected by the tripped thermal relay or breaker If faulty, repair or replace the faulty device Unlock the [EMERGENCY STOP] button by turning it clockwise Press the [MASTER ON] button After that press the [RETURN] button Check the workpiece having been ground and restart the cycle if the workpiece is not damaged If the workpiece cannot be ground continuously, change the workpiece to the new one

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G496

สาเหตุ: 105-E-2-******** PROGRAM START FAULT (Program activated does not executable) Errors in the electric circuit

การแก้ไข: Please contact Toyoda Machine Works

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4A0

สาเหตุ: 095-E-2-TIME CYCLE TIME OVER

Operation of proximity switches, limit switches or pressure switches is faulty (example: dog is not set correctly)

Cables connected to proximity switches, limit switches, pressure switches, solenoid valves, motor, etc. are broken

Solenoid valve is faulty or motor fails to operate correctly

Cycle is interrupted due to uncontrollable cylinder operation speed due to oil leakage or oil temperature change, faulty actuators, etc.

The TOYOPUC input/output card is faulty

Setting of the velocity data or timer data is changed.

การแก้ไข:

a The alarm occurred due to incomplete cycle: Check the following items by referring to the cycle diagram supplied with the machine

Cycle interrupted operation step (device)

Device involved with the trouble

b The alarm occurred although the cycle finished correctly:

The data (velocity data, timer data) might have been changed or actuator speed has been changed due to oil leakage or oil temperature change

Change the setting for the cycle time over setting timer (internal timer of TOYOPUC)

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4A4

สาเหตุ: 080-I-2-******** DRIVE UNIT INSERT FAULT (PIN)

Due to variations in the workpiece dimensions, the driving fixture (key or pin) is not correctly inserted to the workpiece

The workpiece slips since the brake is not applied to the workpiece

The driving fixture cannot be inserted to the workpiece due to breakage or dimensional error of the driving fixture

Faulty driving fixture shifter

Setting error or failure of the driving fixture insert confirmation device (proximity switch, etc)

Timer-set length of time is too short (standard setting: 10 sec)

การแก้ไข:

Check the dimensions of the workpiece drive section and correct or replace the workpiece if the dimensions differ from those specified in the drawing

Adjust the brake (steady rest) of the workpiece

Check the driving fixture, and if the driving fixture is faulty, repair or replace it

Repair or replace the driving fixture shifter

Adjust the position of the driving fixture insert confirmation device (proximity switch, etc) Replace the driving fixture insert confirmation device (proximity switch, etc)

Correct the setting of the timer to an appropriate value

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4A8

สาเหตุ: 095-I-2-SPINDLE WORK SPINDLE INDEX FAULT (exceeding operational time set)

สาเหตุ: Timer-set length of time is too short

การแก้ไข: Correct the setting of the timer to an appropriate value

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4B0

สาเหตุ: 081-N-3-CNC CNC READY OFF

- The CNC unit is faulty

- The servo amplifier is faulty

- Breakage of the cable connecting the CNC unit and the servo amplifier, faulty PLC, faulty TOYOPUC input card

การแก้ไข:

Check the alarm status if an alarm other than G4B0 occurred simultaneously

At the screen, check the contents and code number of the alarm occurring simultaneously with G4B0 Take corrective action for these alarms

If MC1K is used independently for such as a loader, not as an auxiliary axis of CNC, check the details of the alarm at the MC1K console (MC1K-P) and take corrective action by referring to the MC1K instruction manual

*** NOTE ***

**** G4B0 does not take place independently with an exception of MC1K alarm Check alarms that occurred simultaneously with G4B0 and take corrective action for themG4B0 does not take place independently with an exception of MC1K alarm Check alarms that occurred simultaneously with G4B0 and take corrective action for them***

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4B1

สาเหตุ: 082-N-3-AMP SERVO READY OFF

สาเหตุ:

(a) The servo amplifier is faulty

(b) The servo ready signal cable is broken or its connector is loose

(c) TOYOPUC input card is faulty

การแก้ไข:

Check the servo amplifier for failure An alarm number is displayed in the display unit if a servo amplifier alarm occurs Record the alarm number Remove the cause of the alarm by referring to the indication (5.6 "Servo System Alarms" / 5.6.1 "Alarm of Servo System")

Check the cable of the servo ready signal and its connector

After confirming the servo ready circuit in the electric circuit diagram, start the LdNT and check the ON/OFF status of the servo ready signal

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4B4

สาเหตุ: 150-E-2-SS OVERRIDE SWITCH FAULT

สาเหตุ:

The override switch is faulty, its cable is broken or its connector is loose

การแก้ไข:

Check the override switch, its cable and connector for failure, breakage and loose connection Take appropriate measures according to the results of check

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4B5

สาเหตุ: 150-E-2-SS SELECT SWITCH FAULT

The pulse handle axis selection switch is faulty, its cable is broken or its connector is loose

การแก้ไข:

Check the pulse handle axis selection switch, its cable and connector for failure, breakage and loose connection Take appropriate measures according to the results of check

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4C7

สาเหตุ: 080-C-2-******** CONTACT DETECTION FAULT (for wheel dressing)

- Failure of the contact detection head or broken cable between the contact detection head and the head amplifier

- Failure of the contact detection amplifier

- Short-circuit in the output signal cables of the contact detection amplifier, or faulty TOYOPUC input device

การแก้ไข:

- Replace the contact detection head Replace the cable between the contact detection head and the head amplifier

- Replace the contact detection amplifier

- Check continuity of the contact detection amplifier output cable, and if short-circuited or disconnected, replace the cable Check the TOYOPUC input device, and if faulty, replace the device

- === END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4D0

สาเหตุ: 025-H-3-M1M1 HYD. PUMP OVERLOAD

(a) Faulty hydraulic pump unit

(b) Setting error of the motor breaker

(c) Broken motor breaker

การแก้ไข:

A If the breaker in the motor power line is tripped:

- Check the hydraulic pump unit controller If there is an error with the controller, locate the cause following the instructions stated in the instruction manual of the hydraulic pump unit Measure the current in the cable to the hydraulic pump unit and, if the measured current value exceeds the rating largely, replace the hydraulic pump unit

If the breaker trips although the motor current value is normal, check the breaker setting and adjust it to a correct value

B If the breaker in the motor power line is not tripped:

(c) Check the breaker If faulty, replace the breaker

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4D1

สาเหตุ: 023-H-3-M1M3 LUB.PUMP OVERLOAD

(a) The lubrication pump is faulty.

(b) The motor is damaged or short-circuited.

(c) Setting error of the motor breaker

(d) Damage to the motor breaker

(e) Disconnection of the motor breaker circuit

การแก้ไข:

A. When the motor breaker has tripped:

a. Separate the lubrication pump and run the motor only. If no error occurs, the pump is faulty. Connect the pump and measure the motor current.

(a) If the measurement is abnormally higher than the rated value, check the pump, and if faulty, replace the pump. If the measurement is a little higher than the rated value, check the pressure setting. If the filter is clogged, etc., correct the condition.

b. If an error occurs when the motor runs without load, the motor may be faulty.

(b) Measure the insulation resistance and winding resistance, and if faulty, replace the motor.

Turn the motor by hand, and if heavy, repair the motor.

(c) If the motor breaker trips under normal motor current, check the setting for the motor breaker, and adjust it to the appropriate value.

B. When the motor breaker has not tripped:

(d) Check the motor breaker for damage, and if damaged, replace the motor breaker.

(e) If the motor breaker is not faulty, check continuity of the motor breaker circuit.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4D1

สาเหตุ: 023-H-3-M1M3 LUB.PUMP OVERLOAD

(a) The lubrication pump is faulty.

(b) The motor is damaged or short-circuited.

(c) Setting error of the motor breaker

(d) Damage to the motor breaker

(e) Disconnection of the motor breaker circuit

การแก้ไข:

A. When the motor breaker has tripped: Separate the lubrication pump and run the motor only.

If no error occurs, the pump is faulty. Connect the pump and measure the motor current.

If the measurement is abnormally higher than the rated value, check the pump, and if faulty,replace the pump.

If the measurement is a little higher than the rated value, check the pres-sure setting. If the filter is clogged, etc., correct the condition.

If an error occurs when the motor runs without load, the motor may be faulty.

Measure the insulation resistance and winding resistance, and if faulty, replace the motor. Turn the motor by hand, and if heavy, repair the motor.

If the motor breaker trips under normal motor current, check the setting for the motor breaker,and adjust it to the appropriate value.

B. When the motor breaker has not tripped: Check the motor breaker for damage, and if damaged, replace the motor breaker.

If the motor breaker is not faulty, check continuity of the motor breaker circuit.

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4D2

สาเหตุ: 021-H-3-M1M2 FLUID BEARING PUMP OVERLOAD , If an alarm does not occur when the motor is operated independently (disconnected from the pump):

(a) Faulty pump ,If an alarm occurs even when the motor is operated in almost the no-load state:

(b) Damaged or short-circuited motor

(c) Setting error of the breaker in the motor power line, If the thermal relay has not been tripped:

(d) Damaged breaker in the motor power line

การแก้ไข: Measure the motor current with the motor loaded under actual operation load.

If the measured value is abnormally larger than the rating, check the pump.

If faulty, replace the pump.

When the measured value is larger than the rating only a little, check the pressure setting.

If the filter is clogged or other abnormality is found, replace the filter element.

Measure the insulation resistance and the motor winding resistance.

If the measured value is abnormal, replace the motor.

Turn the motor by hand. If heavy, repair or replace the motor.

If the breaker trips although the motor current is normal, check the breaker setting and adjust it to a correct value.

Check the breaker. If faulty, replace the breaker.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4D3

สาเหตุ: 021-S-3-7M1 WHEEL SPINDLE OVERLOAD

(a) Too large stock of removal, or faulty machining at the prior process

(b) The wheel interferes with the workpiece during the right wheelhead rapid advance due to a faulty mounting posture of the workpiece, etc.

(c) The wheelhead feed coordinate is not correct.

(d) The wheelhead interferes with truing parts or covers during the rapid advance.

(e) Setting error of the current sensor

(f) Faulty current sensor

(g) Disconnection of the current sensor cable

การแก้ไข:

- Measure the workpiece dimensions. If the stock to be removed is too large, change the setting or replace the workpiece.

- Check the workpiece index position and mounting posture, and if faulty, correct the position or posture.

- Align the wheelhead feed axis coordinate.

- Check interference between the wheel and covers, and if any, correct the situation.

- Check that the current sensor is set to the default, and if not, correct the setting.

- Measure no-load current at the wheel spindle motor. If the error occurs though the current sensor setting is higher than the measurement, the current sensor is faulty. Replace the sensor.

- Check continuity of the current sensor cable, and if disconnected, replace the cable.

- Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4D9

สาเหตุ: 020-V-3-M1M5 MIST COLLECTOR OVERLOAD

(a) The mist collector filter is clogged.

(b) The mist collector motor shaft is burnt.

(c) Faulty motor

(d) Setting error of the motor breaker

(e) Damage to the motor breaker

(f) Disconnection of the motor breaker circuit

การแก้ไข:

A. When the motor breaker has tripped:

a. Turn the power off and check the mist collector.

(a) Check that the mist collector motor shaft can be turned by hand.

If not, the motor shaft may be burnt. Replace the mist collector.

(b) Check that the filter is not clogged. If clogged, replace the filter.

b. Measure the motor insulation resistance and winding resistance.

(c) Replace the mist collector if insulation resistance is faulty.

c. Measure the motor current.

(d) If the motor breaker setting is not the same as the default, reset it to the default.

B. When the motor breaker has not tripped:

(e) Check the motor breaker for damage, and if damaged, replace the motor breaker.

(f) If the motor breaker is not faulty, check continuity of the motor breaker circuit.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4E0

สาเหตุ: 028-Q-3-******** COOLANT PUMP OVERLOAD

(a) The coolant pump is faulty.

(b) The motor is damaged or short-circuited.

(c) Motor breaker setting error

(d) Damaged motor breaker

(e) Broken motor breaker circuit

การแก้ไข:

- Overhaul the coolant pump, and check that the suction port is not clogged with chips and that the impeller is not rusted.

If abnormal noise or vibrations are observed during rotation, replace the coolant pump.

- Measure the insulation resistance and winding resistance of the coolant pump motor, and if faulty, replace the coolant pump motor.

- Check that the motor breaker is set to the level specified on the electric circuit diagram, and if not, correct the setting.

If the error occurs though the motor breaker has not tripped, the thermal relay is faulty.

- Replace the motor breaker.

- Check continuity of the motor breaker circuit, and if disconnected, replace the cable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4E2

สาเหตุ: 028-Q-3-******** SUCTION PUMP OVERLOAD

(a) The draw-up coolant pump is faulty.

(b) The motor is damaged or short-circuited.

(c) Setting error of the thermal relay

(d) Faulty thermal relay

(e) Disconnection of the thermal relay circuit

การแก้ไข:

- Overhaul the draw-up coolant pump, and check that the suction port is not clogged with chips and that the impeller is not rusted.

If abnormal noise or vibrations are observed during rotation, replace the draw-up coolant pump.

- Measure the insulation resistance and winding resistance of the draw-up coolant pump motor, and if faulty, replace the draw-up coolant pump motor.

- Check that the thermal relay is set to the level specified on the electric circuit diagram, and if not, correct the setting.

If the error occurs though the thermal relay has not tripped, the thermal relay is faulty.

Replace the thermal relay.

- Check continuity of the thermal relay circuit, and if disconnected, replace the cable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4E3

สาเหตุ: 028-Q-3-T2M5 BED WASHING PUMP OVERLOAD

(a) The bed washing coolant pump is faulty.

(b) The motor is damaged or short-circuited.

(c) Setting error of the thermal relay

(d) Faulty thermal relay

(e) Disconnection of the thermal relay circuit

การแก้ไข:

Overhaul the coolant pump, and check that the suction port is not clogged with chips and that the impeller is not rusted.

If abnormal noise or vibrations are observed during rotation, replace the coolant pump.

Measure the insulation resistance and winding resistance of the coolant pump motor, and if faulty, replace the coolant pump motor.

Check that the thermal relay is set to the level specified on the electric circuit diagram, and if not, correct the setting.

If the error occurs though the thermal relay has not tripped, the thermal relay is faulty. Replace the thermal relay.

Check continuity of the thermal relay circuit, and if disconnected, replace the cable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4E6

สาเหตุ: 028-Q-3-T2M16 SEPARATOR OVERLOAD

(a) Overload magnetic coolant separator

(b) The motor is damaged or short-circuited.

(c) Setting error of the thermal relay

(d) Faulty thermal relay

(e) Disconnection of the thermal relay circuit

การแก้ไข:

a. When the motor breaker has tripped:

Observe the magnetic coolant separator to check if foreign matter is caught or chips have accumulated in it.

Shut off the power supply and clean the magnetic coolant separator.

If overload occurs even when the motor is operated independently, measure the insulation resistance and the motor winding resistance of the motor. Replace the motor if the measured value is abnormal.

Check that the thermal relay is set to the level specified on the electric circuit diagram, and if not, correct the setting.

b. When the motor breaker has not tripped:

If the error occurs though the motor breaker has not tripped, the thermal relay is faulty.

Replace the thermal relay.

Check continuity of the thermal relay circuit, and if disconnected, replace the cable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4E3

สาเหตุ: 028-Q-3-******** CHIP CONVEYOR PUMP OVERLOAD

(a) Overload chip conveyor

(b) The motor is damaged or short-circuited.

(c) Setting error of the thermal relay

(d) Faulty thermal relay

(e) Disconnection of the thermal relay circuit

การแก้ไข:

a. When the thermal relay has tripped:

Observe the chip conveyor to check if foreign matter is caught or chips have accumulated in it.

If the caught foreign matter is released or chips move when the chip conveyor is operated in the reverse direction, repeat normal and reverse direction operation in the inching mode to discharge foreign matter or chips.

If foreign matter or chips cannot be removed in the reverse operation of the chip conveyor, shut off the power supply, discharge coolant and then clean the coolant tank.

If overload occurs even when the motor is operated independently, measure the insulation resistance and the motor winding resistance of the motor.

Replace the motor if the measured value is abnormal.

Check that the thermal relay is set to the level specified on the electric circuit diagram, and if not, correct the setting.

b. When the thermal relay has not tripped:

f the error occurs though the thermal relay has not tripped, the thermal relay is faulty. Replace the thermal relay.

Check continuity of the thermal relay circuit, and if disconnected, replace the cable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4EA

สาเหตุ: 028-Q-3-******** CIRCULATION DIRTY PUMP OVERLOAD

(a) The dirty tank circulation coolant pump is faulty.

(b) The motor is damaged or short-circuited.

(c) Setting error of the thermal relay

(d) Faulty thermal relay

(e) Disconnection of the thermal relay circuit

การแก้ไข:

Overhaul the dirty tank circulation coolant pump, and check that the suction port is not clogged with chips and that the impeller is not rusted.

If abnormal noise or vibrations are observed during rotation, replace the dirty tank circulation coolant pump.

Measure the insulation resistance and winding resistance of the dirty tank circulation coolant pump motor, and if faulty, replace the dirty tank circulation coolant pump motor.

Check that the thermal relay is set to the level specified on the electric circuit diagram, and if not, correct the setting.

If the error occurs though the thermal relay has not tripped, the thermal relay is faulty.

Replace the thermal relay.

Check continuity of the thermal relay circuit, and if disconnected, replace the cable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4EC

สาเหตุ: 028-Q-3-******** CIRCULATION PUMP OVERLOAD

(a) The circulation coolant pump is faulty

(b) The motor is damaged or short-circuited

(c) Setting error of the thermal relay

(d) Faulty thermal relay

(e) Disconnection of the thermal relay circuit

การแก้ไข:

Overhaul the circulation coolant pump, and check that the suction port is not clogged with chips and the impeller is not rusted.

If abnormal noise or vibrations are observed during rotation, replace the dirty tank circulation coolant pump.

Measure the insulation resistance and winding resistance of the circulation coolant pump motor, and if faulty, replace the circulation coolant pump motor.

Check that the thermal relay is set to the level specified on the electric circuit diagram, and if not, correct the setting.

If the error occurs though the thermal relay has not tripped, the thermal relay is faulty. Replace the thermal relay.

Check continuity of the thermal relay circuit, and if disconnected, replace the cable.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4F1

สาเหตุ: 084-H-3-M1OM3  LUB.OIL TEMPERATURE CONT.FAULT

(a) An alarm with the oil temperature controller.

(b) The oil temperature controller is disconnected or the TOYOPUC input device is faulty.

การแก้ไข:

Confirm the details of the alarm by checking the alarm display on the lubricating oil temperature controller.

eliminate the cause, and reset the alarm state by following the instructions stated in the instruction manual of the oil temperature controller.

Example: Trip of compressor motor thermal relay.

Check and replace the compressor motor.

Check continuity of the normal signal cable of the lubricating oil temperature controller, and if disconnected, replace the cable. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4F2

สาเหตุ: 084-Q-3-T2OM1 COOLANT OIL TEMPERATURE CONT.FAULT

The temperature controller has not started.

An alarm occurred with the temperature controller (overload of the motor, etc.).

Disconnection of the normal signal cable from the temperature controller, or faulty TOYOPUC input card.

การแก้ไข:

Check the breaker of the temperature controller. If it is OFF, turn it ON. If the breaker trips frequently, there will be problems such as motor insulation error and overload, take the corrective action by referring to the instruction manual of the coolant temperature controller.

Check the status of the coolant temperature controller by referring to the instruction manual of the coolant temperature controller.

Check continuity of the normal signal cable from the temperature controller, and if disconnected, replace the cable.

Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4F3

สาเหตุ: 084-H-3-M1OM1 BRG.OIL TEMPERATURE CONT.FAULT

- An error occurs to the oil temperature controller.

- The oil temperature controller is disconnected or the TOYOPUC input device is faulty.

การแก้ไข:

Check the contents of the error on the wheel spindle bearing oil temperature controller (check the error indication). Refer to the instruction manual for the oil temperature controller, remove the cause of the problem, and reset the error as described.

Example: A thermal relay at the compressor motor has tripped.

Check the compressor motor, and replace the motor if necessary.

Check continuity of the normal signal cable of the wheel spindle bearing oil temperature controller, and if disconnected, replace the cable.

Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC1D

สาเหตุ: 072-W-2-********PART ROTATING FAULT

- The workpiece does not rotate while the work spindle is rotating due to a faulty drive part (example: drive pin, etc.) for the workpiece. Due to a faulty drive part, the workpiece continues rotating in contact with the wheel though the work spindle has stopped.

- Faulty shape of the workpiece (example: faulty drive pin hole on the workpiece).

- The work spindle speed is not consistent with the workpiece rotation speed detector setting.

- The workpiece rotation speed detector (example: proximity switch) remains on or off due to a failure.

- The workpiece rotation speed detector is disconnected or the TOYOPUC input device is faulty.

การแก้ไข:

  - Check the workpiece drive unit for damage or wearout of drive parts. If damaged or worn out, replace the part. Check the operation of the workpiece drive unit, the workpiece holding clearance, etc. If there is any failure in operation, repair or replace the workpiece drive unit.

  - Check the workpiece shape (workpiece holding diameter, drive pin hole diameter, etc.), and if faulty, replace the workpiece.

  - When the work spindle speed is altered, adjust the setting of the workpiece rotation speed detector (example: motion detector, etc.) correctly to the work spindle speed.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC20

สาเหตุ: 073-J-0-******** TAILSTOCK CENTER OVERTRAVEL (Defective part shape and clamping state)

The overtravel confirmation switch (es) are activated due to a faulty workpiece center hole, a drop or the absence of workpiece, etc.

This switch is not turned on in the normal condition. When overtravel occurs, it is turned on.

These switches are not turned on at the same time in the normal condition. When overtravel occurs, they are turned on at the same time.

In the case of 3: Center moved beyond the range set by the PSW.

Faulty overtravel confirmation switch (es), or setting error of overtravel confirmation switch (es).

Disconnection of the overtravel confirmation switch cable, or faulty TOYOPUC input device.

การแก้ไข:

When the overtravel confirmation switch (es) are activated: Place the [OPERATION] selection switch to [MANU.], retract the tailstock center and check fault of the center hole or fault of workpiece position.

Set the overtravel confirmation switch (es) properly (example: adjust the dog position). If the error cannot be reset, replace the overtravel confirmation switch (es).

Check if the setting of PSW is done.

(Operation 7.6.2 “Setting the Tailstock Spindle Position”)

Check continuity of the overtravel confirmation switch cable, and if disconnected, replace the cable.

Check the TOYOPUC input device (example: input card), and if faulty, replace the device.

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC21

สาเหตุ: 073-J-1-******** TAILSTOCK CENTER UNDERTRAVEL

สาเหตุ:

Faulty workpiece posture or center hole

Faulty setup change of the tailstock center

Setting error of the tailstock center most advanced position confirmation switch

Faulty tailstock center most advanced position confirmation switch

Disconnection of the tailstock center most advanced position confirmation switch cable, or faulty TOYOPUC input device

การแก้ไข:

When the tailstock center has not moved to the most advanced position:

Check the workpiece posture and center hole. If faulty, correct the posture or replace the workpiece.

Check that the tailstock center is compatible with the workpiece. If not, perform a setup change of the tailstock center or replace the workpiece.

When the tailstock center has moved to the most advanced position:

If the workpiece is in good condition, set the tailstock center most advanced position confirmation switch properly (example: adjust the dog in the most advanced position). If the PSW is used, adjust the tailstock spindle position.

(Operation manual 7.6.2 “Setting the Tailstock Spindle Position”)

Replace the tailstock center most advanced position confirmation switch. If the PSW is used, check the center pressing confirmation proximity switch.

(Operation manual 7.6.2 “Setting the Tailstock Spindle Position”)

Check continuity of the tailstock center most advanced position confirmation switch cable, and if disconnected, replace the cable. Check the TOYOPUC input device, and if faulty, replace the device. If the PSW is used, check the PSW device for the occurrence of abnormalities. If there is an abnormality, check the PSW device head.

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm G4E3

สาเหตุ: 105-S-2-******** WORK SPINDLE STOPPED IN GRINDING CYCLE , Since the conditions necessary for work spindle rotation are not satisfied, work spindle rotation cannot be continued.

Example: There is a defect in the workpiece center hole and the center advance end confirmation proximity switch turned OFF as the work spindle rotated.

การแก้ไข:

- Display the work spindle rotation condition circuit of device code 4 by the LdNT and find the condition that is OFF.

- Check the device related with the found condition and if there is an abnormality, take the appropriate action.

Example: There is chipping in the workpiece center hole. The workpiece is replaced.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC36

สาเหตุ: 085-S-2-******** RAPID FEED OVERLOAD

(a) Too large stock of removal, or faulty machining at the prior process

(b) The wheel interferes with the workpiece during the right wheelhead rapid advance due to a faulty mounting posture of the workpiece, etc.

(c) The wheelhead feed coordinate is not correct.

(d) The wheelhead interferes with truing parts or covers during the rapid advance.

(e) Setting error of the current sensor

(f) Faulty current sensor

(g) Disconnection of the current sensor cable, or faulty TOYOPUC input device

การแก้ไข:

(a) Measure the workpiece dimensions. If the stock to be removed is too large, change the setting or replace the workpiece.

(b) Check the workpiece index position and mounting posture, and if faulty, correct the position or posture.

(c) Align the wheelhead feed axis coordinate.

(d) Check interference between the wheel and covers, and if any, correct the situation.

(e) Check that the current sensor is set to the default, and if not, correct the setting.

(f) Measure no-load current at the wheel spindle motor. If the error occurs though the current sensor setting is higher than the measurement, the current sensor is faulty. Replace the sensor.

(g) Check continuity of the current sensor cable, and if disconnected, replace the cable. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC37

สาเหตุ: 085-S-2-******** GRINDING FEED OVERLOAD

1. Too large stock of removal, or faulty machining at the prior process

2. The wheel interferes with the workpiece during the right wheelhead rapid advance due to a faulty mounting posture of the workpiece, etc.

3. The wheelhead feed coordinate is not correct.

4. Grinding feedrate is too fast.

5. Setting error of the current sensor

6. Faulty current sensor

7. Disconnection of the current sensor cable, or faulty TOYOPUC input device

การแก้ไข:

a Measure the workpiece dimensions. If the stock to be removed is too large, change the setting or replace the workpiece.

b Check the workpiece index position and mounting posture, and if faulty, correct the position or posture.

c Align the wheelhead feed axis coordinate.

d Set the grinding feedrate at an appropriate value.

e Check that the current sensor is set to the default, and if not, correct the setting.

f Measure no-load current at the wheel spindle motor. If the error occurs though the current sensor setting is higher than the measurement, the current sensor is faulty. Replace the sensor.

g Check continuity of the current sensor cable, and if disconnected, replace the cable. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC60

สาเหตุ: 083-T-3-9INV1 ROLL INVERTER FAULT

(a) An alarm occurred with the diamond roll inverter.

(b) The regenerative resistor of the diamond roll is overheated.

(c) Disconnection of normal signal cable or the frequency arrived signal cable from the diamond roll inverter, or faulty TOYOPUC input device

การแก้ไข:

(a) Check alarm indication on the diamond roll inverter unit. Refer to the instruction manual for the diamond roll inverter unit, check the contents of the alarm displayed on the Digital Operator, and remove the cause of the problem.

(b) Check the operation if diamond roll motor start/stop has been repeated frequently in a short time. When the alarm occurs although the diamond roll is not operated as described above, check the motor power cable for loose connection, discontinuity and wrong connections. If there is any abnormality, make corrections.

(c) Check continuity of the normal signal cable and the frequency arrived signal cable from the diamond roll inverter unit, and if disconnected, replace the cable. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC61

สาเหตุ: 083-T-3-9INV1

(a) Failure of the diamond roll motor shaft

(b) Faulty diamond roll inverter

(c) Disconnection of the speed arrived signal cable of the diamond roll inverter, or faulty TOYOPUC input device

การแก้ไข:

(a) Measure the diamond roll motor shaft load current to judge if the diamond roll motor shaft fails to rotate smoothly. Repair or replace the diamond roll motor if the motor shaft is faulty.

(b) Check the diamond roll motor inverter for the occurrence of abnormalities. If the inverter is faulty, eliminate the cause by following the instructions stated in the instruction manual.

(c) Check continuity of the speed arrived signal cable of the diamond roll inverter, and if disconnected, replace the cable. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC62

สาเหตุ: 120-T-2-********

(a) The diamond roll regeneration resistor is overheated.

(b) Short-circuit of the diamond roll regeneration resistor overheat signal cable, or faulty TOYOPUC input device

การแก้ไข:

(a) Check if the diamond roll motor is started and stopped frequently at short intervals. If not, check the motor power cable for imperfect contact, disconnection or miss-connection. If any, correct the wiring.

(b) Check continuity of the diamond roll regeneration overheat signal cable, and if faulty, correct the wiring. Check the TOYOPUC input device, and if faulty, replace the device.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC64

สาเหตุ: 047-T-1-********

(a) Low oil mist lubricating pressure (below the setting level)

(b) Damage in air circuit piping (air leaks)

(c) Faulty pressure switch

(d) Setting error of the pressure switch

(e) Disconnection of the pressure switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the oil-mist pressure by reading the pressure gauge of the air pressure reducing valve to confirm if the pressure meets the set value specified in the hydraulic/pneumatic circuit diagram. If the pressure is lower than the specified value, adjust the pressure using the air pressure reducing valve. If the source air pressure is low due to clogged air filter, replace the air filter element.

(b) Check piping and hoses in the air circuit, and repair if there is air leakage.

(c) Replace the pressure switch if an abnormality is generated though the oil-mist pressure is normal on the pressure gauge.

(d) Check if the setting of the pressure switch is identified with the setting value in the hydraulic/pneumatic circuit diagram if the pressure switch is a adjustable type.

(e) Check continuity of the pressure switch cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC91

สาเหตุ: 048-Q-1-********

(a) Low wheelhead coolant pressure (terminal pressure) (below the setting level)

(b) Damage in wheelhead coolant circuit piping (coolant leaks)

(c) Faulty pressure switch

(d) Setting error of the pressure switch

(e) Disconnection of the pressure switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the coolant pump motor rotation direction with the arrow plate. If the rotation direction is reversed, correct the wiring. Overhaul the pump, and if damaged, replace the pump.

(b) Check that the indicator lamp at the coolant ON solenoid valve lights. If not, refer to the electric circuit diagram, start the LdNT and check that the signal is correctly output to the solenoid valve. If the solenoid valve indicator lamp does not light though the signal is output, check continuity of the solenoid valve cable, and if disconnected, correct the wiring. If the solenoid valve cable is correctly connected but the indicator lamp does not light, replace the solenoid valve. If coolant does not flow by turning on the valve manually, replace the solenoid valve.

(c) If the error occurs with normal indication on the pressure gauge, overhaul the piping to the coolant pressure switch, and check that it is not clogged with dust or chips. If clogged, clean the piping. If the error persists after cleaning, replace the pressure switch.

(d) Check that the pressure switch is set to the level specified on the hydraulic/pneumatic circuit diagram, and if not, correct the setting.

(e) Check continuity of the pressure switch cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC92

สาเหตุ: 068-Q-0-********

(a) Low clean tank coolant level (below the lower limit)

(b) Faulty float switch

(c) Disconnection of the float switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the amount of coolant, and if short, supply coolant.

(b) Check that the float switch is not damaged and that the float switch can be turned on manually. If faulty, replace the float switch.

(c) Check continuity of the float switch cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC93

สาเหตุ: 068-Q-0-********

(a) Low clean tank coolant level (below the lower limit)

(b) Faulty float switch

(c) Disconnection of the float switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the amount of coolant, and if short, supply coolant.

(b) Check that the float switch is not damaged and that the float switch can be turned on manually. If faulty, replace the float switch.

(c) Check continuity of the float switch cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC94

สาเหตุ: 068-Q-0-********

(a) Low dirty tank coolant level (below the lower limit)

(b) Faulty float switch

(c) Disconnection of the float switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the amount of coolant, and if short, supply coolant.

(b) Check that the float switch is not damaged and that the float switch can be turned on manually. If faulty, replace the float switch.

(c) Check continuity of the float switch cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC95

สาเหตุ: 068-Q-0-********

สาเหตุ:

(a) Low coolant level (below the lower limit)

(b) Faulty float switch

(c) Disconnection of the float switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the amount of coolant, and if short, supply coolant.

(b) There is a possibility of the trouble of the float switch when abnormality occurs in the amount of coolant liquid for grinding though it is normal. Check the float switch. And adjust, repair or replace if there is some fault.

(c) Check continuity of the float switch cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC96

สาเหตุ: 068-Q-1-********

(a) The pump drawing the coolant from the dirty tank to the clean tank is faulty, or the coolant tank holds too much amount of coolant.

(b) Faulty float switch

(c) Setting error of the float switch

(d) Disconnection of the float switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the coolant amount in the clean tank. If the amount is small, check if the pump that feeds coolant from the dirty tank to the clean tank is faulty. If faulty, repair or replace the pump. If a magnetic coolant separator or other equipment is provided in the circuit between the dirty tank and the clean tank, check the equipment to see if it is clogged causing overflow of coolant. If coolant amount in the clean tank is normal with coolant amount in the dirty tank too much, total amount of coolant will be excessively large. Reduce the coolant amount to the correct amount.

(b) If the alarm occurs although coolant amount is correct in both the dirty tank and the clean tank, the upper limit detecting device of the float switch will be faulty. In this case, check the upper limit detecting device (example: proximity switch) and replace it if faulty.

(c) When the detecting device is normal, check the float switch setting if it matches the coolant level. Correct the setting if the setting is wrong.

(d) Check continuity of the upper limit detecting device cable of the float switch, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC98

สาเหตุ: 050-Q-1-********

(a) When coolant flow amount is set for small-amount, the flow amount is lower than the set lower limit amount (flow amount is too small).

(b) Setting error of the small-amount lower limit detection flow meter

(c) Faulty flow meter

(d) Disconnection of the flow meter cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Increase the coolant flowrate using the flow control valve. Refer to the flow meter, and adjust it to the appropriate level.

(b) Set the flow meter properly. (6.9.1 "Replacing the Flow Meter")

(c) Replace the flow meter. (6.9.1 "Replacing the Flow Meter")

(d) Check continuity of the flow meter detector cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC99

สาเหตุ: 050-Q-1-********

(a) When coolant flow amount is set for small-amount, the flow amount is higher than the set upper limit amount (flow amount is too large).

(b) Setting error of the small-amount upper limit detection flow meter

(c) Faulty flow meter

(d) Disconnection of the flow meter cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Decrease the coolant flowrate using the flow control valve. Refer to the flow meter, and adjust it to the appropriate level.

(b) Set the flow meter properly.

(c) Replace the flow meter.

(d) Check continuity of the flow meter detector cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC9A

สาเหตุ: 050-Q-1-********

(a) When coolant flow amount is set for large-amount, the flow amount is lower than the set lower limit amount (flow amount is too small).

(b) Setting error of the large-amount lower limit detection flow meter

(c) Faulty flow meter

(d) Disconnection of the flow meter cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Increase the coolant flowrate using the flow control valve. Refer to the flow meter, and adjust it to the appropriate level.

(b) Set the flow meter properly.

(c) Replace the flow meter.

(d) Check continuity of the flow meter detector cable, and if disconnected, replace the cable. Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC9E

สาเหตุ: 028-Q-0-********

(a) The chip conveyor is overloaded.

(b) The torque limiter is not adjusted properly or malfunctioning.

(c) Mounting or setting error of the limit switch

(d) Damage to the limit switch

(e) Disconnection of the limit switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Check the chip conveyor for any blockage or chips. If blockage is removed or chips are moved when rotating the chip conveyor in the reverse direction, repeat normal/reverse rotations in inching mode until blocks or chips are discharged. If not, turn the power off, drain the coolant, and clean the coolant tank.

(b) Adjust the torque limiter. (Refer to the chip conveyor instruction manual.)

(c) Correct the limit switch setting of the torque limiter.  (Refer to the chip conveyor instruction manual.)

(d) Turn the limit switch on and off manually, and check the activation. If not activated properly, replace the limit switch.

(e) Check continuity of the limit switch cable, and if disconnected, replace the cable.

Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA GCS32 ===

อาการ: Alarm GC9F

สาเหตุ: 150-Q-0-********

(a) The run mode is not selected on the chip conveyor operation panel.

(b) Faulty selector switch

(c) Disconnection of the selector switch cable, or faulty TOYOPUC input card

การแก้ไข:

(a) Select the run mode on the chip conveyor operation panel.

(b) Replace the selector switch on the chip conveyor operation panel.

(c) Check continuity of the selector switch cable for the chip conveyor operation panel, and if disconnected, replace the cable.

Check the TOYOPUC input card, and if faulty, replace the card.

=== END ===

=== MACHINE: TOYODA ===

อาการ: Magnet ไม่หมุน

สาเหตุ:  Magnet NG

การแก้ไข:  เปลี่ยน Magnet ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: ประตูงัดกับ Cylinder

สาเหตุ:  ตำแหน่งไม่ตรง

การแก้ไข:  เจาะรูยึดใหม่ + ปรับตำแหน่ง

=== END ===

=== MACHINE: TOYODA ===

อาการ: Bearing PS Fault

สาเหตุ:  Oil Filter อุดตัน

การแก้ไข:  เปลี่ยน Oil Filter

=== END ===

=== MACHINE: TOYODA ===

อาการ: Tailstock Center Alarm

สาเหตุ:  Solenoid Valve Tailstock NG

การแก้ไข:  ทำความสะอาด + เปลี่ยนใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Magnet เสีย / น้ำล้น

สาเหตุ:  ชุด Gear มีปัญหา

การแก้ไข:  เปลี่ยนชุด Gear head ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: น้ำมันไฮดรอลิกรั่ว

สาเหตุ:  Pump เสีย

การแก้ไข:  เปลี่ยน Pump

=== END ===

=== MACHINE: TOYODA ===

อาการ: หินดับเปิดไม่ติด

สาเหตุ:  Motor Coolant Overload

การแก้ไข:  เปลี่ยน Motor Coolant

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Shutter

สาเหตุ:  Limit Switch ค้าง

การแก้ไข:  ทำความสะอาด Limit Switch

=== END ===

=== MACHINE: TOYODA ===

อาการ: Coolant Flow Fault (Flow ต่ำ)

สาเหตุ:  ท่อน้ำสกปรก / Flow ต่ำ

การแก้ไข:  ทำความสะอาดท่อน้ำ

=== END ===

=== MACHINE: TOYODA ===

อาการ: Shutter Alarm

สาเหตุ:  Proximity Center Advance เสีย

การแก้ไข:  เปลี่ยน Sensor

=== END ===

=== MACHINE: TOYODA ===

อาการ: Table ไม่ขยับ

สาเหตุ:  Motor C สายไฟขาด

การแก้ไข:  บัดกรีสาย + เปลี่ยน Oil Cooling

=== END ===

=== MACHINE: TOYODA ===

อาการ: น้ำมันรั่ว

สาเหตุ:  Pump เสีย

การแก้ไข:  เปลี่ยน Pump

=== END ===

=== MACHINE: TOYODA ===

อาการ: Tailstock ไม่ถอย

สาเหตุ:  Solenoid / Limit SW ค้าง

การแก้ไข:  ทำ Manual และแก้ไข Valve

=== END ===

=== MACHINE: TOYODA ===

อาการ: น้ำหน้าหินไม่ไหล

สาเหตุ:  Terminal 25 เสีย

การแก้ไข:  เปลี่ยน Terminal

=== END ===

=== MACHINE: TOYODA ===

อาการ: Fluid Bearing PS Fault

สาเหตุ:  Filter NG

การแก้ไข:  เปลี่ยน Filter

=== END ===

=== MACHINE: TOYODA ===

อาการ: Oil Skimmer Overload

สาเหตุ:  ชุด Skimmer สกปรก

การแก้ไข:  ถอดซ่อม

=== END ===

=== MACHINE: TOYODA ===

อาการ: Tailstock Under travel

สาเหตุ:  สาย Prox ขาด

การแก้ไข:  เปลี่ยน Proximity

=== END ===

=== MACHINE: TOYODA ===

อาการ: สายพาน Step Oil miss

สาเหตุ:  สายพานสกปรก / คลาดตำแหน่ง

การแก้ไข:  ทำความสะอาด + ตั้งสายพาน

=== END ===

=== MACHINE: TOYODA ===

อาการ: Pressure Switch รั่ว

สาเหตุ:  Pressure Switch เสีย

การแก้ไข:  เปลี่ยน Pressure Switch

=== END ===

=== MACHINE: TOYODA ===

อาการ: หน้าจอไม่ตรง

สาเหตุ:  หน้าจอเสีย

การแก้ไข:  เปลี่ยนจอใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Motor Overload C

สาเหตุ:  Amp เสีย

การแก้ไข:  เปลี่ยนสายไฟ Motor

=== END ===

=== MACHINE: TOYODA ===

อาการ: Master on ไม่ได้

สาเหตุ:  สายไฟแกน C ขาด

การแก้ไข:  บัดกรีสายใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Position Deviation

สาเหตุ:  ค่าแกนเพี้ยน

การแก้ไข:  ปรับตั้งแกนใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: หินดับเอง

สาเหตุ:  Connector หลวม

การแก้ไข:  บัดกรี/เทปละลาย

=== END ===

=== MACHINE: TOYODA ===

อาการ: PLC Com error

สาเหตุ:  Connector หลวม

การแก้ไข:  ขยับและ Reset

=== END ===

=== MACHINE: TOYODA ===

อาการ: เครื่องไม่ทำงาน

สาเหตุ:  Prox spindle ขาด

การแก้ไข:  เปลี่ยน Prox + Set ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Locator end NG

สาเหตุ:  Prox เสีย

การแก้ไข:  เปลี่ยน Prox

=== END ===

=== MACHINE: TOYODA ===

อาการ: A447

สาเหตุ:  ไม่ทราบสาเหตุ

การแก้ไข:  Reset Master on

=== END ===

=== MACHINE: TOYODA ===

อาการ: Magnet เสียงดัง / น้ำรั่ว

สาเหตุ:  ชุด Magnet หลวม / น้ำเข้า

การแก้ไข:  ปิดแผ่นอะคริลิคชั่วคราว

=== END ===

=== MACHINE: TOYODA ===

อาการ: Reed SW ไม่ทำงาน

สาเหตุ:  สายขาด

การแก้ไข:  ต่อสาย

=== END ===

=== MACHINE: TOYODA ===

อาการ: Servo Alarm 80, 8E

สาเหตุ:  Amp เสีย

การแก้ไข:  เปลี่ยน Servo Amp

=== END ===

=== MACHINE: TOYODA ===

อาการ: Main Pressure Drop

สาเหตุ:  Vane Pump เสีย

การแก้ไข:  เปลี่ยน Pump

=== END ===

=== MACHINE: TOYODA ===

อาการ: Main Pressure Drop

สาเหตุ: Vane Pump NG

การแก้ไข: เปลี่ยน Vane Pump ตัวใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Sequence circuit

สาเหตุ: สายไฟปุ่ม Return ขาด

การแก้ไข: ตัดต่อสายไฟใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Wheel Head Table Lub PS Fault

สาเหตุ: Pump Hydraulic 68 ทำ Pressure ไม่ได้

การแก้ไข: ปรับแรงดันที่ Pump Hydraulic 68

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Servo Amp / กด Master ON ไม่ติด

สาเหตุ: Servo Amp แกน Z NG

การแก้ไข: เปลี่ยน Servo Amp, Set Parameter, Reset Servo, Master ON

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Auto Sizer

สาเหตุ: Auto Sizer NG

การแก้ไข: ปรับลมใหม่, เปลี่ยน Auto Sizer, Set D1+D2 ใหม่ ทดลอง OK

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Position Deviation

สาเหตุ: ค่าแกน X, Z, C ไม่ตรง

การแก้ไข: Reset Servo และตั้งค่าแกนใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Flow Much / Flow Low limit

สาเหตุ: Flow น้ำไม่ได้

การแก้ไข: ปรับ Flow sensor และปรับ Valve Coolant Flow

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Link Fault

สาเหตุ: D-Link Terminal SV2 หมดอายุการใช้งาน

การแก้ไข: เปลี่ยน D-Link Terminal ใหม่ เปิดเครื่อง Master ON ok

=== END ===

=== MACHINE: TOYODA ===

อาการ: ปุ่ม Start ชำรุด

สาเหตุ: ปุ่ม Start กดไม่ได้

การแก้ไข: เปลี่ยนปุ่ม Start ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Servo Drive Amplifier

สาเหตุ: Amp Spindle L เสีย

การแก้ไข: เปลี่ยน Servo Amp Spindle L และ Amp P3se

=== END ===

=== MACHINE: TOYODA ===

อาการ: Before truing, contact detect approach count over. Store pin position correctly

สาเหตุ: ขณะ Dressing wheel สัญญาณ AE Sensor ไม่ออน

การแก้ไข:เปลี่ยน AE Sensor ที่ Pin NG

ตรวจ AE Sensor ของ Wheel พบว่าไม่ออน (ไม่มีของเปลี่ยน)

ยืม AE Sensor rotor จาก OP5 ทดสอบ ออนปกติ

คืนของ OP5 และสั่งซื้อของด่วน Marposs

วันที่ 10 เม.ย. 2025 เปลี่ยน AE Sensor rotor ใหม่และทำการ Setup เครื่อง

=== END ===

=== MACHINE: TOYODA ===

อาการ: Servo alarm (Servo drive amplifier alarm 88)

สาเหตุ:  Servo amplifier NG, Encoder ROD NG

การแก้ไข:  เปลี่ยน Servo amplifier แกน C, ตั้งค่า Sarcos parameter, เปลี่ยน Heidenheim ตัวใหม่, ตั้งแกน C, ทดลองผลิตงาน OK

=== END ===

=== MACHINE: TOYODA ===

อาการ: A430 Servo alarm (Sarcos optical cable disconnect)

สาเหตุ:  Servo amplifier X Axis NG

การแก้ไข:  เปลี่ยน Servo amplifier X Axis, Store parameter to amplifier X axis, ทดลองเดินงาน OK

=== END ===

=== MACHINE: TOYODA ===

อาการ: สายพานของหินมีเสียงดัง

สาเหตุ:  สายพานหย่อน

การแก้ไข:  ปรับตั้งสายพานให้ตรงมาตรฐาน

=== END ===

=== MACHINE: TOYODA ===

อาการ: C45F Wheel head table Lub.PS Fault

สาเหตุ:  Filter อุดตัน

การแก้ไข:  ปรับ Pressure ขึ้นเล็กน้อย, ทำความสะอาด Oil filter

=== END ===

=== MACHINE: TOYODA ===

อาการ: Proximity switch จับสถานะการหมุนชิ้นงานไม่ได้

สาเหตุ:  Proximity sensor เสีย

การแก้ไข:  เปลี่ยน Proximity sensor, ทดลองเดินงาน OK

=== END ===

=== MACHINE: TOYODA ===

อาการ: หน้าจอ Touch Screen กดไม่ติด

สาเหตุ:  Touch Screen เสีย

การแก้ไข:  เปลี่ยนหน้าจอ Touch Screen ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Wheel Inverter fault

สาเหตุ:  Wheel head fault ติดขัด

การแก้ไข:  เปลี่ยน Filter, เติมน้ำมัน Wheel head

=== END ===

=== MACHINE: TOYODA ===

อาการ: B91A Diamond roll contact detection signal

สาเหตุ:  AE sensor ไม่ on ขณะกรีดหิน

การแก้ไข:  เปลี่ยน AE sensor, Set หินกับ Diamond ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Air Fault Pressure Switch Air สกปรก

สาเหตุ:  Pressure Switch Air สกปรก

การแก้ไข:  ทำความสะอาด Pressure Switch Air

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Air Fault Pressure Switch Air NG

สาเหตุ:  Pressure Switch Air NG

การแก้ไข:  เปลี่ยน Pressure Switch Air ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: หินดับ Alarm Fluid bearing PS fault

สาเหตุ:  Filter อุดตัน

การแก้ไข:  เปลี่ยน Filter ตัวใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Servo / Encoder Error

สาเหตุ:  Encoder Servo NG

การแก้ไข:  เปลี่ยน Encoder ของ Servo Motor แกน C

=== END ===

=== MACHINE: TOYODA ===

อาการ: กระบอกลมรั่ว / ประตูถอยออกไม่สุด

สาเหตุ:  Seal กระบอกลม NG

การแก้ไข:  เปลี่ยน Seal กระบอกลมและทดสอบการทำงาน

=== END ===

=== MACHINE: TOYODA ===

อาการ: กด Master ON ติด ปล่อยมือไฟดับ

สาเหตุ:  Magnetic Contactor NG

การแก้ไข:  เปลี่ยน Magnetic Contactor ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: สายน้ำมัน Hydraulic แตก

สาเหตุ:  สายน้ำมัน Hydraulic เสื่อมสภาพ

การแก้ไข:  เปลี่ยนสายน้ำมัน Hydraulic ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Machine ไม่สามารถ Master ON ได้

สาเหตุ:  Contact ช่วยทำงานค้าง

การแก้ไข:  เปลี่ยน Contact ช่วยตัวใหม่ และทดสอบ Master ON

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Work Spindle Error

สาเหตุ:  ไม่ทราบสาเหตุที่แน่ชัด

การแก้ไข:  Reset เครื่องและทดสอบหมุน Work Spindle

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm Wheel Table Lub PS Fault

สาเหตุ:  Filter Lubrication อุดตัน และ Pump ไม่ทำงาน

การแก้ไข:  เปลี่ยน Line Filter และ Pump Hydraulic สำหรับ LUB Wheel

=== END ===

=== MACHINE: TOYODA ===

อาการ: Flow CHC ต่ำกว่าค่ากำหนด

สาเหตุ:  Flow Sensor Contact Out1/Out2 ไม่จ่ายไฟ

การแก้ไข:  เปลี่ยน Flow Sensor ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm A586 External device does not open

สาเหตุ:  CPU เสีย

การแก้ไข:  เรียก Maker ยืนยันอาการ เปลี่ยน CPU ลง Data CNC และ Program ใหม่ ทดสอบ OK

=== END ===

=== MACHINE: TOYODA ===

อาการ: ประตูเปิด–ปิดกระแทกแรง

สาเหตุ:  Cylinder ประตูรั่ว และปรับ Speed ไม่ได้

การแก้ไข:  เปลี่ยน Cylinder ประตูใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Motor Hydraulic Overload Master ON ไม่ได้

สาเหตุ:  Breaker Motor เสีย

การแก้ไข:  เปลี่ยน Breaker ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Servo Alarm เปิดเครื่องแล้ว Alarm

สาเหตุ:  Servo Amplifier X Axis NG

การแก้ไข:  เปลี่ยน Servo Amplifier X Axis ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: น้ำ Coolant ไหลออกนอกเครื่อง

สาเหตุ:  Chip และเศษผ้าสะสม ทำให้น้ำไหลย้อน

การแก้ไข:  ทำความสะอาด และยาแนว Cover ด้วย Silicone

=== END ===

=== MACHINE: TOYODA ===

อาการ: หินหยุดไม่ทราบสาเหตุ

สาเหตุ:  สายพาน Spindle ขาดทำให้หินไม่หมุน

การแก้ไข:  ทำความสะอาด Pulley wheel head spindle และเปลี่ยนสายพาน Pulley wheel head spindle ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Auto sizer error

สาเหตุ:  Auto sizer ไม่เคยได้รับการ Calibration

การแก้ไข:  Set auto sizer ใหม่ และ Set up wheel position ใหม่

=== END ===

=== MACHINE: TOYODA ===

อาการ: Alarm coolant pressure low

สาเหตุ:  ใบพัด Motor coolant แตก

การแก้ไข:  เปลี่ยน Motor coolant pump ใหม่

=== END ===

=== MACHINE: TOYODA JOURNAL GRINDING ===

อาการ: Work Spindle Index ไม่ทำงาน

สาเหตุ:  สายพาน Spindle หย่อน

การแก้ไข:  ปรับตั้งสายพาน Spindle ใหม่

=== END ===

=== MACHINE: TOYODA JOURNAL GRINDING ===

อาการ: C.433 Dlwk Satellite Fault

สาเหตุ:  ข้อมูลหน้าจอหายบางส่วนจากระบบ CPU/PLC

การแก้ไข:  ลงข้อมูล PLC ใหม่หลังสลับ CPU และสาย Link

=== END ===

=== MACHINE: TOYODA CAM GRINDING ===

อาการ: Tail Stock ไม่ทำงาน

สาเหตุ:  Reed Switch สายขาด

การแก้ไข:  เปลี่ยน Reed Switch ใหม่

=== END ===

=== MACHINE: TOYODA JOURNAL GRINDING ===

อาการ: Alarm Fluid Bearing P. Fault

สาเหตุ:  Pump ระบบ Fluid Bearing สกปรก

การแก้ไข:  ถอดทำความสะอาด Pump

=== END ===

=== MACHINE: TOYODA CAM GRINDING ===

อาการ: Alarm Oil Air Lub Unit Fault

สาเหตุ:  Pump Unit Lubrication ชำรุด

การแก้ไข:  เปลี่ยน Pump Unit Lubrication

=== END ===

=== MACHINE: TOYODA CAM GRINDING ===

อาการ: Door Error (Zero Return)

สาเหตุ:  Safety Door ค้าง

การแก้ไข:  ทำความสะอาด Safety Door

=== END ===

=== MACHINE: TOYODA CAM GRINDING ===

อาการ: Pressure Switch ทำงานค้าง

สาเหตุ:  Line Filter อุดตัน

การแก้ไข:  เปลี่ยน Line Filter ใหม่

=== END ===

=== MACHINE: TOYODA CAM GRINDING ===

อาการ: Alarm Coolant Pressure Fault

สาเหตุ:  Pressure Coolant แรงดันไม่ถึง

การแก้ไข:  ทำความสะอาด Pressure Switch

=== END ===